Course No. 		:  ES ZG525

Course Title 		:  AVIONICS SYSTEMS

Nature of Exam 		:  Closed Book

Weightage 		:  30%

1. a) What do you understand by the term ‘Integrated Drive Generator’?   …… (3)

An Integrated Drive Generator (IDG) is a crucial component in modern aircraft electrical power systems. It is a self-contained unit that combines a constant-speed drive mechanism with an electrical ge nerator to provide stable and regulated AC power to the aircraft.

### Key Features of an IDG:

- Constant Frequency Output: The IDG maintains a constant generator speed regardless of variations in the aircraft engine speed, ensuring a stable frequency (typically 400 Hz).

- Combination of Two Systems: It integrates a hydraulic constant-speed drive (CSD) and an electrical generator into a single unit.

- Reliable Aircraft Power Source: IDGs are commonly used in commercial and military aircraft to generate power for avionics, lighting, and other electrical systems.

### Working Principle:

- The engine drives the IDG at varying speeds.

- The CSD adjusts the rotational speed to keep the generator output constant.

- The generator then produces AC power with a fixed frequency.

b) What is the purpose of carrying out an ‘electrical load analysis’ for an aircraft? ..... (2)

Electrical Load Analysis (ELA) ensures that an aircraft's electrical system can handle all onboard loads safely and efficiently. It verifies power supply adequacy, prevents overloads, ensures compliance with aviation regulations, and maintains system reliability by assessing normal and emergency power requirements.

c) Why does an AC electrical supply on an aircraft use 400 Hz?......(1)

Aircraft use 400 Hz AC supply instead of the standard 50/60 Hz to reduce the size and weight of electrical components like transformers and motors, making the system more efficient and suitable for aviation.

2. a) Distinguish between QFE and QNH.     ……(1)

QFE: Atmospheric pressure at the airfield reference point; altimeter reads height above the airfield.
QNH:  Atmospheric pressure at mean sea level (MSL); altimeter reads altitude above sea level.

b) What is the importance of the International Standard Atmosphere?.......(2)

The International Standard Atmosphere (ISA) provides a standard reference for temperature, pressure, and density variations with altitude. It is essential for aircraft performance calculations, calibrating instruments, and ensuring consistency in aviation operations worldwide.

c) What is an ‘air data computer’? List the aircraft systems requiring ‘air data’…... (3)

An Air Data Computer (ADC) is an avionics system that processes data from air pressure sensors to calculate essential flight parameters such as altitude, airspeed, vertical speed, and Mach number.

### Aircraft Systems Requiring Air Data:

- Autopilot & Flight Control Systems – for maintaining stability and altitude.

- Flight Management System (FMS) – for navigation and fuel efficiency.

- Altitude and Speed Indicators – for pilot instrumentation.

- Weather Radar & Wind Shear Detection – for safety and situational awareness.

- Engine Control Systems – for optimizing performance based on altitude and airspeed.

3. a) What are the advantages of a ‘Fly-by-wire’ Flight Control System……. (3)

### Advantages of a Fly-by-Wire (FBW) Flight Control System:

- Reduced Weight & Complexity – Eliminates heavy mechanical linkages, reducing aircraft weight and maintenance.

- Improved Safety & Stability – Provides automatic corrections, stability augmentation, and envelope protection to prevent unsafe maneuvers.

- Enhanced Performance & Efficiency – Enables precise and smooth control inputs, reducing pilot workload and improving fuel efficiency.

b) What do you understand by the term ‘handling qualities’ of an aircraft? …. (2)

Handling Qualities of an Aircraft:

Handling qualities refer to how an aircraft responds to pilot inputs in terms of stability, controllability, and ease of maneuvering. Good handling qualities ensure smooth, precise control with minimal pilot effort under various flight conditions.

c) List the primary and secondary control surfaces on an aircraft. …... (1)

Primary Control Surfaces: Ailerons, Elevators, Rudder.
Secondary Control Surfaces: Flaps, Slats, Spoilers, Trim Tabs.

- a) What do you understand by the term ‘glass cockpit’?  …….(2)

A glass cockpit is an advanced aircraft cockpit that replaces traditional analog dials and gauges with electronic flight displays (EFDs), typically LCD or LED screens. These displays integrate critical flight information, such as altitude, speed, navigation, and engine data, improving situational awareness and reducing pilot workload.

b) Explain the principle of the ‘Head-Up-Display’. What is the advantage of using HUDs on

commercial aircraft?.......(4)

Principle of Head-Up Display (HUD):

A Head-Up Display (HUD) projects critical flight information onto a transparent screen in the pilot’s forward field of view. This allows pilots to access important data, such as altitude, speed, navigation, and landing guidance, without looking down at cockpit instruments. The HUD works using a collimated light system, ensuring that the displayed image appears at optical infinity, reducing the need for eye refocusing.

Advantages of HUDs on Commercial Aircraft:

1. Enhanced Situational Awareness – Pilots can monitor essential flight data while keeping their eyes on the runway or airspace.

2.  Improved Safety – Assists in low-visibility conditions by displaying landing guidance and terrain warnings.

3.  Reduced Pilot Workload – Eliminates the need for frequent instrument scanning, leading to quicker decision-making.

4.  Precision Landing Assistance – Helps in accurate approach and landing, reducing the risk of accidents in challenging weather.

- a) Distinguish between ‘position fixing’ and ‘dead reckoning’ navigation with examples….(2)

### Distinction Between Position Fixing and Dead Reckoning Navigation:

- Position Fixing:

  - Determines an aircraft’s exact location using external references such as GPS, radio beacons, or celestial navigation.

  - Example: A pilot using GPS coordinates to find the aircraft’s precise position.

- Dead Reckoning:

  - Estimates the aircraft’s position based on previously known location, speed, time, and heading, without external references.

  - Example: A pilot calculating the aircraft’s current position by considering airspeed and wind drift from the last known point.

b) Explain the basic principle of ‘satellite navigation’. What is ‘trilateration’? …. (4)

Basic Principle of Satellite Navigation:

Satellite navigation relies on a network of satellites (such as GPS, GLONASS, or Galileo) that transmit signals to a receiver on Earth. The receiver calculates its position by measuring the time delay between when the signals were sent and when they were received. This allows for determining latitude, longitude, altitude, and velocity.

What is Trilateration?

Trilateration is the mathematical technique used in satellite navigation to determine a receiver’s position. It involves measuring distances from at least three satellites to pinpoint a 2D location and from four or more satellites for a 3D location (including altitude). Unlike triangulation, which relies on angles, trilateration is based on distance measurements.

******