Birla Institute of Technology & Science, Pilani

Work Integrated Learning Programmes Division

Second Semester 2024-2025

Mid-Semester Test

(EC-2 Regular)

Course No. 		:  DE ZG561

Course Title 		:  Mechanisms and Robotics

Nature of Exam 	:  Closed Book

No. of Pages        = 3

# No. of Questions = 7

Weightage 		:  30%

Duration 		:  2 Hours

Date of Exam 		:

Note to Students:

- Please follow all the Instructions to Candidates given on the cover page of the answer book.

- All parts of a question should be answered consecutively. Each answer should start from a fresh page.

- Assumptions made if any, should be stated clearly at the beginning of your answer.

| Q.1 | How is the load carrying capacity related to selecting a robot for a given application? What is the impact of the mass of the end effector? | [3] |
| --- | --- | --- |
| Q.2 | The petrol tank of a motorcycle must be spray painted by a robot. The tank is a three-dimensional object with curved surfaces about all axes with a half tear drop shape. The spray nozzle must always be perpendicular to the surface at a distance of 100 mm. Determine the no of degrees of freedom required by the robot. Justify your answer. Figure 1 | [3] |
| Q.3 | Analyze the mechanism given in the figure and determine the degrees of freedom. Figure 2 | [4] |
| Q. 4 | Assign the coordinate frames and write the forward kinematics equation for the 6 dof manipulator for the shown home position (figure 3), Write the equations for T01, T12, T23, T34, T45 and T56. (General Equations are sufficient - Matrix Expansion is not required). Figure 3 | [8] |
| Q. 5 | How is a drive system selected while designing a robot? | [2] |
| Q. 6 | List the steps in using a polygon to determine the Chebyshev’s spacing of n number of accuracy points. Comment of the spacing of the points. | [2] |
| Q.7 | Find the variables θ1, θ2 and d3 for the given arm for the target point P (650,250,350), if X1= 100mm X2 = 150mm, L1 = 400mm. Figure 4 ********* | [8] |