Birla Institute of Technology & Science, Pilani

Work-Integrated Learning Programmes Division

Second Semester 2024-2025

Comprehensive Examination

(EC-3 Regular)

Course No. 		: DE ZG561

Course Title 		: MECHANISMS AND ROBOTICS

No. of Pages        = 2

# No. of Questions = 5

Nature of Exam 	: Open Book

Weightage 		: 40%

Duration 		: 2 Hours

Date of Exam 		:

Note:

- Please follow all the Instructions to Candidates given on the cover page of the answer book.

- All parts of a question should be answered consecutively. Each answer should start from a fresh page.

- Assumptions made if any, should be stated clearly at the beginning of your answer.

| Q.1. | Three identical SRS open-chain arms are grasping a common object, as shown in Figure 1. Find the degrees of freedom of this system. Figure 1 | [6] |
| --- | --- | --- |
| Q.2. | Given the geometric conditions, write a program for moving a cylindrical object of height 150 mm from point P1 to point P2. Make suitable assumptions. Figure 2 | [6] |
| Q.3. | A joint of a robot is rotated from 30 °to 90 ° in 10 seconds with a cruising velocity of 8 deg./second. Find the blending time using parabolic blending and plot joint positions velocities and accelerations. | [8] |
| Q.4. | List the steps in using an H-bridge is achieving position control of a link. | [4] |
| Q.5. | Compute the Manipulator Jacobian for the 3-axis robotic arm shown in the figure. The position of the tool point P given as follows. P=cosθ1[(L3+d3)cosθ2-L2sinθ2]sinθ1[(L3+d3)cosθ2-L2sinθ2]L1+L2cosθ2+(L3+d3)sinθ2 Figure 3 | [8] |
| Q. 6. | Two links with the same mass m, pivoted at points O1 and O2 are shown below. Select the link in which the dynamic torque will be minimum and derive the dynamic torque equation using the Lagrange-Euler dynamic model. (a); (b). Figure 4 | [8] |

***********