Mechanisms and Robotics (Merged - DEZG561DMZG561ESZG561)(S1-25) - Prof. Glynn John-20250802_175123-Meeting Recording

August 2, 2025, 12:21PM

2h 3m 51s

GLYNN JOHN . started transcription

GLYNN JOHN .   0:08
OK, the recording has started.
OK, so did you have a chance to go through your textbook or did you get any questions after reading? Did you try reading first of all and did you get any questions?
Yes, from what we discussed in the last class, any questions so far?
OK, so looks like nobody tried any reading last week.
Right. If you start reading, probably you'll get more questions.
OK, so uh.
In the last class we discussed A . more of history and we also discussed some basics of degrees of freedom, right? So in today's Lecture we'll discuss robot anatomy, right? We of course started with this.
And we'll discuss ARM configuration, so how to get basic configurations by arranging the links and the joins in A . particular manner, then join notation scheme.
And then degrees of freedom of robotic arms, right? And we'll also discuss how to identify robotic arms. Right now we'll start, you know, today's session with A . question, right? Can you identify A . robot type right? If you see A . robot, can you just identify?
What robot it is, what configuration or what type it is? That's one question. And if you have A . task in mind right now, for example, you have A . requirement you want to automate A . particular task. Now if you see A . robot, can you say that whether this robot?
Can be used to automate that task, right? Or any robot. Now can you select A . robot based on the requirement in hand, right? So can you do that? These two are the questions that we are starting the Lecture with.
Now, by the end of the lecture, if you listen carefully and if you understand pretty much the most of what we discussed today, then you'll be able to answer these two questions, right? You should be able to identify A . given robot, and you should be able to say whether A . particular robot will meet.
A given set of requirements, right?
Studies on basic mechanics can help you. Now you have A . mechanical background. Mohnish, you have A . mechanical background or.
Non mechanical background.
See, now studying basic mechanics will help anybody. It's not A . problem, but.
If you have A . OK, you have an electrical background. Yes. See, now you you have to. If at all you're going to read something, you have to read basic kinematics, right? So any book on basic kinematics theory will actually help you understand the basic concepts right of mechanisms, links, joints.
These concepts can be found in basic kinematics textbooks. AB level, second year B level kinematics books will actually have some of the fundamentals and those books will also have concepts on dynamics, right? Dynamic forces and so on. So some basics any book on.
Kinematics will actually give you some basic understanding. OK, so now anyway, those questions are not meant to be answered immediately. So if you answer those questions after we discuss those topics, then that's fine. OK, so we started with two basic.
Joins, right? In the last class we discussed one is A . revolute joint. So that is if you have one link like this, another link like this, and if you connect these two using A . pin right, then this will be able to swing like this, right? So this will be able to swing like this. It's A . pin joint. It can be called.
A pin joint or A . revolute joint. So this is A . basic joint type. The other joint is A . rismatic joint, right? So if you have something like A . shock absorber.
Right. Shock absorber of your motorcycle, right? Any piston cylinder arrangement where one member can slide in and out, right? So that is A . linear joint or A . prismatic joint, right? So this again, you know, these can be constructed in many different ways depending on what kind of mechanical design you're doing. You can construct this in different ways, but the basic movement will just be the.
Scene, right?
OK, so this is A . pin joint. Now you can actually see that you know one part is forked like this, the other part is simply flat, right? So this is another construction. So this is just A . pin joint.
Now A . prismatic joint can be like this right? Now you can have A . guide bar like this and A . sliding mechanism can slide on this. This is possible. Or you can have something like this like A . shock absorber right? Or you can have something.
Like A . slider guided in A . slot, right? I think I showed you that in the last class, right? You can have A . slider which moves in A . slot. So all these will, you know, produce the same movement.
OK, now when you have A . kinematic chain, right? When you have A . kinematic chain which is open right? When you have an open kinematic chain, so that means you have one link attached to A-frame like this and you have A . second link attached to the first link, right? So that means in this particular case.
The frame is actually link number one, but that link is not moving. Second link is this which is attached to the frame and the third link is this. So this is an open kinematic chain. Now in this particular case if you see how many degrees of freedom.
We have. Now we can move this link independently, right? So we get one degree of freedom like this. We get this movement. Then we can also move this link independently, right? No matter where this position is, we can still move this link.
So both the links can actually move independently. The frame is anyway grounded or fixed, right? So if you see here, we have two degrees of freedom, right? Now one other way of looking at it is an open. In an open kinematic chain, the number of degrees of freedom is equal to the number of joints.
In an open kinematic chain, this is true only for open kinematic chains. If you have A . closed kinematic chain, again you have to determine the degrees of freedom using A . different procedure. We'll we'll discuss that right later. But as of now you can keep this in mind. This is A . universal rule, open kinematic chain. When you have that, the number of degrees of freedom is equal to the number of joins.
Right. OK. So the same thing is actually mentioned here. So if you see.
If you calculate it, the number of degrees of freedom will be equal to the number of joints, right? So whatever I mentioned.
Is stated in this these two slides. OK, so the next thing is now we saw A . revolute joint and A . prismatic joint. Right now the prismatic joint is always A . prismatic joint. There's no change there. Nothing is going to change there. Now you can also call it A . linear joint.
And you can use A . symbol L, right? So that is A . prismatic joint. Now we discussed A revolute joint. Now this revolute joint can be further classified into three different categories or three different types. So one is A . rotational joint.
2nd is A . revolving joint and 3rd is A . twisting joint. Now what is the difference between these three right now if you have if you have A . link?
You can connect it to another link using A . pin, right?
OK, you can connect one link to another link using A . pin. So when you do that you get A . revolute joint. Now I can do that in A . few different ways. How is that possible? OK, now first I have one link.
I connect this to another link, so this is what I showed earlier. So now the pin goes through like this, right? The pin goes through like this. So if you see this is link number one, I can ground this if I want this link is.
Ivoted at the joint. Now this can turn. Now if I look at the axis of the pin, the axis of the pin is perpendicular.
To link #1, the axis of the pin is perpendicular to link #2. So the two links can be in any position, but the joining pin axis or the joining pin is perpendicular to both the links or the axis of the joining pin is perpend.
Perpendicular to both the links, right? So this is condition number one. In this case you get A . rotational joint. Now this is given the symbol R, right? So the two links can be in any position, but the joining pin is perpendicular to both the links.
Right. Or you can say that this is the link axis. Now one way of looking at it is this is the link axis. This is the link axis.
But the joint axis is perpendicular to the link axis, right? This is rotational joint R. Now I can do this in a different way also. Now I take one link.
Like this.
This is link #1 S. This will have an axis like this O. The axis is drawn along the length of the link. Now I can take another link like this.
I can join this link with this right? O in this case I can put an axis or a joining pin like this.
I can put a joining in like this O in this case.
This link will ivot about this point like this O if you look at the axis of the joining in.
The axis is perpendicular to one link and it is aligned with the axis of the other link. So it is parallel or aligned with one pin or one link. It is perpendicular to another link, right? So in this case.
I'm just calling this joint a revolving joint V right? So if you see the pin is going to come here, in this case the pin is going to be here. So the pin is aligned with the axis of one link, but it is perpendicular to the axis of another link.
Right O. In this case we are calling this a revolving joint. It's given by the symbol V right? Now we have a third condition. I have one link.
Like this and I'm joining this with another link which is also like this right? Our two links are joined. Now they're joined using a pin like this, so the pin is connecting both the links.
So in this case, the axis of the pin is aligned with the first link. The axis of the pin is aligned with the second link, right? So in this case we are calling this joint a twisting joint T.
Right.
Twisting joint. So all all three are revolute joints, but we are just differentiating them with the axis of the joining pin and the position of the links, right? So based on that we get rotational joint, revolving joint and twisting joint, right?
So any questions on the join types?
Any any questions so far?
Were you able to follow?
Clear, OK.
Listing join. OK, I'll repeat any other questions.
Now again, uh, here.
We have some symbols also. See now in one case, if you see the joining pin is perpendicular to both the links, right? So in the case of rotational joint, the joining pin is perpendicular to both the links, right?
In the second case, the joining pin is perpendicular to one link here and it is aligned with another link, so that is revolving, right? Otherwise you can see that one link is like this and another link is perpendicular to the first link, right? So this is also one link.
Way of looking at it. Of course the joining pin has to go through like this, right? So that is revolving. In the case of twisting, it is just two cylinders, right? Two cylinders, one on top of the other. Place one on top of the other. Now the joining pin can be positioned inside.
Right. Such that it gives you that movement, right. It doesn't have to be a pin joint. In the usual case, what you'll have is you'll have a bearing and you'll have a projection from another link and it'll go and fit in that bearing. And of course we have to retain, we have to put a retaining ring so that it doesn't come out right. So mechanical construction.
Can be complex, can be, you know, different. That's OK. But the overall understanding of it, it is just like, you know, using a pin to connect these two. So in the case of a twisting joint, 2 cylinders, one on top of the other, you're connecting, you're connecting them using a pin.
And the pin axis is aligned with the axis of both the cylinders, right? So that's a twisting joint. So with this, you know we get many different joint types, right? So again, try to follow and you'll get it. At one point, you know you'll be able to.
Clearly understand this. Yeah, you know, motor and a shaft will have, you know, something like this arrangement. Yeah, no doubt. So.
See pump. See when you when you use a coupling then you don't have a degree of freedom that you're pretty much joining it right now. Here you'll have a degree of freedom. You should allow rotation right? So now motor and a shaft you have a degree of freedom because the shaft can rotate. But when you say coupling you are you are pretty much locking that right?
OK, so now let's try to understand a human arm characteristics, right? So how many degrees of freedom does a human arm have right now from the shoulder to the wrist?
Right. Let's not worry about the hand part. Now, from the shoulder to the wrist, how many degrees of freedom does a human arm have, right? Yeah. Any guesses?
Anybody wants to guess?
OK, usually I'll get interesting numbers. OK, starting with three. OK, any other number 3/2.
Number of degrees of freedom of a human arm. See, now you are underestimate. You are underestimating the human arm.
Of.
Yeah, I see this as an insult to nature. OK, somebody's saying 6.
Any other numbers?
5-6 all interesting numbers coming. OK, so now we'll try to 17. No, now I'm just talking only from the shoulder to the wrist, right? OK, so now let's try to understand this right now at the shoulder.
At the shoulder we have a ball and a socket joint, right? So if I have to sketch the arrangement, it is going to be something like this, right? At the shoulder alone we have a ball and a socket joint.
Right now, how many degrees of freedom does a ball and socket joint have, right? We discussed this in the last class.
OK.
OK, now I'm I'm still getting funny numbers. OK, so fine. So let's see how many degrees of freedom. See, now you you can see me on the screen, right? So now your shoulder joint. See, I'm just moving my shoulder, just my shoulder. So I'm just making one.
I'm making a rotation about the vertical axis, so this is 1 degree of freedom. I can also do this, so I'm rotating about the horizontal axis, so I'm just holding my pen as the horizontal axis. The third degree of freedom is I can do this.
I can do this, right? I'm just able to rotate my arm about the axis of this particular link, right? So I have 3 degrees of freedom for the shoulder. For the shoulder alone, I have 3 degrees of freedom.
OK.
So at the shoulder I have 3 degrees of freedom. Now at the elbow. So if you see the elbow joint, I can do only this rotation, right? I can just fold this and I can move it back, right?
So at the elbow, if you see, I just have one degree of freedom.
Right, one degree of freedom at the elbow.
Now if you.
Now in the previous case that I mentioned, now all those joins had only one degree of freedom, right? So when when you're talking about an open kinematic chain, now all those joins had only one degree of freedom. So you have to count. So that means you have to count the number of degrees of freedom of each joint, right? So.
In the usual case of a mechanism, each joint will just have only one degree of freedom. So if the joint has more degrees of freedom, you have to count the number of degrees of freedom. So in this case, if you count it as an open kinematic chain, you have to count the number of degrees of freedom of each joint. Only then it'll make sense here, right?
OK, so now coming to the elbow. OK, now I'll be going back and forth like this. OK, so now if you see the elbow, I can just do only one movement like this, right? So beyond that, I don't have any other degree of freedom at the elbow, right? So if you want to check.
When you're moving your arm, you'll actually feel that you're getting this movement, that movement. Now you have to hold one member like this. You have to hold one part of the arm like this and then check. So I get only one movement, right? So that is the elbow.
Now elbow has got only one degree of freedom as of now, right? I'm just locking that as one degree of freedom. Then after the elbow, what do I have here? Now if I move here, see now to this part of the arm, I can do this twisting, right? So now it happens after the elbow.
So this twisting degree of freedom happens after the elbow. Now this happens between the wrist and the elbow. So now here we have two bones. Right now you've studied the construction of your arm, right? So here you have two bones, so that means you get this twisting degree of freedom. So that is happening between.
Between the wrist and the elbow, right? So now this movement is actually felt at the wrist, right? So some people will count it as a degree of freedom available at the wrist. Or you can see it by the construction and say that this movement is happening between the wrist and the elbow. So that is only one degree of freedom.
Which is a twisting degree of freedom, right? So I'll just write this.
OK, so in this case you're getting one degree of freedom, which happens between the elbow and the wrist. I'll write this as one degree of freedom. So I have 3 + 1 + 1. Now coming to the wrist.
Right now coming to the wrist, what do I have?
Now again, as I mentioned, if you want to check, you have to hold one particular portion of your arm. So I'm holding it here. So that means I cannot twist the arm. If I hold it here, I cannot twist my arm. I'm able to hold it. So now at the wrist I'm checking, I can do this.
I can move it forward and backward. So this is 1 degree of freedom. So rotation about this axis, right? So I'm able to do this. Now I'm holding it. Then I can do this also, right? So I can do this one degree of freedom. I can do this.
So that means I'm rotating about this axis, right? So 2 degrees of freedom at the wrist I'm able to get right. That is if I'm holding this part. Now those who want to argue that you know you have the rotational degree of freedom at the wrist, right? I'm not getting it if I hold it here, but after holding it, I'm able to get 2 degrees of freedom one this way.
And one this way, right? So.
OK, fine.
So here I have two degrees of freedom at the rest. So that means you know I'm able to get one movement like this and one movement like this. So totally if you see 3 + 2 + 2, right? So overall 7 degrees of freedom.
For the human arm, from the shoulder to the wrist, right. So. So this is what we get for the arm alone. Now for the hand, you have more number of degrees of freedom, right? For the hand alone, each finger acts as a.
A separate manipulator, right? So that's how our arm is constructed. Now if you have to construct how each finger is represented.
Now each finger is represented like this. Now again if you want to count this.
OK, now if I take my index finger, right, my index finger alone, I'm holding the base of it so that I don't get any additional movement. So I can move my finger like this left and right, right. And I can move this forward and backward, right? So that means I get 2 degrees of freedom at this particular.
So I get 2 degrees of freedom at the base of a finger, right? No matter what finger it is, you'll get this right. So these four you'll get this movement. So this and forward backward. Now if I hold it here, then what do I get in the next?
Joint. I get only forward and backward. I just get only one rotation. Beyond that I cannot do anything. I hold it hard, then I get only this movement right. Then the next one. If I hold it here, then I get one small movement but very limited degree of freedom. But still I'm able to move the top link, right?
So now the same thing.
Can be said about your four fingers, right? The thumb construction is slightly different. So if you have to look at the number of degrees of freedom of this joint, this has got two, this has got one, this has got one, right?
For.
Four other fingers. This is what you're going to get.
Right, you can check.
Right now if I have to draw the thumb, how is it going to be right? Thumb is slightly different, but.
OK, so coming back to the thumb. OK, so now if I take my thumb again, I get.
You know, see now you have to look at the base of the thumb. Now you have to look at this particular part, which is see now thumb is hidden inside inside your arm like this. So the base will have two degrees of freedom, right? So the base will have one this way and one forward backward movement, right? So the next part if you hold.
It here then it can go only.
You know it can do only one rotation, right? One degree of freedom, then the top part.
It can do one, right? So if you see, it appears as though the thumb has got 4 degrees of freedom, but beyond that it will have one additional movement.
OK, now let's see the thumb, the way it is constructed. See now by looking at it, it appears like this, right? So just like every other finger. So two, one and one.
But if you see, we can actually rotate the thumb and bring it.
And make it opposable, right? So I can I can hold my hand like this. I can move it. I can rotate the thumb and move it and make it opposable, right? So we have opposable thumbs, right? So that means I can keep my thumb on this plane.
I can still rotate it and bring it to the opposite direction so that I can hold an object like this, right? So which means that the thumb has got one rotation here, which we don't get to see easily, but we can move it and see the positional change, right?
So from the mechanical understanding of this, you know, I can just write it like this, right? So it is almost like, you know, having one degree of freedom like this, right? So that means.
I can put one here. So these are the visible or obvious degrees of freedom of the hand. Beyond that we have more here, right? So now from this what do we have? So for.
5 is at 20 plus 121 right? Now beyond that if you see we have more degrees of freedom in the palm. So now from the fingers we are just writing this as 21, so at the palm.
See, now you can make your palm like a cup, right? So you can make your palm. You can change your palm to the shape of a cup. So that means you have more number of degrees of freedom here. So you can make it flat. You can make it like a cup, right? You can.
Bend it like this so you have more number of degrees of freedom in the palm itself. So again, if you want to do more research on that, you can refer to any biomechanics book, right? So again, even there the authors are just contradicting each other.
So from this, what we see is that it has got now the palm has got a few number of degrees of freedom. Now there are many bones. Now this construction is very, very complex here. So you have more number of degrees of freedom, but the range is very, very small, so it is not obvious when we see it.
So the human arm overall has got greater than 30 degrees of freedom. So that means this 7 plus this 21 plus all the degrees of freedom available in the palm put together, you get more than 30, right? So that number is not.
Clear because the bones and the movements are very, very small and we don't have a clear cut number. But of course literature on biomechanics will give you a little bit more details, right? So this is about human arm characteristics. So what we have is something very, very complex, much more.
Complex than a typical industrial robot, but the kind of control system that we have is way too complex, right? So, so far we have not designed something like this, right? So our human brain is very, very complex. So it is able to control these complex mechanisms and we are able to get the movement that we want.
Right, but in the actual robotic arm, a typical industrial arm will have about 6 degrees of freedom, right? In some cases you can get more. 7 degrees of freedom is again possible, right? In commercial industrial robots you can get 7 degrees of freedom also.
But usual typical ones will have 6 degrees of freedom, right? OK, so any questions on human arm characteristics?
Yes, any questions? Is it clear?
OK.
OK, good. So if you understand this part, at least up to this, it's fine, right? Yes, if you are interested in learning more, you can always read more literature on this particular area, OK?
So now you know how many degrees of freedom are required for doing a particular task, right? So that is something we'll simply try to understand first. OK, now I have to move an object.
Along a circle. So this object is going to move only along this circle, and it's not going to go anywhere else. Just along this circle, this object is going to move. How many degrees of freedom will I need? Anybody? Any guesses?
OK.
People are saying one. I'm I'm seeing one, a lot of ones and some twos, OK?
Now let me ask one more question. Also, yeah, I have to move an object along a straight line. This object has to follow only this straight line and nothing else. How many degrees of freedom do I need?
In this case.
OK, I'm seeing 1111 a lot of ones here.
OK, it is one. See in this case it is one degree of freedom. Now here it is possible with one also right now if I have a link like this which is pivoted at this particular center.
Now if I.
Lace a link like this. Now I can put a in here at this particular point O now if I rotate this.
This is only one joint, right? I can just get this movement with just one joint. I can rotate it. Then this link will follow this circle, right? So if you're following only this circle, then this is possible with one degree of freedom.
Right. OK, now let me ask you a different question. Now I have a circle. Now I should be able to reach within the circle. How many degrees of freedom do I need? I should be able to reach any point inside the circle.
Yes, OK, 32.
OK, I see a lot of twos and some threes. OK, so now if I have to reach any point within the circle, then I need to have a mechanism like this, right? I I should rotate one object like this.
Then the second object.
OK.
Now this can carry a pen like this. So now if I have this movement right, then of course I'll not be able to get the entire circle, but still I can do that by arranging the mechanism in a slightly different way. OK, so.
With this arrangement I can pretty much cover most of the circle, right? So let me support it like this. OK, so now this pin is going to rotate and this object is going to move back and forth.
Right O in this case I can cover the entire circle O the two degrees of freedom.
I'll be able to reach any point within the circle, right? So that means I'm just staying only on this plane. I'm not lifting, I'm staying only on this plane, but I'll be able to reach any point within the circle now. Third one.
I have a cylinder or a cylindrical space. Now I should be able to reach any point within the cylinder. How many degrees of freedom do I need?
OK, good. So I'm getting 3 degrees of freedom, so the same thing. So if I have to lift it now, OK, so let me leave this. I'll just draw it again. So if I have to.
Do the same thing for a cylinder. Then yeah, I have to have one pin which I rotate like this, right? I have to get this movement for moving in and out.
Then I need a third movement, maybe moving U and down, right? Something like this.
So I get 3 degrees of freedom. So in this case I'll be able to navigate within a cylindrical space, right? So.
So this is how we have to visualize whether.
You can navigate a given space, navigate within a given space and how many degrees of freedom will be needed. So similarly now for you know a cuboidal space right? Or a three-dimensional space like this, a cube, cuboid, all these spaces. Now you need 3 degrees of freedom if you want to navigate within this particular.
Sace O. This is just a basic understanding.
OK, so now three-dimensional space, 3 degrees of freedom, right? Flat surface, 2 degrees of freedom, right? If you want to cover the entire surface line or a circle, one degree of freedom, then more complex profiles right now if I have an object like this and I have.
Holes from every direction, right? If I have to reach within this hole, then I'll need more degrees of freedom, right? So if I have to simply touch the surface, then it's easier, right? But if I have to reach inside a slot or reach inside a hole, then I need more degrees of freedom. So depending on the complexity of that operation.
You'll actually require more degrees of freedom, right? So we'll just move ahead with this understanding. OK, on configurations. OK, now based on our understanding, now let's try to construct.
Different arm configurations or different robotic arms. Now what are the possible constructions that we can get right now we are gonna use see now in the case of the human arm at the shoulder.
We have 3 degrees of freedom.
Right now, this particular joint is very, very difficult to control, right? You get all the movements in one particular joint. Now in our arm, we are able to do that easily because we have thousands and thousands of muscles and each muscle fiber gets control.
By the human brain and we are able to get all kinds of movements. Now in the case of a typical mechanism, what happens is that we'll have one link, we may have another link. Now this has to move along this move about this axis. Then how can I get that movement?
I would put a motor, right? I would normally put a motor like this at the joint. Then I'll take my motor shaft and connect it with this link, right? I'll connect my motor shaft with this link. Then when I power the motor, then this is going to rotate this way or that way. So this is how.
We are able to control joins or you know I could.
Have one link, I could have another link. Now the other thing I can do is you know I can connect a shaft like this. See now I could have a bearing like arrangement here or I could I could simply have a bearing.
Then I'd put a ulley here.
Right. I'll put a pulley, then I'll put the motor here.
Then using a belt drive, then I'll put a pulley on the motor, then I'll put a belt drive. I'll drive this link, right? If I have to go into the details, then you have to construct something like this. Or instead of a belt, I could use gears or you know, a chain drive, something like that, right? So.
Typically this is how we control joints, right? So you will power the joint with a motor and then you will turn the joint. Now putting this, you know, in a joint and getting 3 dimensional movement is not even.
Possible, right? Or it's very, very complex, right? We cannot fit 3 motors in one joint and get all these movements. We don't want to do that, right? It becomes fairly complex for us to construct that assembly. So normally when we construct mechanisms, we'll put only one degree of freedom.
A joint. You know, at every joint you'll just have one degree of freedom, and for every degree of freedom you'll have one actuator. So you can also see the number of degrees of freedom to be equal to the number of actuators, right? So if you calculate the number of degrees of freedom required, that means you require.
So many actuators. If your number of degrees of freedom is equal to three, you need 3 motors, right? So if your number of degrees of freedom is 6, then you'll need 6 motors like that. Also you can interpret this because we are going to put only one degree of freedom at a joint and we are going to have only one actuator at that particular.
Joint OK now.
Taking this into consideration, then let's try to construct something similar to a human arm, right? OK, so now I have to restrict the movement to only one degree of freedom, or restrict the degree of freedom to 1, so.
Let me construct something like this O I'm just putting A-frame, then I'm just attaching 1 link.
Two see now one link. This link number one is the frame this link #2.
Right then link #3.
O3.
4. So now the number of joints. How many joints do I have? I have one joint here, two and three joints. Now each joint I'm just restricting to 1 degree of freedom, so I'm able to construct something like a human arm with three degrees of freedom.
OK, so now if I construct it in this manner, it's not going to be of much use to me. So what I'm what I'm doing is I'm going to slightly see now. The reason why I'm saying this is not going to be useful to me is that I have the axis like this I have.
The axis like this and I have the axis like this. If I if I use only one degree of freedom and my axes are positioned like this, then my movement happens only along this plane. Even though I have 3 degrees of freedom, I don't get the kind of movement that I desire. I would prefer to have.
A three-dimensional movement if I have 3 degrees of freedom, so that that is something I'm not getting here. So I'm just altering the construction. So what I'm doing is.
Uh, putting one cylinder like this, right?
OK.
OK, so here I have a joint which gives me rotation about the vertical axis.
Right something like this.
I get rotation about the vertical axis. OK, so that means I'll I'll be able to rotate the top part 360 degrees or close to 360 degrees about the vertical axis. Now I'm just adding one more degree of freedom.
Right O I'm I'm just rotating like this.
Now I can add one more link like this.
Right, so I get one more degree of freedom. So that means you know by operating these two I can bring see now I can put an end effector here. If I want I can put a gripper here. Then I can move this closer by controlling these two joints.
Now if I want to reach around this, then I can rotate the first joint, then I'll be able to reach around.
The space right? So here I'm able to construct a three degree of freedom arm by putting the axis in a certain orientation, which gives me three-dimensional movement or axis to three-dimensional space, right?
Now this particular configuration is called the articulated configuration.
Articulated configuration. Now here if you see all the joints are revolute joints right? Now in this case this happens to be a twisting joint. This is a rotational joint. This is also a rotational joint, but all the joints are pin joints, right? Or revolute joints.
Right. There's no prismatic joint involved here, right?
OK, so here we have an articulated arm. So this is 1 configuration. Now what I'm gonna do is I'm gonna take this particular configuration and I'm gonna replace this.
Or I'm going to replace one joint or one pin joint.
With a rismatic joint.
So now here I have a.
This first joint, which is a twisting joint that is in the same position.
Now the second joint is also there.
Now instead of a third revenue joint, I'm just going to put.
A Rismatic joint here.
Right. So in this case I can do the up down movement. I can twist about the vertical axis, then the third one I'll be able to go in and out, right? So this is a twisting joint, this is a rotational joint and this is a linear joint, right? So this can be.
Written as TRL also right so starting from the base TRL. So this is called. This is a joint notation scheme also, so you can represent the arm using a joint notation scheme. Now this particular configuration is called polar configuration.
Polar configuration, right? So first you have articulated where every joint is a revolute joint. Now you replace one revolute joint with a prismatic joint. Then you get a slightly different configuration. So what you'll get will be.
Now if I move this arm around now, I'll be able to navigate through a sherical space, right? O.
Now this in three dimensions. Imagine this right? A spherical space in three dimensions. Now that is called the work envelope of the robot. So the robot can actually reach within the space. It can move within the space in three dimensions. Now that is the work envelope of the robot. So that means you know whatever objects you want.
To pick or move, those objects have to be placed within this envelope. Only then it can reach the object. Now if I place the object here, the robot will not be able to reach right because the gripper is gonna be here.
So the robot should be able to navigate that particular space, and whatever objects you want the robot to access, you have to place those objects within the work envelope of the robot. If I place it outside, it cannot reach. If I place it inside, it cannot reach, right? So this is something you have to keep.
In mind, while selecting robots, you should now the manufacturer of a robot will actually provide the work envelope, right? If you download any robot catalog from any manufacturer, he'll actually provide this information, the work envelope of the robot, the dimensions of the robot, and so on.
And based on that, you have to design the work style of the robot where you want the robot to perform its function, right? So you have to design that space around the robot in such a way that the robot is able to access those points, right? OK, so now we've seen the articulated and we've seen the.
Olar arm, right next, what we'll do is we'll try to replace one more joint. We'll try to replace one more joint.
But you know, a prismatic joint, right? So that means, you know, I can take.
So I'm just taking the first joint, so this is a twisting joint, so that is going to remain.
Now I'm going to replace the second joint also with the rismatic joint. So what what I'm going to do is I can, you know, sketch it like this.
So I have a prismatic joint. So this is going to give me this movement so I can twist it, then I can move forward and backward. But I need some kind of vertical movement. So what can I do? I can just, you know, place a joint like this.
I can get this moment.
Right. So in this case, I'll be able to navigate within a cylindrical space, right? If I have to sketch the work envelope of this particular arm, then I'll be able to, you know, do this something like this.
It should be able to reach within the space, right? So depending on how.
You know how much it can extend downwards. The cylinder will be like this. Sorry.
The cylinder will be like this, right? So within this cylinder, within this cylindrical space, right, the robot will be able to move. So if you see the work envelope is cylindrical, so this particular configuration is called a cylindrical configuration.
Cylindrical configuration. Now what I can do is I can replace one more one more joint with a prismatic joint, right? So I can just take one additional joint and replace that with a prismatic joint.
So what I can do is.
OK, so the base joint itself is made to be rismatic. Then I can put.
Something like this, but you know while doing this you have to be careful why you have to get.
Movement in all three directions, right? So this is vertical movement. Next you're getting forward and backward movement. Now if I put another joint like this, then I'm still moving on this plane, right? Which is of no use to me. I need to navigate A.
Dimensional space O. That means I have to put the third joint like this.
Right. So that means I have to go in and out.
I have to go in and out like this. So the third movement will give me in and out movement right? So one is giving me vertical, one is giving me forward backward. The third one is giving me in and out movement. So in this case.
I'll be able to navigate Uh.
A three-dimensional space which is, you know, cuboidal, right? So which is cuboidal 3 dimensional space, right? So this configuration is called Cartesian configuration.
Partition configuration. So these are the four basic configurations. Yeah, all kinds of, you know, machine tools, all your three axis milling machine, CNC machine, all these things will have this particular movement. Now the construction can be different, right? So here I have given a cantilever construction.
Right. For example, that means I'm I'm supporting on only one side. I'm letting the other side hang freely right now. Instead I could have, you know, a bridge type construction.
Right. So that means one movement can happen like this. It can be supported on both sides like this. And I could have one movement going up and down like this. I could have the entire bridge moving forward and backward. So this is an A-frame type construction, right? So normally we have this construction.
For cranes and so on, right, for heavy equipment. So that kind of construction is also possible and you can still, yeah, CMMS, all your cranes, right, will have such construction. So now that is also possible. Now that is also Cartesian configuration because the movement is only linear in three different directions.
If you get that movement, no matter you support on one side or you support on both sides, it is still Cartesian configuration. So that kind of variations you can do in the mechanism in the arrangement, right? So these are four basic configurations.
Now, apart from these, there is one more configuration, right? So any questions on the four basic configurations? Any questions? Now I just started with a human arm. I restricted the number of degrees of freedom to one for each joint. Then I tried to.
Construct something that gave me 3 dimensional movement. Then from that I just kept on replacing one revolute joint with a prismatic joint. Then I'm able to get four different basic configurations, right? So that's how I did it. So any.
Questions so far, they're able to follow till now.
OK.
Yes, in the articulated arm, yes. What's what's your question?
OK, notation wise, how will you represent the last two? OK, let let me come to that part.
See now when you're represent something with the joint notation scheme, then you have to start from the base. So in this case this is going to be TRR, right? So you start from the base, then you go towards the tip, right? So TRR is how you represent the articulated arm.
Polar arm. This will be TRL, at least this construction right the way this is constructed. Now the cylindrical configuration can be TL and L, so TLL this is.
Possible, right?
Now depending on how you constructed this can change also. So the way I have constructed this is TLL or you can construct it differently also. So the last one Cartesian configuration is LLL the way I have constructed right? So Cartesian it's anyway going to be LLL no matter what.
Right.
Two rotating only, which one?
See now here we are talking only about 3 degree of freedom arms. So the basic configurations are for three degrees of freedom only. So if you have more number of degrees of freedom, then the combination is going to be even more. So we are not even going there, right? So 3 degrees of freedom, you get 4 basic configurations then.
To add more joins as per your wish, right? So four basic configurations we are able to get. OK, now we saw this earlier, right? We saw this. OK, so now the ARM configurations now articulated ARM 3 Rotary joins or three revolute.
Polar 2 Rotary joints, one linear joint, cylindrical 2 linear joints and one Rotary joint, and finally Cartesian 3 linear joints. So these are your four basic configurations, right?
So you see now ARM configuration depends on these factors. Now whatever is mentioned here, right? All rotary joints again, the construction has to be in such a way that it's like this, right? So you get this 360 degree rotation.
Or the vertical axis, then you get. See now you should be able to navigate within a three-dimensional space. Now I can actually construct an arm with more degrees of freedom, but if I don't get 3 dimensional movement then it's of no use, right? So you have to see now here the construction.
Is done carefully so that each arm is able to navigate a three-dimensional space, right? So each arm is able to move about in three-dimensional space, so that's a requirement. Without that, if you add more degrees of freedom, then it's meaningless.
So and again, based on that we are able to get these configurations, right?
Now you choose a particular work envelope depending on the workspace where the robot is gonna perform its operations, right? So if you have a cylindrical workspace and if your entire.
Set of operations will fit within a cylindrical workspace. Then you can pick a cylindrical configuration robot for automation. Now if you're gonna work in a very long rectangular room, right? The room is gonna be very long and it's gonna be a rectangular workspace. Then you have to pick a Cartesian robot.
In some cases, I'll just give you some more information. In some cases, what you can do is you can actually take a regular robot and fit it on a rail, right? So we'll discuss that idea as we proceed. In that case, one robot can actually travel along.
A long space, right? A long rectangular space also, and still perform a task, right? So you can add additional degrees of freedom by putting the robot on a rail, right? By putting the robot on A2 dimensional stage. So that means it'll get X&Y movement.
See, it's not just linear actuator, it's a linear guide, right, which is of course controlled by an actuator. So in that case you can get, you know, more specific movements as per your requirement, right? So all those possibilities are there, but let's try to understand some of the basics now, right?
OK.
So the basic configurations are here. Now what happened was OK, so here we have the work envelope. Now the basic configuration is shown here, Cartesian configuration is shown here and the work envelope right? So Cartesian configuration will have a cuboidal work envelope.
Now a slightly different construction is also shown here, right?
A construction with double sided support, right? So this is also possible. Similarly, a cylindrical configuration will have a cylindrical work envelope. So the work envelope is shown here. A cylindrical robot and the work envelope is shown here.
I sketched the same thing there. Now in the case of a olar robot, you'll have a sherical work enveloe. The work enveloe is going to be erfectly sherical, so that means.
It will be able to navigate within this space right in three dimensions, right? So that's a spherical work envelope. Again, the construction can be like this. The construction can be like this depending on how you want to build it, right?
But overall, it's going to be very similar because you know it's going to be a set of prismatic joints, and then you'll have a revolute joint right now. In the case of the articulated configuration, it's it's going to be more or less spherical, not perfectly spherical, but more or less spherical.
Right. So it is going to reach within a space like this. So the spherical, you know, name is given to polar because it's going to be perfectly spherical, right? So if you say a spherical configuration, it means polar configuration.
Articulated configuration has got another name called jointed ARM configuration.
Is uh.
See now as of now we are talking only about the basic configurations, right? So that means basic construction and with that what work envelope are we getting? So that's what we are discussing now. Now if you want to explore more right now, for example, I can take this robot and put it.
On a rail like this, that's what I mentioned earlier, right? So in that case, what will happen this?
Spherical spherical like configuration can be extended like this, right? So this it's gonna become something like an elongated Dome. So it's gonna go like this right? If I put it on a rail now if I can put it on.
Two rails or I can put it on, you know something like this also.
Something like this. I can put it on another rail in this direction.
So in that case, you know I'll get XY movement. I can, I can make it move like this also. Then this robot will be there. I'm just adding two more degrees of freedom, right? So like that all possibilities are there. So now if you if you're going to simply purchase a robotic arm and use for your.
Automation. The manufacturer himself will actually provide these options. He'll say like, put the arm on an extension like this. We provide this extension. You simply mount it. Now if you're gonna build a robot for your requirement, then yes, you can design it in such a way that you put it on a.
Linear slide and get that additional degree of freedom. So all that is possible right? So now just by putting this arm on a on a slide like this will not change the configuration. So you know if you're asking that question, will the name change right?
For the configuration, no. We are taking an articulated arm and putting it on a linear slide because this articulated arm is a basic configuration. Now we are talking about four basic configurations. So it's like, you know, a brick, right? Using the brick you can build, you know, very different looking houses. So that's a.
That's possible. Here what we have is a brick. So you take this and put it on a linear slide, then you get the kind of work envelope you want, right? OK, now what happened was Japanese.
Companies, they came up with an additional configuration, right? So here we have what is called a SCARA configuration. So this stands for Selective Compliance Assembly Robotic Arm. So what they did was they took.
A robot and created something like this right now. The way they've constructed this is see now in the case of an articulated arm, the case of an articulated arm, it behaves like a human arm.
Put upside down.
OK, so now an articulated arm is going to be something like this. You know, if I turn my arm like this, right? So then this is something that looks like an articulated arm. So what the Japanese companies did was they just took the human arm like this. See, now I can also move my arm like this in this plane.
Right, right. So that is the arrangement they gave this particular arm.
And they ended up with an additional configuration.
Now in this case.
If you see this arm.
OK, so now here you have.
One link sitting on another link like this.
Right. So that means you get one rotation here, just like every other arm. So you get a twisting degree of freedom, not even twisting. So this is actually AV, right? A revolving degree of freedom.
See now one link is like this, one link is like this. The joint axis is like this. So this is AV. Now here this is.
And R.
Right now the third one, they gave an L here, so that means this will come down and pick an object, right? So now in this case, this is a VRL robot, right? So.
This configuration is called a SCARA configuration. So here you get one linear joint and two Rotary joints. But what kind of work envelope will you get? If you see the work envelope of this particular arm, you will get a cylindrical work envelope.
Even though the construction appears to be closer to articulated, you get a cylindrical work envelope, right? So this configuration is called the SCARA configuration, right? So again, this arm is naturally rigid.
Now for example, you stop this arm here. You put a weight on this particular end effector. Now the weight will not move the arm in any direction because the joints are like this, right? So the weight will not move the arm in any direction. So naturally this arm.
Will be rigid and this makes it easy to control right now that statement. If you want you can actually compare this with an articulated arm right now if I put a weight.
On the end effector, now this weight is pulling this arm down. Now if I don't hold the joints with some kind of breaking mechanism or locking mechanism, what will happen to the arm? The weight will pull the arm downwards. This joints will actually flex.
And the arm will naturally come down right? So now I need to have some kind of locking mechanism here. Now this can be done by controlling the motor, or I can have a break at the joint, or I can have a locking mechanism. So any ways it's actually possible, any any one of these ways it's possible.
But if I don't have that, the weight will actually pull the arm down, but in the case of a SCARA arm if you see him.
Now here I can have this as a screw, so in that case it'll be self locking. So the joints automatically are rigid because they are pivoted on a vertical axis, so this mass will not pull the arm down.
So that construction gives it an inherent, you know, advantage, right?
This see now this is not polar configuration. The reason is see now this is giving you a cylindrical work envelope. Polar configuration. Polar configuration has a spherical work envelope. So if you see the arrangements in terms of the join types, it is even though it is looking similar.
To a polar robot. But in terms of its working and in terms of its work envelope, it behaves more cylindrical, right? So that's what makes it different. So and naturally it has got this rigidity because of the position of the joints, right? So that is.
A Scara configuration, right? So it looks like it's time for a break, right? So any questions on join configurations you're able to follow?
You're able to follow join configurations. Any questions on?
Join configuration so far. Clear so far. OK, at least for one person, it's clear.
Yeah, counterweight is possible, but normally we try to control it with the mechanisms we have, right? The motor itself can be used to control. Motor will actually apply an opposite torque and hold the joint in place. So all those things will.
Happen with the controller, right?
Of course locking mechanisms are possible. Heavier robots will have braking mechanism. Now you can actually lock it with the mechanical system itself, right? For example, you can have self locking mechanisms involved and then automatically it'll be locked. So all these things are possible.
OK. We'll just take a break for 5 minutes, then we'll continue. So if you have any questions, you can type your questions on the chat window.
Welcome back.
OK now um.
OK, now for the scara arm.
Now in terms of construction, it has got two Rotary joints and one linear joint, so that looks similar to a polar configuration, but the work envelope is cylindrical, so that's what I mentioned. So if you see the work envelope is cylindrical.
If you consider the movement of this particular arm, right O that's what makes it different and it becomes a separate configuration.
So if you if you turn this joint, it is going to make a circle. So from this particular space it'll be able to reach outside and it can move up and down. So you get this movement. So overall you end up getting a cylinder.
Right, the work envelope is cylindrical.
Right. You can hear me right, all of you.
Well, if you can hear me OK, OK, now let's see if you can identify a given configuration, right? So everybody's back. OK, now what? What is this configuration? The first one? Yeah, what configuration is it?
OK.
Scara.
Yeah, only two people are answering.
One person is saying cylindrical.
See, now this is Kara.
See, now cylindrical arm will be something like this, right?
1.
2.
And three.
It has to be something like this, but that's not what you see here, right? So this is a SCARA configuration, not cylindrical, right? So this is SCARA configuration. See now this is not RRL. So if you see this link is vertical, so this link is like this. So V.
This is this becomes AVR and L right? Just like what we saw earlier. It's very very similar to this, just just the same as this, slightly different in shape.
See now that is what in a SCARA configuration you will get a cylindrical work envelope.
But in a typical cylindrical robot, you'll have one revolute joint and two linear joints, right? In the case of a cylindrical arm, you'll get two linear joints, right? So a cylindrical arm will be like this.
One linear and one linear. So two linear joins will be there in the cylindrical arm, but here you see two revolute joins and one linear joint. So that's what makes it scar. So it's not cylindrical, it's scar. OK, now second one.
What configuration is it?
You have to start from the base, right? If you want to study this, you have to start from the base. Start from the base of the arm. OK, yeah, it has got more degrees of freedom.
Based on the 1st 3 degrees of freedom, you have to name it.
Yep.
Any other answer? OK, not polar.
It is articulated, not polar, because we don't have any linear joints here. Do we have, do we have a prismatic joint anywhere in this? Do you see a prismatic joint anywhere? No, right? All the joints are.
Revolute joins, right? All are revolute joins. So this is articulated. So that means here you have the first axis, the second joint axis is here, the third axis is here, the 4th axis is here.
Fifth one.
If you have a rotation here.
So in some cases you know one might be restricted also. So 1234 if this is 5 and this is 6, so this axis will be 6. Now in this case if you have one more degree of freedom in this direction then it will be 7 right? But.
Usually one of these can be locked. Now if this is not even an axis, if you don't have an actuator there, then you may end up with just six or you can have seven also. So you have to check the.
The catalog of this particular arm to know exactly whether it's six or seven, right? So from the construction, from the looks of it, it appears to be 7. OK, so this is articulated.
OK, next one.
Yeah, uh, third one.
Yes.
Any answers?
OK, everybody's saying articulated.
OK, so if you see the first axis is here.
2nd right now third.
Do we have any other axis in between? Third OK.
Five. And if you have one more here, then it's going to be 6, right? So.
So 6 axis articulated robot articulated.
Right. OK, now what about the 4th 1:00?
4th 1:00.
Put one.
OK, everybody's saying Scara. I think one person said cylindrical.
It's not. It's not polar. Definitely not polar.
It's definitely not polar.
It's definitely not, uh, cylindrical. See now if you see.
One rotation, right?
Right now you see.
So if you see this is SCARA configuration, the only thing is it's put upside down, right? And in some cases you can have more number of degrees of freedom also. Right now in this case you can actually have 4 degrees of freedom if you want. Now if you have one more.
Rotation here then it becomes 4 degrees of freedom. But you know from what we see it appears to be a basic 3 degree of freedom SCARA. Now four is possible with this arrangement. If you make this pin rotatable by including an additional axis then you can get 4. So this is a SCARA configuration.
The only thing is it's put upside down, right?
Uh.
You know, here we don't have any significance of the dotted line. I I just picked these images from a catalog.
Sorry, so I just picked these images from a catalog. So usually when we see these dotted lines it it should represent an axis, but in this case they're not. OK, so.
Both one is scalar configuration. OK, now what about this?
Yes.
What do you have on that image?
You know we can't. You have to get. I have to get more information. OK, now this is not Cartesian. OK, somebody's guessing correctly.
See now if you see this arm here you have one.
Rotation possible, right? And you have a vertical movement possible here, and you have one horizontal movement possible here, right?
See now there are two linear joints. If you see this is linear, this is linear, right? So in this case the basic configuration is cylindrical.
Right. It is cylindrical, but here you have a wrist which has got three more degrees of freedom. So now this is a is a famous Stanford arm, right? So this arm is called a Stanford arm. Again, you know, more modifications will actually give it.
In some cases you'll have a joint here. Now in this particular case, a visible joint is here. So all these possibilities are there, but this is a the basic configuration is cylindrical in this particular case.
OK, now what about this one right now? What number is this 45? OK, now six.
Yes, this one. What is this?
OK, uh.
OK, seven. OK, I asked only 6/6 is Cartesian.
Right, so since you have only linear moments 1-2 and three.
Six is Cartesian. Yeah, 7.
Yes, seven is obviously polar because see now here you have this rotation at the base, then you have this rotation.
And the third one is an extension and beyond that you have a wrist and you have more movements there, but the basic configuration is olar.
Right.
Yeah, what about 8?
8.
OK, so this is articulated.
The only thing is it's put upside down South. That means you will get one axis here, one rotation here, 2-3 right and four will be a twisting axis.
5.
Then six. You'll get one rotation here also at the rest, right? So 6 degree of freedom articulated robot.
OK, so now the 9th one. So we have not discussed this particular part, right? Any guesses?
So in this particular case, this is called a delta robot, right? So now this is not an open kinematic chain. If you see the construction, this is a closed kinematic chain. This is called a parallel robot. So.
Delta configuration. So now this is an emerging type. Now this was developed in the late 80s, mid 90s. So now this is becoming popular. Now it will behave like a Cartesian robot, but it has got.
You know what is called, you know, a construction which is a closed kinematic chain. It's called a parallel robot, right? So the way it works is it's going to do this, this movement in and out and up and down. So it's going to behave like a Cartesian robot, but the construction is different.
Three degrees of freedom and it is very, very versatile. It's very convenient to use it for a lot of automation operations. OK, now coming to so far we've seen arm movements. Now you can add a wrist.
To an arm and you can increase the number of degrees of freedom. So you can take a wrist as an attachment and you can just attach it with the arm, right? So now in this particular case, you see you've attached a wrist with which has got more degrees of freedom to a basic arm and you're able to get.
More degrees of freedom, right? So.
Now the wrist will have a few degrees of freedom like this, right? So in this case, pitch, roll and yaw, right? So whenever you get a rotation about the vertical axis, that is called yaw, right? Any rotation about the vertical axis is yaw.
Now if there is rotation.
In the forward backward direction, that means this part is going down and up right. Then that is a pitching motion or pitching movement. Now if it is making this left and right rotation, then that is a rolling movement, right? So when you are driving your car and when you're when you're making.
A sharp turn, the body of the car actually rolls, so that left right movement is called A roll. Front and back is called pitching, right? And movement about the vertical axis is yaw. So you can attach a wrist to an arm and you can increase the number of degrees of freedom, right?
So this is something we discussed earlier. So the degrees of freedom of a robot is used to describe the number of directions that a robot can pivot or move or or rotate a joint. OK, so now we already saw.
Uh, these different uh.
Movements. Now from this we can actually write the joint notation scheme. Now here what? What configuration is this? Now again, what configuration is this?
Yes.
Now, now I think you you got confused. After seeing all those different arms, you got confused. OK, it's articulated, right?
Articulated configuration now TRR. So I can write it like this starting from the base TRR right? So this is the joint notation scheme. So based on these.
Joint notations. We can we can write the joint rotation skills. We we did that earlier also. Now for a polar arm, so this part is twisting, this part is rotational and this is linear right? So we can write TRL. We just did that earlier also.
Now what is this? What arm is this?
Yes, what arm is this?
OK, good. It is cylindrical, right? So cylindrical I can construct like this also.
Now I can do this.
O This kind of construction is also possible.
So in this case this is made as a V This is possible then.
I can give two other degrees of freedom, right? So I can make one movement up and down like this. I can put one movement left and right like this, right? So here the way they have constructed this particular arm is.
OK, let me do it this way, the way they have done it.
Now I can remove this, then I can just put a base like this.
So one linear movement, then AV and an L, right? So LVL. So starting from the base, I can write this as LVL. So cylindrical arm, but slightly a different way of constructing. Instead of using AT, we can actually use AV here.
The way you're constructing it, so LVL. In this case it is cylindrical. OK, now this is fairly straightforward. This is a Cartesian arm, so LLL, right?
OK now.
OK, now here for the following arm and body designations, describe the robots using sketches. Now can you do this now? Can you construct a robot?
With the join notation scheme that they've given.
Uh.
Here what? What do you have here?
See, now here they've just given something randomly. This is for the exercise, right? For Polar, yes, it is TRL, right? Now you can try with other combinations also. TRL is the usual thing that you see, right? You can try with other.
Join types also and see if you can get a polar. It's possible. OK, so now for what is given here, can you construct a robot? Now let's try right? So LLR, right? So let me try.
LLR so I can do something like this.
O1 linear.
Movement like this. Then if I have to give another linear movement then it has to be perpendicular. Only then I'll I'll be able to reach the other direction, right?
So L&L I got so LLI got now R.
If I have to give an R, then I have to be careful.
I have to reach.
A perpendicular direction. Only then I'll be able to navigate within a three-dimensional space, right? So that means I have to turn about the vertical axis like this, right?
So in this case, this is an LLR, right? Don't ask me whether this is going to be useful or not. It depends on the kind of operations it's going to perform. So now if I have to.
OK, now Srinath, so to answer your question, see now in the case of V this is.
A revolving joint, right? So.
One link is perpendicular. The joint pin is aligned with one link and perpendicular to the other. So this V joint.
You can actually construct here, right?
So yes, you can have the mechanical arrangement in any way, but from the representation it's AV. OK, so now coming to this LLR.
Now if I have to sketch the work enveloe of this.
Uh, I'll get.
Something like this. Then if I include the vertical movement also then I'll end U with something like this.
Right. So when you construct an arm, you have to make sure that whatever joins given you should be able to move in three-dimensional space, right? OK, so this is 1A robot and its work envelope. Now second RLR.
It looks very uh.
Again, challenging. Now you can try. See now this is taken from one of the reference books. So now R.
So let me fix this part R.
So.
R then L.
So this is going to give me L.
And 3rd R.
Again, if I if I put.
A third R like this, then I'll not be able to navigate within a three-dimensional space, so I need to.
Put it like this.
And put an axis like this so that I get this three-dimensional movement. So what is going to be my work envelope? Again, a very funny looking.
Yeah.
Then.
So you know, a three-dimensional spherical work envelope is what I'm getting right? No, something like a spherical shell is what I'm getting. So this.
Can move about this way.
OK, so you have. The only thing that we see here is we have to move and reach a three-dimensional space RLR. OK, the third one is LRR.
O How can you do LRR? Let me try like this.
O This is L easy.
Or I can do like this.
R like this and the third R.
I have to do the third hour has to be in a different direction, otherwise I will not be able to get a three-dimensional movement right? So here also it is. Now let's try in some other way.
OK, now this.
OK, so if I do this.
Then if I do this, then I'll get a meaningful movement, but it is not R. This becomes LVR, right? So then it becomes a Scara. But here I cannot do that. Now what they've asked is LRR.
Now let's see if we can do something like something useful with it.
So uh.
2nd R.
Not helping.
If I do this, I'll get see now if I try and do this, sorry if I if I just do this, if I call this as L&R and R, what is the? What is the problem with this arrangement? Anybody?
If I do this, yes it is LRR, but what is the problem with this?
Any particular problem? Oh, good.
Good. Yes, it is on the same lane. So this is the problem. So that means I cannot have this construction. The only thing I can do is just like any other.
Just like the previous two, I have to do this right? O then I'll get a three-dimensional space like this.
Right then this L So now this is going to give me a movement like this, then you know a movement like this and then I'll be able to pull this backward also.
Also, right? So I'll be able to move this backward. The entire thing I can just move backward. So I'll get, you know, a three-dimensional work envelope, right? So LRR. Then what else do we have? LVR. LVR is good.
LDR is fairly straightforward.
So L.
V is this?
So this is L.
This is V Now what can you do here? OK, R in this case is this right? So this is a scara.
So this is a SCARA configuration. So LVR is nothing but SCARA.
OK, now what is the last one? LLTRL, right? So LLTRL. So what should we do here?
Now TRL is something we already know, right? So TRL is.
PRL is a polar robot, so this is T.
R.
And L.
OK.
OK, so now what are they asking us to do? LLTRL? What should we do here?
Anybody.
TRL I've just, you know, sketch put on two rails. Good. So I can take this and mount it on a slide like this.
So this can be.
One rail. So that means I get one additional movement in this direction, right? So I'm able to get one additional movement in this direction, one linear movement. Now with this arrangement, I'll be able to make the robot move along a straight line, right? That means, you know, imagine having.
A row of workstations along a wall, right? You have a lot of workstations arranged along a wall, and you want the robot to just drop objects at every workstation and then go, right? So in that case, you can just put the robot on a long rail like this and it can just perform a task at the same time.
AME time it can just move along a straight line like this. Now if I want the robot to move around in a room, what I can do is I can just put this.
Uh, you know.
Like this.
Right now I can just put this on a rail in this direction.
Like this. So in this case I get this movement as well as a linear movement. So that means it can move along a plane. At the same time it can perform the operation. Now if I have to put this on the floor of a room, then nobody can walk on the floor, right? So if I put this on the floor.
Can you walk on the floor? The robot will be moving on the floor, threatening everybody. So what I would do is I would put this upside down. I would just put these rails on the ceiling, put the robot upside down, right? So in that case, yes, at least you can walk on the floor. When the robot is moving, you just run away, right? So that it won't hit your head.
So that is possible. So you can just put the robot upside down. That's one option. The other thing is you can put the rails on the wall also, right? So that means the robot will be able to move along the wall and you can stay away from it and you can make all the workstations accessible to the robot. So all those.
Things are possible. So a typical example is a spray painting shop where they just put this robot on a rail and put it along a wall so it can just move along the length of the vehicle and it can do the spray painting. So that is something possible here, right?
So this is it. Now if a question like this is given, you should be able to sketch the configuration of the robot. You should be able to sketch the work envelope of the robot.
Right. OK, now let's uh.
Try.
I know. Let's see if you can answer all these questions. OK, now here I have some questions.
You know for the exam you're asking or for a real application?
For the exam, you just make a sketch to show the concept clearly, right? So you don't have to mark any dimensions or you don't have to measure each and every line, right? Just love just how I sketched it. If you show a sketch which people can understand, that is fine.
OK, now in this case, here you have an object which is placed at a point. Now you have to move this object and place it inside the slot. How many degrees of freedom do you need?
Yes, any answer?
How many degrees of freedom do you need?
OK, people are giving answers. OK, I see two.
Any other answer? OK, I see a lot of twos coming.
OK, good. Now you have to lift this object, right? So that will require one degree of freedom. Then you have to move this object to this particular location. Then you have to move it down. So that means lifting it up and down, that requires 1 degree of freedom and forward backward movement will require one degree of freedom. So this is 1.
This is 1 S. You require 2 degrees of freedom for this particular task, right?
OK, yeah, how many degrees of freedom do you need? You have to ick this object, lace it inside the slots.
OK, I see trees, some trees.
OK, one person is giving two. OK, now you have to lift the object up right for moving. Before moving you have to lift the object up and while placing it you have to move it down. So the up down movement is definitely required. Then you have to move it along a straight line, so that means you are coming along this line.
And to move it to any particular position, you need forward backward movement, right? Now you have to reach other slots. That means you have to move in this direction right forward and backward. So 123.
So 3 degrees of freedom will be required. Minimum minimum 3 degrees of freedom will be required in this particular case, right?
OK, here.
In this case you have you have three different shells, right? And you have to place the objects.
Inside the slots, right? OK, I see a lot of threes coming.
See, now this is one way of estimating how many degrees, sorry, how many degrees of freedom you require for a particular task. Right now you may have a simple task like this, right? So in order to find out how many degrees of freedom will be needed, based on that you can choose a robot, right? So.
Here you have to lift the object. Definitely you have to lift the object and you have to place the object inside a slot. That means you need up and down movement. You need forward and backward movement, right?
Uh, and you need left right movement.
To reach all the slots you need left, right, forward, backward movements. But with the up, down movement you can actually go up and reach the next level. You can go up and reach the next level, so that means you require one.
In this direction, two in this direction, three in this direction. But with these three degrees of freedom, you'll be able to reach all the different levels, right?
OK, now sure.
Yeah, you have to place your object in these slots. How many degrees of freedom will be needed?
OK, I see a lot of threes here.
Any other answer? OK, I see threes and fours.
OK, so now just like any other object, you have to lift this also, right? So that means for lifting you require one degree of freedom. Now to move along this direction, you definitely require one more degree of freedom. So this is one, this is 2.
Now if you see this slot, this is away. So that means even to reach to this location you need an additional degree of freedom, right? So you need to move here. You'll require a third degree of freedom, but if you move to this location.
You'll not be able to place the object, right? So that means you have to rotate the object.
You have to rotate the object a little bit to align it with the angle of the slot, right? So in that case you will definitely require a minimum of four degrees of freedom, right? So now this is.
Said without, you know, see now, if you're not making any modifications, see now somebody will argue, you know, what if I rotate this cylinder? Here we are not rotating the cylinder. We are keeping the cylinder in its position and we are just finding out.
How many degrees of freedom will be needed for the robot to perform this operation right? So here we are not customizing. See now if you rotate the cylinder then that means the operation is getting customized here. So if you get a different part then you'll not be able to do the same thing.
So now you have to look at this particular requirement. You can you can always have more degrees of freedom when you select a robot. You should not have less, right? So in this particular case you require a minimum of four.
Right.
OK, here for this conditions.
For this case, yes.
OK, I see fours, I see 544.
OK, 3-4, OK, see now we have to lift it and place it, no doubt.
Then we have to move this object along this line, so definitely linear movement will be required. Then this movement will also be required if you want to reach this particular slot then.
This slot is oriented like this, so that means this rotation is needed.
Right. So 1234 right now, not just that. If you see this is rotated in this direction, right? So that means you have to align.
This in this direction and you have to place it right. So again, you have to be very careful. In this case, the this movement has to be in the last degree of freedom, right? So only then you'll be able to reach this particular point and then you'll be able to move it inward.
So a minimum of five degrees of freedom will be needed. So you need one more rotation in this direction will be 5 S minimum five and you might actually require more, right? So a minimum 5.
Minimum 5 degrees of freedom will be required to perform.
This operation, right?
For the previous one.
See now 123 and four. See now 4. Previous one was 4. Now in this case you have one more curve, so you get 5, right?
OK, now let's look at one other case. OK, now here.
OK.
So you have to reach every location and then a last twist is also there, right? So 6 degrees of freedom needed a minimum minimum of six will be needed to you know if you're going to reach this particular object from one location, you will require 6 degrees of freedom.
Now the problem is, even with six degrees of freedom, you'll not be able to reach all the slots, right? So the reachable ones, you'll require 6 degrees of freedom to go and perform this operation. Now there'll be a slot on this side. Now the robot will not be able to reach.
That slot so that again, in order to reach a slot which is hiding on the other side, you'll actually require more degrees of freedom. So in this particular case, since there is one additional rotation compared to the previous one you have, you need 6 degrees of freedom, right? So.
Now this is one way of calculating right now for complex operations you have to make sure. See now this will not be sufficient if you're gonna deal with automating a very complex operation. So if you're gonna deal with a very complex operation or for example if you see robots doing.
Spot welding on a vehicle body or robots doing arc welding on a vehicle body. So they'll have both the arrangements. That means the robot will have all the degrees of freedom required to reach the space, and they'll also have arrangements to rotate the object also. So in that case you'll get additional degrees of freedom.
So they'll lift the entire body of the car and rotate it, and the robot will be able to perform the operations. So if you want to get more degrees of freedom, then you have both the arrangements. You have all the degrees of freedom on the robot, then you have other arrangements to lift the object, rotate it, turn it, so that way you'll get more.
Degrees of freedom. So if it is a very complex part, then you can you have to have such arrangements, right?
You can. You can actually have a prismatic joint. See now here when I say this particular movement, then I I mean a prismatic joint, right? So now you can do that with rotation also, right? If you make the robot large enough.
Then you can achieve that movement using rotations. Primarily it depends on the work envelope of the robot and the number of degrees of freedom of the robot. So first thing is the robot should be able to reach the space. So that means the entire working space should be within the reach of the robot. That means within the work envelope of the robot.
And not just that, the robot should be able to reach every location in the correct orientation to perform the operation. So you have to ensure that also. So in this case, if you're going to insert an object into the slot, then the robot has to come.
To this orientation and then perform the operation. So can the robot do that? So you have to ensure that right? So the guy who's selling the robot will simply sell the robot and go away. But before buying it, you have to make sure that the robot will be able to perform this operation. For that you need to understand.
The number of degrees of freedom required. Based on that, you can select the robot. If you're going to purchase a robot, you have to know this. If you're going to design a robot, yes, you have to clearly understand the requirement, right?
See now, yes you can. As I mentioned, you can perform certain operations using pure rotations, right? But while assessing the number of degrees of freedom needed, you just see from the operation that you have to perform, right?
Based on that, you find out how many degrees of freedom will be needed to get an initial estimate. So what you are doing is we are getting an initial rough estimate. Then if you have a particular robot configuration that you are checking, whether that particular configuration will be able to do the job, then you have to see.
If you can get the required movement with that configuration, right? So again, it should not be customized for only one part, right? So if you have many different parts where an assembly operation has to be done, then you have to check for all the different parts that you're going to deal with, right? So overall.
This is a starting point. I'm not saying this is going to give you a perfect final answer. No, this is a starting point. You can estimate this way. Then finally, depending on the complexity. Now, as I mentioned, if you have to reach from this direction, the robot will not be able to do no matter how many degrees of freedom you have.
So in order to reach a spot under the cylinder, then you need more number of degrees of freedom. So in that case you have to have more arrangements for rotating the object. Now you can actually watch.
Videos on assembly operation on vehicle bodies or if you see any video on spot welding of a car body or something like that, you'll actually see that they lift the entire car body and rotate it and present it to the robot.
And the robot will have, you know, minimum or six, minimum of six or seven degrees of freedom. So that's how you automate a very complex operation, right?
OK, so looks like that's all the time we have for today's lecture, so we'll stop here.
So if you have any quick questions you can ask.
Everything is fine. You are able to follow.
So next week we'll have an additional class on Sunday also, right? So if you if you check your calendar, you'll see an additional class scheduled on Sunday also. So that is just to compensate for the class cancellations on the 16th, right? On the 16th we have.
WILP convocation, so we may have class cancellations on that weekend. So in order to compensate for that, I'm just scheduling a class next week itself on Sunday.
WhatsApp group will be created when the lab starts, so that will start probably, you know, two weeks from now.
WILP convocation will be on the 17th, so 16th weekend classes may be cancelled for some faculty members, right? So those who are attending the convocation, you know, might cancel their classes.
So in order to compensate for that, I'm just scheduling it next week. So next Sunday you'll have an additional class. Don't miss it, OK?
Any questions?
Slides will be there in Teams right? You can go to see now if you want to see where the slides will be located. See now you have to click on Teams, go to this course Mechanisms and Robotics, my name, then Files.
Right. Yeah.
PPT files. If I click on them, then you'll see the slides there, right? So you can get the slides from teams. So I'll just load the slides after the lecture, OK?
Two classes at the same time when next Sunday.
Next Sunday, for what subject? Anyway, I'll check, I'll check.
What other subject do you have at the same time?
Yes, it's selective. OK, what is the name of the paper?
Anyway, I'll ask them to check, but that's the slot I got. OK, MEMS. OK, so very few people will be taking that elective. OK. Design for additive manufacturing. OK. Anyway, I'll just check. OK, if there's any change, I'll let you know. OK then.
So I'll stop the recording.

GLYNN JOHN . stopped transcription