Mechanisms and Robotics (Merged - DEZG561DMZG561ESZG561)(S1-25) - Prof. Glynn John-20250809_175115-Meeting Recording

August 9, 2025, 12:21PM

2h 3m 18s

GLYNN JOHN . started transcription

0:09
Just a second.

GLYNN JOHN .   0:18
OK. So in today's lecture, we'll discuss some fundamentals of robot technology. So we'll start with robot specifications and then we'll go through some analysis of mechanisms.
Now in the last class uh.
We discussed some of the joint types and the degrees of freedom, right? So we'll continue from there.
Just A . second.
OK.
Now in the last class we discussed ARM configuration, right? So we just discussed 4 basic ARM configurations and then one additional ARM configuration was A . SCARA. So now when we add A . wrist to an ARM, right? So now if you look at an ARM configuration, so the ARM.
Types were actually created based on the number of revolute joints and the number of prismatic joints and depending on the work envelope of the arm right now.
The basic four arm configurations were based on A . three degree of freedom arm, right? Now if you take an arm and add A . wrist to it, right? So.
If you take A . wrist like this right now, for example, here we have wrist configurations. Now we can add A . wrist to an arm and we can increase the number of degrees of freedom. Right now in this particular case we see two axis RT wrist, right? So now in this particular case.
Is this part will be attached to the arm so so you have to come from the base like this. So this comes first R.
Comes first and then T comes after that. So this is an RT wrist right? Now the reverse of that is also possible. So if you join this part with the arms, then if you come in this direction, T comes first then.
R comes later. So T is the twisting joint, R is the rotational joint. So in this case this is A . TR wrist. So we can take this two degree of freedom wrist and add it to any arm and we can increase the number of degrees of freedom.
Similarly, we can also have A . TRT wrist, right? So we can attach this part to the arm and from this direction this is read as TR and T, right?
So this will add A . few degrees of freedom to the arm, right? So this is A . twisting joint. This is A . rotational joint. Again, this is A . twisting joint.
Now we can also add A . wrist which has got this pitch rule and your configuration. So now here if you see there are three different axis and three different directions, so.
A rotation is possible about each axis. So now this front back rotation is always known as pitch. Now rotation about the axis of the arm is always roll, right? So if you rotate about the axis of the arm, so which is this axis.
If you rotate about that axis, then it is roll. Now always rotation about the vertical axis is yaw, right? So pitch, roll and yaw. So in this case you have the axis oriented in three different directions and A . rotation is possible for you know about each axis, right?
So again, this can be added to an arm and you can increase the number of degrees of freedom, right? Now you all can hear me, right?
Hearing is not A . problem.
OK. Thank you.
OK.
OK, now you can take any of these risks and add to A . regular arm and increase the number of degrees of freedom. So for example, if you take A . three degree of freedom wrist and add it to A . three degree of freedom arm, then you get 6 degrees of freedom automatically, right? So.
Risk can be made as standard products which can be attached to arms, right? Any particular arm. OK, now what are the challenges in dealing with robots, right? So what are the challenges involved when we are designing robots?
And then we're using them. What? What kind of challenges do we come across right now? Robots require higher mobility and dexterity, right? So now they have to move.
With maximum degrees of freedom. So that's one aspect. The other thing is, even if the task is very complex, right? So you have to still do it now. Of course, replacing human beings will always be difficult, but when the task requires much bigger weight.
To be lifted and manipulated. In that case, A . robot will clearly have an advantage, but there are always challenges. Now if you look at the structure of an open kinematic chain. So in this particular case, this is an open kinematic chain.
Right, so this is not forming A . loop. This is an open kinematic chain. So in this case we have beams. Now if you take this as A . structural member like A . beam, then it is supported on only one end, right? So now if you take this particular member.
It is supported on this side and on this side there's no support right now. If you take this member, it is supported on one side. It is attached to A . rigid grounded joint on this side. On this side it is.
You know loosely hanging right with an additional load of this particular link. So now if you take any particular beam, it is only A . cantilevered beam. So and again the joints are hinged joints, so that means they are supported by bearings and you will have A . motor stopping it or A . brake or A . belt stopping it.
In that particular position. So they're hinged joints, naturally not rigid and supported only on one side. Now if you have A . beam which is supported on both sides, like A . simply supported beam or A . fixed beam, then the beam will be A . lot more stable, right? So in this case it is supported on only one end, so it's not.
Naturally stable, right? So obviously this part can vibrate. If you move very quickly and stop, this part can definitely vibrate. Now because of the load, this can easily deflect, right? So all these problems are possible.
Right. So overall it is going to be poor in stiffness because it is supported on only one side, right? So now this is going to affect your accuracy, right? And your load carrying capacity. So just like our human arm, it is attached on only one side. So if you're going to lift A . heavy weight and stretch your arm.
Automatically you cannot hold that position for A . long time, right? So in in the case of A . robot, it can hold the position, but it it will still experience deflection, right? So overall the load carrying capacity is going to be affected. So imagine lifting A . heavy weight and stretching your arm, right? It's it's not going to be easy.
So similarly the robots are the robotic arm is also going to deflect when you stretch it fully with A . weight. So now that is going to affect your load carrying capacity and your overall accuracy. So these are some of the issues involved now.
When you have A . joint, you will definitely have A . small error, right? So now you can never design A . device with 0 error. That's not even possible, right? So we'll have an error of maybe .5 percentage or maybe .2 percentage, but we'll always have an error, either positive or negative, but we'll never have.
Have an error which is 0. It's not even possible, so errors will always be there. Now in this case what will happen is that you have all the joints in the series. Now one joint is here and another joint is here and another joint is here. Like that you have many joints in different orientations.
Now if you see there is going to be an error. Now imagine this joint has got an error now because of that error.
This joint is going to be away like this. Now this joint will also have an error. So because of that this is going to deviate A . little bit. Now the next joint will also have an error. Now because of that it's going to deviate even further.
Right, so the errors get compounded because the joins are in series, one after the other. Now if you have A . little bit error, see now again, we know that.
Uh.
This deflection is equal to this R Theta right? The deflection is you know R Theta. So now if your radius increases automatically, this deflection is going to keep increasing again. You know each joint is going to add some error to it.
And that is going to increase the overall error, right? So now errors accumulate because the joints are in series. So this is another problem, another challenge that you have to deal with, right? OK, so the position and motion of each joint is affected by the position and motion of the other joints.
So because they are in series now here there is a force known as Coriolis force. Now of course we'll discuss the details of that later. Now some of you may have studied already. Now what happens when you turn a link like this?
Now if you have a link which turns now while it is turning, if a mass on that is moving right, so there is a mass on the link. Imagine standing on a fan blade which is rotating right? So imagine there is a very large fan blade.
Which is rotating and you're standing on that blade, right? You're moving along with that blade. Now, as long as you're standing there, the blade is rotating. Now what you'll experience will be a centrifugal force, right? Because the blade is turning.
Because of inertia, you'll experience a centrifugal force trying to push you outside. So that is something now we all know right now while the while this blade is turning if you try to move.
Either inward or outward, right? Then suddenly you'll experience a force in this direction, either this or in this direction. So you are trying to move while the blade is rotating. If the blade is stationary, there's no problem. The blade is rotating at that point if you try to move.
Radially inward or outward, there'll be a force which will be pushing you in a perpendicular direction. Now that force is called the Coriolis force, right? So now force is equal to mass times acceleration. So now Coriolis acceleration will be that acceleration in that case, right? So if a force is pushing you.
In the perpendicular direction, then that acceleration is called the Coriolis component of acceleration, right? So now that is again caused due to inertia.
So now in the case of a robot, how will this happen? Now you may have a link, you may have a link like this. Now while this link is turning, this link will also turn right. So now overall if this link is turning like this, now if this link is turning, so that means the center of.
Mass of this link is moving closer, closer to the center of rotation, right? O. Now this will be equivalent to a mass moving inward.
Right. So while this joint is turning, the second joint is also turning when two joints are turning simultaneously. Now you can have multiple joints turning simultaneously also. So in that case the mass is either moving inwards or outwards. So because of that this Coriolis.
Force or call or his acceleration will also become significant because all these masses are moving very fast, right? Because we want the robot to move really fast. Now, because of this, a lot of forces are acting on the robotic arm, which you have to deal with, right? So that means you have to design for that force, and this force may cause stress.
Pose may cause deflection, right? So all these things will affect your accuracy, right? If it is causing deflection, then it's gonna affect your accuracy. So overall you have to deal with all these things. So these are some challenges involved, right?
Then the next part, the next challenge is in manipulation and control, right? So now you have an arm. For example, you have an arm. The arm has got so many joints here, here, here and because of all these joints.
Finding out the position of this particular point, right? So now you have so many links and so many joints, the the joints will change the angle, right? So the joints will help us change the angle of each and every link, and because of that we'll be able to move two different positions.
Now you have to perform a calculation to find out the position of the tool point, right? The the tip of the end effector where your tool is going to point right. So now in this case, if you have a drilling tool there, the tool is going to point there. So now that is the direction of the tool and the point here is known as the tool point, right?
So now you need to find out the position of the tool point at every instant. So that means the robot is making a small movement at every instant. You should know the position of the tool point. Now this is something we'll learn starting from the next class. This is called forward kinematics, right?
So similarly, if you want to reach a particular point, you need to calculate the angles and then move the arm, right? So again, this calculation has to be performed depending on how quickly you want to change the location, right? So that means you'll be picking an object from your, then you'll go to another point, then you'll go to another point. So every time you have to perform this calculation.
And then drive the robot to that particular location. So that means this requires computational power, right? So that means you need to have a very fast computer to perform this task, right? So if you want to move quickly and change multiple positions in a given second.
Then the computer has to perform these calculations quickly, right? So that again is a challenge. But with faster computers, it's getting easier. Now imagine the situation like 30 years ago, right? So with slower computers, now this thing can easily crash the computer. You may have to restart it or you may have to.
Fix some problems. So now overall manipulation and control. So one thing is performing the computation and getting the output from the computer. Now once you get the output from the computer, now who's gonna drive the arm to that location? The computer is gonna tell you that you have to go to.
This location. In order to go to this location you need to set Theta one, Theta two, Theta three and so on. Now each value will be calculated by the computer. It's going to give that. Then after that the controller has to drive each join to that location specified by the computer.
Right, so now that's what actually happens. Now we calculate. We specify a target location. We perform the calculation to find out what joint angles have to be set, right? So now that is done in inverse kinematics. Then the computer, you know, calculates all these angles then.
It gives those angles to the controller. The controller will drive each motor to make the arm reach that particular location. That means each joint has to go to that.
A specified angle, then the arm will reach that target point. So that forward kinematics. Once the angle is given, the controller has to set the angle by giving the signals to the actuators and that position has to be monitored by feedback sensors, right? So.
All these things have to happen only then the ARM can go to that location. So overall you see a lot of computation happening here and they should happen without any confusion or trouble or any crashing, right? So these are the challenges involved in.
Designing and building robots, right?
OK, robot specification. So any questions in the challenges? Any questions so far?
Yes.
Clear.
OK.
OK, fine.
Next topic is robot specifications, right? Normally when we buy a product, we check the specifications, right? So if you're going to buy any product whatsoever, right? So it may be a kitchen appliance or it may be a motorcycle or it may be a car, whatever it is. Now what we check will be the specifications.
Right. So how much power? How much speed? What mileage, right? What is the weight? So all these things are specifications. So normally any particular instrument or any particular device, vehicle, whatever it is, we check the specifications before purchasing it, right? So.
We need to understand how to specify a robot, right? Similarly, when we do product design, when we design a product, we again work with a set of specifications. So some target specifications will be given initially to us and based on that we'll perform the design. Then after we design it, we find out that the specifications.
Are either not met or either in some cases you may even surpass the specifications. So in that case you have to revise the specifications. So that is something we do in product design, right? So we start with specifications, target specifications and finally we revise the specifications.
When we purchase a product, similarly we look at the specifications, we make sure the specifications meet our requirements and then we go and purchase the product. Now if you have to specify a robot now in any case, in either case, either you're gonna purchase a robot, you should know the specifications that you need.
If you're going to design a robot again, you should understand the specifications clearly. So how do you specify a typical robot? OK, now here you can specify the robot by the.
By these things right now here by specifying the drive system, right? Speed of motion, load carrying capacity, then control systems, precision of movement and work volume, right? So apart from this, you know you can actually, you know, specify a few more, right?
So we look at what these things are. OK, so drive system. What is a drive system? Now we have to power each joint. So that means we have to have an actuator at every joint. Now what kind of actuator are we gonna use, right? So now we might simply.
You might simply think, why can't we put a motor and simply drive that particular joint? Yes, that's possible, but how much torque can a motor produce, right?
Now, again, that's a challenging question. Yes, a motor can produce any amount of torque, maybe several thousand Newton meters. It's possible. But how heavy is that motor going to be, right? So that's another question. Now, yeah, we can keep asking questions, but let's try to answer a few.
Now, if you're going to lift a very heavy object, now let's imagine you're going to lift an object which is going to weigh like 5 tons, 10 tons. So in that case, your electric motor may not be suitable.
Right, So what happens is that see now in the case of a joint, now the first joint. See now let me draw a simple articulated arm right now in this case.
OK, so in this case.
The first joint, you know there's not much of A challenge because the motor is sitting in a stationary part. Now if I have to just just imagine I'm just putting a direct drive, right? So here the motor is sitting in a stationary part. The weight of the motor doesn't matter much, right?
Now for the second joint, I have to put the motor at this location.
This location. So that means when this part turns, the motor will also turn. So the weight of the motor is going to affect the movement of the robot. Third thing, third joint, I have to put the motor here or somewhere here, right? If I use a belt drive, maybe I can put a motor here and then drive it using a belt or I could.
Put a motor directly at the joint. Now in this case, the weight of the motor is going to pull this arm down, right? So the weight of the motor is going to influence the design here. Similarly, if I have more number of joints, then I have to put more motors like this. So mounting the motors on a joint itself is going to be.
Challenging. And if the power or if the torque produced by that motor is very high, the motor is going to be very large also, right? So there is a limitation here. Now in the case of hydraulic drive, your motor is going to sit on the floor, right? And it's going to operate a pump and.
The fluids will be pumped to different actuators, right? So at every point you're going to have a Rotary actuator, so the the fluid is going to be pumped to each actuator. So the heavy object is sitting on the floor. No matter how heavy it is or how powerful it is, it is simply sitting on the floor now.
Only the hose is going to run to the actuator, right? So in this case, this is going to be a lot lighter, right? So again, hydraulic systems can be very, very powerful. Now we are familiar with our earth movers, right? All those JCB's which are used to demolish buildings.
Move a lot of heavy material. Now all these operate on hydraulic systems, so they can lift very heavy objects and the mass is going to sit on the floor, right? So in this case, hydraulic system is going to be faster. It's going to be very, very powerful in terms of lifting heavy objects, right?
So if you're going to lift objects in the order of tons, right? Several tons, 3 tons, 5 tons, even half a ton, right? So it's better to go for a hydraulic Dr. system right now if you're going to lift from, you know, half a KG.
3 kg, five kg, 10 kg like that, you know in in kilograms, right? Maximum 50 or 100 kilograms maximum. So beyond that you could go for a hydraulic drive. But and if you're simply lifting in kilograms, not in tons, then electric electric drive will be very convenient, right? Electric motors, smaller electric motors.
Motors will do the job. You can have gearing. You can increase the torque using gearing and belts and all that. Now it's going to be lighter. Electric motors will be easy to control. They're cleaner, right? So hydraulic systems may leak. They're a bit clumsy to deal with, but they're very powerful.
If you're dealing with lighter objects in the order of kilograms, then electric drive is going to be very convenient, easy to control, typically for smaller robots. Now, but that's what they mean by smaller robots, robots which lift.
In the order of kilograms right now, pneumatic Dr. can be used right if your accuracy requirement is not much right. So if your accuracy is not very important, you're gonna pick an object from one location.
Go stop at another location. You don't want to stop in between, right? So if you are moving from one point to another point, you're not stopping in between or trying to control the position in between, then pneumatic Dr. is OK and is a compressible fluid. So that means, you know, controlling it and stopping it, it's going to act like a spring, right? So controlling it and stopping it will be.
Challenging. So this is OK for simple pick and place material handling operations. Basic automation, simpler automation solutions involving low accuracy, but speed can be higher, right? It can increase the speed.
Uh.
You can add all that, but see, in the end of the day, the robot has to move fast, right? So you want speed and dealing with a compressible fluid is always a challenge and again.
The bike, you know, controlling the valve, you know in both directions, it's going to, it's going to oscillate left and right. So overall this is a challenging situation. Yes, you can have advanced drives to improve the situation, but electrical system will be a lot easier to deal with, right? So for any light application, electrical system will be.
A lot easier for heavy ones. Hydraulic systems will be fine. Lighter ones, you know, low accuracy, very simple task pick and place task pneumatic system can be used, right? So if you look at most of the commercial robots, you will not see a pneumatic system. You do not see any kind of pneumatic system in a typical commercial robot.
Most of them will be electrical because most of the operations will require lighter systems. Now if a robot has to lift the body of a car and turn it and all that, then you have to go for hydraulic system, right? So easy to specify the robot based on drive system here, right?
Now the next thing which is important for us is speed of motion, right? So if you want to reduce the cycle time, right? If you want to increase the productivity now, you have to act fast, so speed of motion.
Is another specification that you have to be careful with. Now you have to select carefully. Now the speed of the robot is gonna influence your overall output. Now speed is governed by physics, right? So just because I can sit on a motorcycle, suddenly I cannot reach.
100 kilometers per hour, right? So I have to gradually accelerate and then reach 100 kilometers per hour. Now if I have to stop, I cannot stop immediately. I have to slow down and stop because of inertia. So that means here it's not totally under your control you have to deal with.
Uh, nature or physics, right? So if you have to reach the maximum speed, now the maximum speed is here.
Which means that you have to accelerate and then reach the maximum speed. You cannot reach that suddenly. Now somebody might ask, you know, why can't I reach the maximum speed suddenly, right? So what will happen?
So in order to reach the maximum speed, suddenly you have to accelerate really fast, right? Force is equal to mass times acceleration. You need to get more force. Yes, you can. You need better acceleration. You have to apply more force.
Now you're going to apply the force, or you're going to apply a torque using a motor, right? Torque is equal to force times radius. So the torque has to be extremely high. Only then you'll get a force which is very high, which can move the robot really fast. Now of course.
Is equal to two Pi NT by 60, right? So that means if you need infinite force, you require infinite torque. If you need infinite torque, you require infinite power, right? So in that case, your acceleration can be.
Very, very high, right? So if you want to accelerate from zero to 100 kilometers per hour in almost no time, which means it says that you require infinite amount of power, right? So acceleration, force, force to talk, talk to power, so.
If you want infinite acceleration, you require infinite power. So everything finally comes to power, right? So which we don't have, right? We don't have infinite power. So we are limited by the amount of power we can use because again, we are dealing with dry systems, belts, pulleys, they have to.
Take that amount of power and again the links have to handle that power. So overall we have limitations everywhere. So for whatever power we choose, we can get a certain maximum amount of acceleration, right? So now here the maximum speed here.
You know you have to accelerate and then reach the speed. So now this part is what I'm talking about. You have to accelerate and then reach the maximum speed. You cannot suddenly reach it. So if I try to go reach the maximum speed like this in this direction, then I require infinite amount of power, right? So I cannot do that. So I have to.
Some time gradually increase the speed and reach the maximum. Now if you're making a short move.
Then I cannot reach the maximum speed. I will accelerate. I'll reach a certain maximum, then I have to stop. Then I have to slow down and stop. So I've not even reached the maximum speed here. So when I'm making a longer move, I have a chance.
So I reach the maximum, then travel with the maximum speed for some distance. Then I have to slow down and then stop, right? So here, if you see, higher speed can be reached by large robots with arm extended fully. So now how is the speed measured, right? What speed are we talking about?
Now if you take a robot, if you.
Extend it fully like this. Now add the wrist. End effector is going to be like this. Add the wrist. What is the speed? So now that is your maximum speed, right?
Now again, V is equal to R Omega, right? So Omega is your angular velocity, R is your radius, so V is your tangential velocity, right? So this is your tangential velocity. So your V is equal to R Omega. So it depends on how quickly you are turning.
And how much you're extending at that point at the wrist, you're getting a maximum speed. So the maximum speed is specified at the wrist, right? So if the arm is stretched fully, now how quickly can this joint turn? Based on that, you can calculate the speed at the wrist.
So here what they're saying is, you know, 500 degrees per second, you know, is considered good, right? It's considered, you know, high speed joint movement, right? So 500 degrees per second, so which is not much in terms of RPM. If you see it's not much, but the arm has to.
Move the weight, accelerate, decelerate and then stop, right. So this is the best you can get around or in that range, you know, slightly higher, slightly lower in that range you can get. Hydraulic robots tend to be faster than electric drive robots. Now the reason I explained, you have less inertia, you have all the heavy objects sitting on the floor, right?
Robots with electric drives have better control. Yes, we can control the motor using electronic circuits, you know, and you know you can control them better than controlling the fluid flow, right? So how much fluid flow is needed, you know, instead of controlling the fluid flow and.
Stopping a joint, we can simply control the shaft and justice stop the joint. So again, these are somewhat lighter robots. Again, easy to control, right? So now speed will also affect.
How accurately you're going to position the wrist, right? So again, the accuracy will be measured at the wrist, right? So now you are moving from here and you're going to stop here. So you want to stop at this particular position. That means you have to slow down and then stop, right? So.
If you have to stop accurately, you have to slow down well in advance and then stop at that particular location, right? So if you overshoot, then again you're not meeting that requirement, right? So overall, if you reduce the speed a little bit, then your robot will get more accurate. Now if you.
Do the opposite, then your accuracy will be lost, right? So that is again a challenge. Weight of the object to be handled. See now if you're dealing, if you're lifting A heavier object, then you're moving fast. Inertia is gonna be more. Inertia force is gonna be more. You cannot stop suddenly. So in this case you have to bring the speed.
Right again, distance to be moved is directly connected with this. How much can you accelerate and how far is that position and can you reach the maximum speed and then reach there right? So overall.
These are the things that will affect the speed of the robot, right?
Any questions on drive systems or speed?
Yes.
OK.
See.
Now when you're handling fragile objects, right objects which can break again, the objects can break due to many reasons. Now can you hold the object without breaking it? So that means you know how much gripping force you're going to apply on the object. So that is one thing that can break the object usually.
You will break the object while gripping it, right? So imagine lifting a large sheet of glass, right? If you lift it by holding it at one corner, automatically the weight of the glass plate will actually break it, right? So overall.
It totally depends, right? So now imagine you have a glass plate, so that means you need to have proper support.
You need to have proper support to lift it. If you don't support the glass plate properly, automatically it's weight itself is going to break it. Now imagine lifting it, holding it here, right? Automatically it'll break it. So you you just make one small movement, this is going to break automatically, right? So.
Again, this cannot be said easily, so you have to look at it, support it and see. And if you make one sharp movement and stop automatically, it's gonna break. So if this.
Sheet a glass plate size is very very large. Then you have to design A customized end effector to hold it at multiple points.
And then support it properly, right? So again, somebody will ask, can I move an egg without breaking it? Yes, you can. Again, you have to test it out. Try it, break a few eggs, then you'll find out, right? OK.
Compared to pneumatic.
Comma electric. So now you're asking which one is better? Is that your question, Vinod?
See, now which one is better? There's no clear answer as to which one is better. Now what? What weight are you going to lift? If you're going to lift several 100 kilograms, then hydraulic system is the only option, right? At that point you you may not get an electric robot. An electric robot will not be suitable for that.
So heavier objects, hydraulic now, lighter objects, electrical. Now if it is low accuracy, see now if you are gonna build an automation system with minimum cost, right with minimum expenditure if you want to build an automation system to automate a very.
Very simple process. In that case you can go for a pneumatic system. All you need is a compressor and a PLC and a few cylinders. You can actually build a system which can automate it, right? So that is possible for a very lightweight basic automation system. Now you can also build a robot like that, right? So with a very.
Basic accuracy. If you wanna move from one point to another point, you can put a hard stop and just start from one point, reach the other point. It's gonna stop there because you have a hard stop, right? So that way you will get that level of accuracy needed for a task, right? So if you wanna customize something, yes.
Pneumatic systems are OK. Beyond that, if you want to do spray painting or welding, in that case pneumatic systems will not be suitable, right? So in such cases, electrical systems will be better, right?
Do we learn about sensors, electronics in this course a little bit? You know, whatever is related to robots, you know we'll discuss. But of course you have learned about sensors right from your UG, right?
A little bit now, whatever is related to robotics, we'll actually learn. OK, now coming to robot specifications right now, we have accuracy, we have precision right now.
These are terms that are familiar to us right from, you know, basic metrology, right? So can we combine my systems? You can try nothing wrong in combining Dr. systems. But again, why? Why take the headache, right?
So you can combine nothing wrong in that. If you're gonna come up with a new design combining Dr. systems, yes, nobody's gonna stop you, but it'll be too much of A headache, right? OK, now accuracy. Now you have a target location, you're trying to reach that particular location, but you're not able to.
To reach it right, you're trying to reach it now. This is your error right now. This is your target location, but you're trying to go near it and that is your worst case scenario. That's your error now.
In other cases, you are able to go within this particular error, right? So in this case, your accuracy will be plus or minus this value, right? So accuracy can be written as plus or minus this error value, right?
So you want to reach a particular point, but you're not reaching, you're slightly away from it. Now how much away from that you are trying to, how much away from that you are reaching when you want to reach this particular target point, right? So that's your accuracy, so.
A.
In your instruments, right? So when you look at all your measuring instruments, we have an accuracy and we have a repeatability, right? So repeatability is your precision, right? So in the case of accuracy, you have an error. So now the worst case error is your accuracy, right? Plus or minus.
Because that error value is your accuracy. Now in the case of precision, you're trying to repeat. You want to reach this point, but every time you are reaching very close to the same location, right? So that means your accuracy may be this.
But in terms of precision, you're landing very close. You're trying to reach a zone which is, you know, near the previous point, right? Every time you reach a particular location, you're reaching.
A nearby point. Now in this case you can see that you're very precise because this error is smaller, but it's not very accurate because this error is larger, right? So precision is you know how closely you can come back to the same point again and again, right? So if you are.
Coming very close to the same point. So now every time you want to reach this point, you're not coming close to that point, but every time you're coming close to this point, right? So you're reaching this point. That means you're precise but not very accurate, right? So now if you want to reach this target point now every time you reach.
Something like this, right? You are reaching like this.
You're somewhat accurate because you are coming around the point, but you're not very precise because this distance is more, right? So this is a fundamental difference between precision and accuracy, right? So if you do calibration, then if you after calibration, if you're able to reduce this error, then you are.
ARM is going to be both precise and accurate, right? It's possible.
Now similar to your instruments. Now the accuracy is measured at the wrist, right? So the accuracy refers to the robot's ability to position its wrist at a target point within its work volume, right? So how close to that target point can it come?
So now it's measured at the wrist. Now whether it's load carrying capacity or accuracy, precision, everything will be measured at the wrist for a robot. The fundamental reason for that is, you know, the wrist is the extreme point according to the manufacturer.
Now the end effector is usually attached after you purchase the robot, right? So that means the end effector will be usually customized. The gripper or any kind of end effector that you use will be customized for that particular application. So the risk is what is going to be the same right according to the manufacturer.
So the accuracy and repeatability are not measured at this point. They're measured at the wrist, right? OK.
Now we have what is called a spatial resolution, right? Now there is something called a controlled resolution. Before we understand a spatial resolution, we have what is called a controlled resolution.
Now uh.
The robot will have a sensor, right? A feedback sensor which will allow which will give.
Or which will generate pulses at different points, right? So now you have a sensor. The sensor will generate pulses if you rotate a particular joint right now. How many pulses will you get per degree? Or how many pulses will you get per revolution right now?
If you calculate that now, for example, you have 360 degrees in a circle. Now when you rotate 360 degrees, your sensor is generating 360 pulses, right? It's generating 360 pulses.
That means you can locate only one degree at a time. That means the next pulse is generated. You move a degree right? Now, if you're generating twice the number of pulses, then you'll be able to reach half a degree, right? So now your control resolution is.
Controlled by the amount of the number of pulses your sensor is generating for every revolution, right? So if now if it is generating 720 pulses, then that means every half a degree you're able to.
Generate A pulse. So that means if you have a joint, now the joint is here.
Now the radius of the arm is, let's imagine 1m.
Right.
This is 1 meter and every half a degree.
You're generating 1 pulse, so that means you can reach this point. Now you convert this to radians and you multiply it with this R. Then you'll find out the length of the arc. So that means after you stop at this particular point.
The next point will be. Now you want to calculate that and check, right? So.
Two Pi radians is what?
360 degrees is 2 Pi radians, right? OK, so two into.
5 / 360 will give you the radians for one degree .017. So one degree is equal to 0.0174 radians.
Now half a degree.
This divided by two, right? So 0.00872 Radians. Now I multiply this with R * 1000, right? 1000 MMM. So now here that means this arc length is.
8.72 MM right? Imagine.
So 8.72 MM. So if your robot has got this resolution, then what accuracy will you or what accuracy will you have now here if you want to relate this to accuracy. Now if I have a target point at this.
Articular location. Now I can reach from this point to this point. Worst case scenario, my accuracy is going to be half of this.
Accuracy is going to be this divided by two is 4.36 MMM.
Very poor, right? So I would prefer an accuracy of, you know, .1 MM or you know, smaller than that. But here I'm getting 4.36 MM see. So that means 720 pulses per revolution will not be good enough for me, right? I'll probably require like 3000 pulses.
Or even more, right? So to get a decent level of accuracy, right? So now if you see in the worst case scenario, your accuracy is half of the controlled resolution. So that's what is mentioned here. You can see that now that's what I did there.
So that means from one point to another point, if I have to know or if I have to move now, I'll know only this target location or this address.
Or these are called addressable points, right? So I have this address, then my next address is this.
Address 2 is here in between. I cannot stop because my system doesn't know how to stop in between, right? So this resolution is called controlled resolution. Accuracy is half of controlled resolution, right? So that's what is mentioned in this particular.
Diagram. Now what is this spatial resolution? Right now here a definition is given. It's the smallest increment of movement into which the robot can divide its work volume. Now, which sounds very similar to controlled resolution, but the only difference is that controlled resolution will give you.
Movement from one point to another point. Now if you add some mechanical inaccuracies right now, every joint is gonna have a small amount of play, right? Every joint is gonna have a small amount of wear because of that.
This location will slightly change here, right? This location will slightly change here. Because of that you will get an approximate resolution like this. So that resolution is called spatial resolution, so controlled resolution.
You include that with mechanical inaccuracies, then you get spatial resolution, right?
OK, so now this is what I was talking about. South the ARM will be more accurate.
At the inner parts of the work volume compared to the outer parts of the work volume, because once you stretch fully, then you know this distance is going to be more. So this is 1 addressable point. This is another addressable point. You don't know what exists in between because you're going to jump from here to there.
So your sensor will directly influence your.
A resolution here, your spatial resolution, your control resolution here. Now of course you can improve it right now. For example, I have one shaft here, one pulley here. Now if I attach my encoder right at the output shaft, then my encoder's resolution will be very important. Now imagine I have a design like this now.
Now I have a motor with a smaller pulley. Now I put my encoder here, then I'll get slightly better resolution because I have a reduction here.
But if I have any play at, you know, at this particular location, then I will not have too much control, right? So it totally depends on your choice. Now it's always better to put the sensor at the output shaft, but the sensor has to have a very good.
Amount of, you know, very good resolution, right? OK.
So I think I confused you really well. So any questions on controlled resolution?
Spatial resolution accuracy.
Yes.
Means of achieving high accuracy in robots. You know you choose the sensor with the best possible resolution. You choose a sensor with you know reliable performance which which can produce gear system is not the best option. Gear will actually gears will actually give you.
Better reliability, but gears will not give you better accuracy. Now normally what they use is, you know, they use fiber reinforced belts normally and extremely high precision robots. They use, you know, polyurethane belts which are glass fiber reinforced.
Right. So you you can actually buy those belts nowadays pretty much everywhere. So even if you search in Amazon, you'll probably be able to buy those belts. So you have to design because those belts will not extend too much. So the extension will be minimum, very, very minimal.
So fiberglass fiber reinforced polyurethane belts you know are used in many of these semiconductor industry robots which are very, very accurate encoders, optical encoders with you know high so now you can see what kind of.
Resolution you can actually get right? You can actually get 3000 pulses per revolution encoders on most of the websites. Now again if they have two channel output then the.
The number of pulses will be multiplied by 4, right? So you can get fairly good accuracy with, you know, off the shelf components. You can you can simply buy these encoders. Nowadays everything is available, right? Probably 1015 years ago many these many of these things were not available in India, but nowadays pretty much everything is available, so.
It's not much of A challenge, but you have to buy it, try it out, and again, your shaft deflection, your bearing play, all these things have to be checked, right?
See now only the targeted points can be seen. Now, as far as the robot's controller is concerned, only the addressable points are the points it can see. Beyond that, it is totally blind, right? So it is getting pulses at those points. That means it can see only those points. It's blind between those points.
So you provide more addressable points, then your controller will have better vision, right? Or you know, better way of, you know, sensing those points. That's how I have to put it. You know, vision will give you a totally different meaning, but the controller will become totally blind if you don't give too many points, right? So.
Anything between the addressable points, the control is blind totally.
Now by that you mean you know you want to follow a path. You know if you want to follow a particular path, it totally depends on how you position the points in between, for example you have.
A path to follow like this. Now the robot will actually approximate the movement into straight lines.
So it will take all these points, then it will put straight lines and then you know it'll make it a curve. So if you have more number of points closer to each other, then this will become more or less a smooth curve.
Right.
I'd see now I'd see you can just like I mentioned addressable points.
Will be dictated by the sensor right now. Again, the drive system. See now, depending on where you put your sensor, if you put your sensor at the driven shaft, then whatever pulses it is generating per evolution, the same resolution is what you're going to get at the shaft, right?
Now if you have a reduction mechanism right now, this is what I explained earlier.
Now if you put your sensor here now for one revolution of this, or you know, just a second now for six revolutions of this, this will make one revolution.
Right. So in that case if you have like 1000 pulses.
1000 pulses per revolution sensor attached to this part of the drive. Then when this makes 1 revolution, you'll get 6000 pulses, right? So in that case, naturally you'll be able to increase the accuracy of the output shaft. You can actually get more addressable.
Points on the output shaft, right? So that is possible. But between these two shafts you should not have any play, right? So that will confuse or that will actually fool you into believing that this is very accurate, but in fact it may not be so accurate, but if you put it directly on the output shaft.
You will get exact measured information, but you have to have more resolution in the sensor, right? So it's your choice. If you have a very reliable mechanical system which is foolproof, yes, you can actually do something like this also.
You can put the sensor here and get better accuracy. That's possible, right? So we have to play with the mechanical system, the electrical system, and get the best what we can, right? OK, the next specification is load carrying capacity.
What do we mean by load carrying capacity? Now how much weight can you carry, right? You know, depending on the person, each person will have a load carrying capacity, right? So one person might be able to lift 50 kilograms, another person might be able to lift 100 kilograms now.
A weightlifting professional probably will be able to lift 200 kilograms, right? So each person has got a load carrying capacity. Now similarly, each robot will have a load carrying capacity. Now the load carrying capacity is measured.
At its weakest position in the case of a robot, right? So the load the robot can carry at its weakest position.
Without including the weight of the end effector while maintaining its specified accuracy.
Right. So that means if you have a robot like this, now the robot is extending like this from from this location. Now this is the wrist, right? So how much weight can you put at the wrist?
Now, because of this weight, the deflection should not cause the robot to lose its accuracy. Right now, of course, this arm will deflect. This arm will deflect, but that deflection should not be more than.
The specified accuracy, right? So weakest position is when the arm is fully stretched.
Right. So when it is fully stretched, it's the weakest. Right now you can try with your arm also, right? So you can just extend your arm, try to lift a weight. It's very difficult, right? When you move your arm closer to your body, it's easier to lift a larger weight. So weakest position is the one which is.
Which in which the arm is fully stretched, right? So at that point, how much weight can it carry without, you know, sacrificing or without compromising the accuracy? Now the weight of the end effector is not included in this, right? So which means that if the company says.
The load carrying capacity is 3 KG right the end effector.
End effector is end effector weighs about one KG.
Then the net load carrying capacity is equal to 3 -, 1 is equal to two KG, right? Because the end effector's mass was not included earlier, right? So when they made the robot, they did not.
They did not know how much the end effector is gonna weigh, right? So now you attach it after that. So now you have to buy a robot in such a way that you know the weight of the end effector well in advance. So that see, imagine the weight of the end effector is also 3 KG, then your load carrying.
The capacity will be 0, the net load carrying capacity will be 0. So you have to make sure the end effector is lighter so that you get maximum load carrying capacity, right? So the load carrying capacity minus the weight of the end effector at the weakest position. So that's your effective or net load carrying capacity.
So usually the end effectors are custom made, so you have to look at the kind of object you're going to lift. You design the end effector well in advance, or you purchase the end effector well in advance, or at least you know, get the specifications before purchasing the robot.
Right. OK, so these are the specifications of a robot, right? So if you understand all these terms, then you can actually specify a robot if you wanna buy one, or if you're gonna design a robot, then you'll know what target specifications will be reasonable, right?
OK. Any questions so far?
Any questions?
Yeah, you you will get the accuracy of every robot that you buy, right? Accuracy will be specified, right? So they'll simply say accuracy is equal to, you know, plus or minus .1, something like this they'll say. But you have to understand that it is at the weakest position.
Measured at the wrist, right? So this is something now very accurate robot will have an accuracy of plus or minus .1 MMM.
Right. So this is something that will be specified by the manufacturer, right? So yes, you will get the accuracy specified by the manufacturer.
Yes, any other questions?
OK, we'll just take a break for 5 minutes. Now when we come back, we'll discuss analysis of mechanisms, right? So if you have any questions, you can type your questions on the chat window.
Brajesh Kumar, I don't understand your question. Short method.
To do what?
I think every control system will have a closed loop feedback, right? So every every control system will have you know, a feedback control.
So you see now overall, if you want to increase the accuracy, yes, you have to increase the control resolution first of all. So that means your sensors have to produce more pulses and the mechanical system should not have.
Too much play or inaccuracies in the manufacturing or in the design, right? So overall the mechanical system has to be very, very precise. The sensors have to be.
With a maximum resolution and then the control system has to finally control everything perfectly, right? So overall the control system.
You know, controlling the entire system and stopping at that particular point. So that is always a challenge, right? So put together, you'll get your accuracy.
And explain which one? Which one? What? What is your specific question?
OK, so now when we talk about mechanisms, so now I actually gave you an introduction a few classes ago, right? So.
If you have a link.
Uh.
Which is connected to another link using a joint, right? So if there is any relative movement, normally when you call it a kinematic link, when there is some kind of movement, right? When?
When there is relative motion. So if so, if I have this particular part grounded, then the other link has to move with respect to the previous one. So if there is a relative movement, then you call it a kinematic link, right? So the moment you call something a link, then that means it is moving with respect to.
Something else, right? So if you look at the types, the most common type is a rigid link, right? So something which is solid, something which is very stiff, and it can move.
You know the movement is definite, right? So now that is a rigid link. Now if you have something like a rope or a chain, right, which is rigid in only One Direction. So if you pull a rope now in this direction, it is very rigid, right? But if you push it.
It is not going to be rigid, it's it's going to become flexible, right? So that is called a flexible link. So if you have a link which is rigid only in One Direction, then that is a flexible link. There are all other directions that is flexible, then that's a flexible link. Example is a rope or a chain.
Now third one is a fluid link, right? So if you have a fluid which is pushing a mechanism, right? So in a hydraulic system you have oil which is going to push a piston on the other cylinder and because of that the piston is going to move and actuate A mechanism, right? So in this case this oil.
Uh, in that hose is a fluid link, right? OK, now what is a structure?
What is a structure? Now if you have an arrangement, right? I have maybe an arrangement like this.
Something like this. Now I could support it at two different points. Now if I push it in this direction, if I push the top in this direction, it's not going to move. The way it is constructed, there's not going to be any.
Movement. It's not going to deflect at all, right? O in that case it is a structure right? Now I can have a structure like this.
Right. If I have a triangular construction, then this is a structure, so there's no relative motion. So that means, you know, I cannot move this link alone. It's not possible. It's not going to move, right? So in that case it is a structure, no relative motion then.
It is a structure, right? So assemblage of resistant bodies having no relative motion between them and meant for carrying loads. So that's a structure. Now what is a kinematic pair? So if you have.
Two links attached using a joint, right? Attached together using a joint. Now one link is grounded, the other link is moving, right? So in this case you have relative motion between them, right? So this is a kinematic pair, right? A pair of scissors, right? So.
In the case of a pair of scissors, right, you cut.
By holding one, see now here you're connecting these two with a joint. Now you will hold one part firmly, the other part will just move, right? So in this case, this is a kinematic pair. So one part is grounded, the other part is just moving, right?
So only then it is functioning properly, so it's a kinematic pair, right? So when you have relative motion between links connected or two links connected together, it's a kinematic pair. Now when you have more such links, then it's a kinematic chain, right?
So if I have more and more links connected together.
Then it's a kinematic chain.
Right. So now in in this case each link can move with respect to the previous one, right? So now the kinematic chain can be an open kinematic chain or a closed kinematic chain. So now when I have something like this.
It's a closed kinematic chain.
This is a closed kinematic chain. When the first link is connected with the last link, it's a closed kinematic chain. So I can ground this. I can ground one link, then I can if I move it. If I move this for example, this will also move.
This will move like this, right? So now in this case this is a mechanism. This is a four bar mechanism. In this particular case this is a four bar mechanism, right? So when one of the links in the kinematic chain is.
Fixed. Then this is known as a mechanism.
Now when you power this, when you put a motor here, right? So to power this, then it becomes a machine, right? So now this is something you read in basic kinematics. Now how do you find out?
Whether something is a mechanism or something is a structure, right? So we have a certain, of course, you know we we've discussed all these things, right? So in our simple link you have.
Two joins or you now you have the provision to have two joins, right? So.
Something like this. Now in some cases the link can be bent like this. You have the joint here, you have the joint here, so in between it is bent. So again this is this is a simple link with a point of interest.
Now you can have a complex link like this, but more joints possible, right? So you can have a plate like structure with more joints. Now you can attach more links to this.
So pin joint we already discussed, right? So prismatic joint or sliding joint we've already discussed. So you know, using these mechanisms, using these members, you can actually construct mechanisms, right? OK, now.
When you have a pin like this, so in this case if you have a slot like this and you have a pin like this, now the pin will have a rotational degree of freedom, right? And the slot is gonna give a.
A linear degree of freedom. So in this case you'll end up having two degrees of freedom at that particular point. So what you have to do is you have to split it into two members, right? So you can split it into two members like this and you connect a link separately, right?
So in this case this will be a sliding joint and this will be a revolute joint. So these are some constructions which you can see in mechanical systems. Now here this is a screw joint. We already discussed this. This has only one degree of freedom, right?
Now you can also have a gear pair, right? You can have a Cam and a follower. So all these are kinematic elements. So these are representations, right? So if you want to represent all these.
Using a kinematic diagram, then you use these representations for gears, for camera and follower, and so on.
OK, now here, in order to find out the number of degrees of freedom or whether this structure can move or not, we have a method right? Some of us must have studied this in second year of engineering.
Engineering, right? Some of us may not have studied, so it doesn't matter. So I'll just go through it, right? OK, now if I have an arrangement like this, right? Three links connected together. Now I could ground one of them. Now will this?
Be a mechanism or will this be a structure right? Or I may have a.
Mechanisms like this.
Where I ground this now, is it a mechanism really? Or I'm simply assuming that it's a mechanism or it is a structure? I don't know, right? Or I may put a member like this, right? I may put a member like this.
And now I may have the same question. Is it a mechanism or a structure, right? How can I find out? Now we can normally we can visualize A mechanism to a certain extent, right? But our judgment may be wrong at times, right? So what we have to do is we have to use an established method then.
We have to use. You have to use our intuition also, right? So again, you can't trust A1 formula like this and you know, say that this is definitely a mechanism, right? So you have to use your intuition. At the same time you have to use some standard methods. So here we have a kidsburg criterion, right? So now here.
We have an equation called Kuzbach criterion. Now M stands for mobility or number of degrees of freedom.
Or DOF degrees of freedom, right?
Now here we have a formula 3 into L - 1. So L is number of links.
J is number of joins.
Now H is the number of higher pair.
Now what are these higher pairs, right? If you look at a gear pair, now normally you'll have a small line of contact, right? So when you when two gears are meshing, there'll be a small line of contact. The contact will be along a small line, right?
In some cases you may have a contact at only one particular point. Now in this case you may have a situation where the contact is at only one point. In such cases now you call them as higher pairs, right? So gears, cams and so on are considered to be higher pairs.
Now pin joints and links connected using pin joints are lower pairs, right? OK, so in this particular case we have just three links and three joints and no higher pair. That means no gears or cams found here.
So we have 3 links and three joints. So in this case, let's look at this mechanism or this particular arrangement. Now L is equal to 3 and J is equal to 3 and H is equal to 0. So if I put it in this equation, M is equal to 3 into.
3 -, 1 -, 2 into 3 right? -0 So now this becomes 6 -, 6, right?
So 3 -, 1 is 2/2 threes are 6 -, 2 threes are six, so M is equal to 0. So that means mobility is equal to 0. So in this case.
This is a structure, so this is not going to move, right? So that's what this equation is telling us. Now this equation has been derived based on the number of degrees of freedom of each joint, and of course this has been verified.
And it works in most of the cases, right? In probably 95% of the cases, this equation will give you accurate results. There are some exceptions where this equation will not work, right? So there are cases where you have multiple parallel links where.
This particular equation may not work, but in most cases it will work right? So we can use that as a starting point. You check with this equation, then after that you use your intuition. If your intuition is not helping you, you build a model, you build a CAD model. If that is not helping, then you build a physical model and then you verify, right?
OK, so the next one, uh.
Why is the 0 here? I don't see any gears here. I don't see any cams here, right? You have only pins. See now you have only links connected using pins. So now those are called lower pairs because you have surface contact.
Now when a pin is connected with a link, you have a surface contact that is considered a lower pair. Now when you have gears and cams, you have a point contact or a line contact, so that is called a higher pair, right? So that's all I don't see any gears here.
I don't see any cams here, so I don't see any higher pairs here. H is equal to 0, right? OK, now the second one. So I'm just going to count the number of links 123.
Four number of joints.
123 and four right? So L is equal to 4J is equal to 4H is equal to 0 right? So 3 into 4 -, 1 -, 2 into.
4 right? - 0 So this is my M for the second one, right? So in this case it is 9 -, 8, right? M is equal to 1, so mobility is equal to one. That means this mechanism has got one degree of freedom.
So that means if I rotate this link, everything else is going to move except that grounded link, right? So this is going to move. This is going to move. So it has got only one degree of freedom because one link will control the movement of the other. So that means if you have one degree of freedom, then you require only one actuator.
Right, so we can have multiple variations of this mechanism. Now this is called a 4 bar mechanism. So modifying the four bar mechanism can give you many other mechanisms, right? So if this system has got only one degree of freedom, then you require only one actuator.
OK, now if you have two links connected by a joint.
This joint is called a binary joint, right? When two links are joined at the same connection, then the joint is known as a binary joint. Now if you see this L is a number of links and.
J is a number of binary joints. It's mentioned here as binary joints, right? So when you have more links connected at a joint, when you have more links connected at a joint, then that joint is called a ternary joint.
Right. So when you have a ternary joint in any mechanism, when you're counting that.
So this is known as a ternary joint. It's equivalent to two binary joints. So ternary joint is equivalent to two binary joints. So that means if I have to count this joint, I have to count it as two. Only then this method will work right? So if if I count the binary joint as one.
If there is a ternary, if there's a ternary joint, then I have to count it as two, right?
For 3D mechanisms, we have a different formula, slightly modified version of this formula, pretty much the same formula. Now the numbers will change because the number of degrees of freedom will change, right? So we'll we'll go to that, OK.
Now you can also have more number of joints. Now you can have a quaternary joint. You can have four links at a joint. So now for two links at a joint, you count it as one. For three links at a joint, you count it as two. So if there are four links, you count it as three, right?
So this is called a quaternary joint. Again, you can get more details of this in any kinematics book, right? So now again, you can.
Solve for this if you want right now. Can anybody tell me how many degrees of freedom this particular mechanism will have just by looking at it?
Yes.
OK, five. Wow, one.
Any other answers one?
3.
6/2.
Very interesting. OK, now if you look at this mechanism, the top one is a structure, right? We check this, right? The top part is just a pure structure. So that means that part is not going to move, it's simply going to sit right?
Now if you see the lower part, if you see the lower part, it is just a fourbar mechanism. The lower art is just a fourbar mechanism, right?
So in that case, the lower part will have only one degree of freedom. The upper part is just a structure which is sitting on it, so which will not have any influence. So this mechanism will have only one degree of freedom. Now you can analyze this mechanism by counting this.
As a ternary joint, this is a ternary joint. This is a binary joint, binary joint, binary joint. You can analyze it and see if you're getting one. You want to check.
Do you want to try it and tell me?
Just try this and tell me whether I'm right.
Do you want to try?
Now you have to be very careful. This is a ternary joint. This is a ternary joint.
Yeah, verify, verify please. I'm waiting for you.
Number of links. OK, so looks like somebody's confused. OK.
OK, yeah, yeah, don't worry about it. It's not such a complex problem. See, now this link is grounded, right? One link has to be grounded only then you have to check, but that is also a link. You have to count it. So now what I'll do is I'll count this one.
23456 right? So that means number of links L is equal to 6 right? Number of joins if I have to count the number of joins, so one.
123 comma 4 right? So this joint is 3 comma 4-5 comma six and seven right? So J is equal to 7 H is equal to 0 right? So you can check M is equal to.
Three into 6 -, 1 -, 2 into 7 -, 0, right? So 15 -, 14.
Equal to 1, right? So you can find out by looking at it. Now if you see this structure is sitting on one link, so that means that structure is just a part of that link and the lower part is just going to behave like a four bar mechanism.
So that has got only one degree of freedom. So this mechanism will have one degree of freedom. So that is again, if you try mechanisms like this, you know, if you try to analyze mechanisms like this, then you'll be able to visualize this quickly, right? OK, now let's try a few more, right?
OK, now, OK, now let's look at this particular case. Now in a typical robot constructions, we'll we'll have, you know, links and joins. So that's what that slide is saying. So we'll we'll look at that later. So now let's look at this open kinematic chain, right?
So in this case I have the frame as frame is 1, frame is link number one. This is link number 2345 right? So L is equal to 5.
OK, now number of joints 1234, so J is equal to 4. H is obviously zero, right? So M is equal to 3 into 5 -, 1 -, 2 into 4, right? -0. So this is equal to.
12 -, 8, so 12 -, 8. So number of degrees of freedom M is equal to 4 in this case. Now again, if each joint has got one degree of freedom, number of degrees of freedom of an open kinematic chain is equal to the number of joints, right?
So that is clearly verified here. So if you have 4 joints, each joint is 1 degree of freedom. You have total 4 degrees of freedom, right? Now let's look at this particular case now here.
Whatever is grounded. See, now this part is grounded. Whatever is grounded.
Is a part of the frame right now the floor is not moving. On the other end, the floor is not moving. So that means everything belongs to the same floor or everything belongs to the same building, right? So that's how you have to see it.
So if I count this as one link, the frame, that means the entire thing is the frame. So don't count 1 frame here and one frame here. You'll be wrong, right? Whatever is not moving is a part of the frame, right? So you count it as one.
12345 right? So L is equal to 5 J.
12345 right? So J is equal to five. So M is equal to 3 into 5 -, 1 -, 2 into 5 -, 0 right? So 12 -, 10 right?
Is equal to two degrees of freedom. So M is equal to two in this case. So this is actually A5 bar mechanism, right? So one link more than the one link and one join more than the four bar mechanism. Now in this case if you have to control this mechanism now for example.
If I have to control the position of this point, if I have to know where this point is, then I have to know this angle.
Then I have to know this angle or any any any side right? So either the inside or the outside, whichever one is fine. Either this you know this angle and this angle Theta one and Theta two should be known. Only then I can find out the position of this link.
If Theta one alone is known, Theta two is not known, then you know what? What can happen is I can rotate this here. Now this will change the position of this and this will change the position of this. Now point #3 can move even though.
Data one is fixed, still .3 can move. So if I want three to be, you know, estimated correctly, then I should be able to.
Fix Theta one and Theta two only. Then I can know where .3 is. So this is what we mean by two degrees of freedom here. So that means I need 2 actuators. I need 2 actuators, 1 motor here, 1 motor here. Then if I know these two values, then I'll know where that three is going, right?
OK, now let's look at this mechanism. Any guesses? Anybody wants to guess how many degrees of freedom this mechanism will have? C. Any guesses?
OK, Anand Rai is suggesting 1. Brajesh Kumar is suggesting 3.
Dalal won anybody.
That's all. No more guesses. OK, let's find out. So now here whatever is not moving is part of the frame. So all these are not moving, so they are part of the frame. So this is counted as one.
5-6 right? L is equal to 6. Now what about J?
So 1/2.
34567 right? So J is equal to 7 and H is equal to 0, so M is equal to 3 into 6 -, 1 -, 2 into 7 -, 0, right?
So 15 -, 14 is equal to 1. So M is equal to 1, right? So those of you who guessed one, you were correct right now one. The reason why I asked that question is, you know, if you see this part.
This is a four bar.
Right O that means this art is going to have only one degree of freedom automatically right now again if you see this part.
This part.
OK, you have slightly more than that, but still uh, it will not make it.
Uh.
OK, one.
Still, it is actually controlled by this particular.
Arm. So even though you have more number of degrees of, even though you have more number of links here, you're not able to do much right? So it is giving you only that because this particular plate.
Is shared by too many links. You're not able to get more degrees of freedom. Now in this case, if you see this part is a fourbar.
This part is also a 4 bar, 2 independent 4 bars connected at one particular.
Center plate, right? So again, this will give you only one degree of freedom, right? So now we can check that.
So I'm simply guessing by the four bar, not beyond that, right? So you can't simply guess it all the time. But when a four bar is visible, that portion of the mechanism will have only one degree of freedom. So beyond that you may have more movements, but you have to be careful when you when you try to guess, right? So this is.
123456 right? That's all you have. So L is equal to 6 J.
1234567 so J is equal to 7, H is equal to 0, so you have pretty much the same thing. So M is equal to 1, right?
OK, fine so.
So that's all you get. Now we have seen all these, right? So the answers are given here.
OK, now here we have something interesting, right? So now one thing which makes it interesting is that we have a ternary joint here. The other part is we have a sliding joint here, right? OK, now let's try this.
So.
This part is the frame, so everything is grounded here. So that means this is 123456.
7-8 right now again, this part, whatever is stationary, they're just connected, so that's it. So L is equal to 8. Number of links is 8.
Number of joints.
OK, so 123.
Four comma 5678 sliding joint nine right? J is equal to 9 H is 0 so.
M is equal to 3 into 8 -, 1 -, 2 into 9 -, 0, so 21 -, 18 right 3.
Unless we missed anything, this is 3, right? Yeah.
So now if you see this particular arrangement.
This part, this part 3456, right? So even if you leave this part initially, you see too many links are involved, so automatically you get more degrees of freedom.
Now the only thing which is restricting the degrees of freedom is this arrangement, right? So you can actually try and analyze this mechanism if you don't have the other two, right? So all those variations you can try.
Right.
It's not a binary joint, it's a ternary joint. Binary joint means 2 links at a joint. See now this is what I mentioned here.
OK, so ternary joint three links at a joint. You have to count it as two.
Ternary joint right? Binary joint is 2 links, 1 joint, ternary joint. 3 links at one joint, so you have to count it as two. If it is quaternary joint, it'll be 4 links at one joint. You have to count it as three, right?
OK.
Now we are familiar with this particular thing. Now here usually we'll have one joint here also. Now in this case we are just given the planar problem of this, right? So the planar version of this problem is given. Now if you look at this, the entire part is grounded. Now we have to assume that the entire.
The entire part is grounded. Only this arm alone has to be analyzed. So now just by looking at this, if you assume that the entire body is grounded, how many degrees of freedom does this arm have? Any guesses?
Any guesses?
Just by looking at it.
Now, one hint I can give you, right? Uh Somebody saying 21331 hint. You know, if you look at an arm, the number of actuators will be equal to the number of degrees of freedom, right? So in this arm, how
Many actuators are there or how many actuators do you see?
That will tell you the number of degrees of freedom. So that's one way to guess it, right? OK, now let's try to analyze it. So now this is grounded, so I'm just counting this as link number one. So this is 2.
Right. So entire beam or the boom, right, whatever you call it.
So that is counted as two, right? This is 3, this is 4, right? This is 5, this is 6, this is 7, right?
8.
9.
10.
11/12 right? So I've pretty much counted everything that moves and including the one that is stationary, right? So L is equal to 12. OK number of joints.
OK, so one.
Two sliding joint three. This one is 45 sliding joint 6.
9 sliding joint 10.
11/12.
13 Now this one is 14 comma 15 right? Because you see this is a ternary joint, so J is equal to 15. H is obviously 0 here, so M is equal to 3 into.
12 -, 1.
-2 J - 0 right? So 11 threes are 33 -, 15 twos are 30. So M is equal to three, right? So as I mentioned earlier.
The number of degrees of freedom is equal to the number of actuators. You see one actuator here, one hydraulic cylinder here, right? One hydraulic cylinder here and a piston. Third hydraulic cylinder is here, right?
So number of actuators will be equal to the number of degrees of freedom.
So I I have a lot of interesting answers here. OK, so last few have been three. OK, good.
OK, now I think we'll have enough time for spatial mechanisms right now. Here this Kitzbach criterion can also be called as Gribler's equation when you remove that hedge part, right? So higher pairs, we don't see them too much.
If you remove them from the equation, not much of difference it's going to make. So if you remove that higher pair component from the equation, then that equation can also be called as Grebler's equation, right? So now here we have the Grebler's equation for a spatial mechanism, right?
Now the only difference you see is M is equal to three for planar mechanisms, M is equal to 6 for spatial mechanisms. Now here the formula is written in a slightly different way, right? So I'm simply going to take this formula and use it right?
So I'll just take this formula and use it. Now M as I mentioned is 6 for spatial mechanisms. That means 3 dimensional mechanisms. M is equal to 6, right? So that means 6 into N -, 1 -, J, right?
Plus Sigma FI right? So this is the formula. Now here N is the number of links.
So now it doesn't matter. You can you can use L if you want. Hardly makes a difference. J is the number of joins.
Now Sigma FI is the total.
Number of degrees of freedom of the joints.
OK, so now anyway, I'll explain these terms. You don't have to be confused now, so let's look at some of the mechanisms.
See, now the bucket doesn't do anything on its own. So the bucket is. If you look at that mechanism, the bucket is just one piece. The entire bucket is just one piece.
Right. Which is pivoted at this particular point. So the entire bucket is going to move. So that is link #12. So in the count, the bucket is counted as link #12, right? You can see that.
So I right so.
That is just one link. Beyond that, there's nothing to explain there. Now in the case of an end effector, end effector is not considered as a separate degree of freedom. So end effector is just an additional mechanism. It is not counted along with the degree number of degrees of freedom of the robot, right?
OK, now coming to this part.
We have a spherical joint, which is possible in some cases. Right now we know that the spherical joint has three degrees of freedom, right? We discussed that earlier. Right now we have what is called a universal joint. Now a universal joint. Now if you look under a truck, right, for example.
If you look under a truck, you'll see something rotating, right? Most of us must have seen the drive shaft rotating. So now the drive shaft will actually have something like this.
A Hooks universal joint. It'll have a joint like this where one link is connected to another link with the help of a cross in between, right? So there'll be a cross like structure, you know, like this and the two links will be connected using the cross like this.
So now this is this is called a hooks.
Hooks Universal joint.
So now this term universal joint is used when you have one rotation like this and another rotation in a perpendicular direction. Now the same thing is possible here. Also, in the case of a Hooke's universal joint, you have one axis rotating.
Like this or one rotation is about the horizontal axis, the other rotation is about the vertical axis, right? So when you have rotation about two perpendicular axes, then that is called a universal joint. So it'll have two degrees of freedom right now cylindrical joint we already saw.
You know, one linear motion, one rotary motion. So we normally don't use this arrangement. We normally pin it and we use only one degree of freedom. Prismatic revolute. We have already seen helical. We have already seen helical has got only one degree of freedom. Prismatic has got.
One revolute has got one, cylindrical has got two, right? So we know these joints. The only new one is the universal joint here, which is just a combination of two revolute joints placed perpendicular to each other, right?
OK, now here they're just showing.
The number of constraints and you know the number of degrees of freedom, right? So 111 cylindrical 2, universal 2 and spherical 3. So that's what you see from this. That's what you get from this table. OK, now in this case.
We have.
A mechanism. Uh, OK, let me remove this. Sorry.
OK, we have a mechanism.
Uh, which is called the Stewart Gau platform.
Now they've just given a descripcion of this mechanism. Tewart Gau platform consists of two platforms. The lower one is stationary, right? So the lower one is stationary.
And that is regarded as the ground. The uer one is mobile, connected by 6 universal rismatic sherical serial chains. So that means this part is a universal joint because you see two.
Rotations about perpendicular directions and this is prismatic. Now here you have a spherical joint, so this is what they're describing. So now what you can do is now here you should not go into the details of what is inside each joint, right? If you have to analyze this mechanism.
What you should do is you should simply take it as a joint and you should not go into the mechanism of the joint, right? So now for example, if you go inside this joint, you have two links connected by a pin. So if you start counting all the internal parts of this, then it gets too confusing, right?
O You should not do that. You have to draw what is called the equivalent mechanism. Now that's what I'm doing now. O I'm just drawing something which I think is the equivalent.
Now I'm just calling this as universal joint, prismatic, serial, or sorry, spherical, spherical, serial chain, right? Spherical. Now all three put together. Now this is the serial chain, right? So like that I have 6.
Mechanisms O I'm just drawing the equivalent.
Mechanism.
OK, universal, prismatic, spherical strain. So I can just, you know, do it.
Like this UBS then.
OK, now something like this at this point.
Right. So SP and U. OK, so this is the equivalent mechanism that we have, right? So in this case, how to count, you know how to count the number of links and joints. Now let's go back to the gribbler's equation. OK, degrees of freedom.
DOF.
Is equal to 6 into N - 1.
Minus J.
Right plus Sigma FI.
Sigma FI, right? So this is the formula. So now I can count the number of links and the number of joins, right? OK, now let me count that. OK, so this is link number one, right? Which is grounded.
2.
To me.
Right 45678910.
11/12.
13 and finally 14, right? So that means N is equal to 14.
OK, number of joins J. How many joins do we have here right now? 6 joins at the base, 6 joins in the middle, 6 joins at the top, right? So now if I count just the number of joins, we have just 18 joins.
Right. But the problem here is the only complexity that we have. This is our spatial mechanism. Now each joint does not have one degree of freedom. So in the previous case, every joint had only one degree of freedom. But here it's not the case. So that's why we get this term Sigma FI, right? So here.
Now, how many universal joins do we have?
6 universal joints times number of degrees of freedom of each universal joint 2.
How many prismatic joints do we have? 6 prismatic joints. Number of degrees of freedom of each prismatic joint is just one, right?
OK, plus.
6 spherical joints. Number of degrees of freedom of each spherical joint is 3, right? So this is your Sigma FI, right? Simply counting the total number of degrees of freedom. So 6 twos are 12 + 618.
+ 18 So here we get 36, right? So now we have to put all these numbers in the formula. With this we'll get the number of degrees of freedom. So dof is equal to 6 into N is 14 -, 1 -, 18.
Plus 36, right? So that means 6 * -19 + 14 is -5, right? Plus 36.
So -30 + 36. So DOF is equal to 6. So now this particular platform with this construction has got 6 degrees of freedom, right? So in this case that that means the platform can go.
Forward, backward, right? So that means that this can go forward, backward, left, right, up, down. It can rotate, rotate about each axis, right? So just like an object floating in the air, this platform can move.
So this platform is used in simulators. So if you sit on on the top platform, you know on a chair, then you can be lifted, moved left and right, up and down. You can be twisted about any axis, right? So 6 degrees of freedom is possible.
For this particular mechanism, right? So Sigma FI. See now here at the bottom, how many universal joints do you have? 123456 universal joints.
Each universal joint has got two degrees of freedom, so you can see that here, right? So 6 into two, 6 * 2 prismatic joints. Number of prismatic joints 1234566 prismatic joints.
Each prismatic joint has got only one degree of freedom, right? Again, if you don't remember prismatic joint, one degree of freedom, right? Third one, number of spherical joints 123456.
Right, 6 spherical joints. Each spherical joint has got 3 degrees of freedom, right? 6 * 3, so totally you get 36, right? So you put all the numbers in that formula, you will be able to get the number of degrees of freedom, right? So very simple.
But the only thing is if you go into the mechanism or if you go into the joint and you know, try to count every pin and every washer there, then you'll go wrong. So you take a joint as one big unit.
Right. Uh.
And you count that as a joint, and then you include the number of degrees of freedom in Sigma FY, right?
On the number of actuators, see now if you have only linear actuators, you put all your linear actuators at the prismatic joint, then you'll be able to get the kind of movement that you want, right? If you have only linear actuators, for example, you have like mechanisms which can extend, extend and contract like you know.
Hydraulic cylinders. If you put 6 hydraulic cylinders at the prismatic joints, then you will get the kind of movement that you want. With this mechanism, of course the members have to be evenly spaced and if you correctly constrain them with the joints as shown here.
You will definitely be able to get the six degrees of freedom.
Yes, you got it right. So now you have to control the mechanism from a joint which you can, you know, easily activate, right. So that's that's the only way you can do it here.
Right. So with that, we'll stop here. Now there are a few more problems. We'll continue those in the next class, right? And with that, we could start forward kinematics, right? OK, then. Have a good night. Thank you very much.

GLYNN JOHN . stopped transcription