Mechanisms and Robotics (Merged - DEZG561DMZG561ESZG561)(S1-25) - Prof. Glynn John-20250830_175436-Meeting Recording

August 30, 2025, 12:24PM

2h 2m 33s

GLYNN JOHN . started transcription

GLYNN JOHN .   0:03
Ring started.
OK. Any questions so far on forward kinematics?
Off.
Any questions so far? If you have any questions on what you have discussed so far in in the class, in the lab, you can ask.
Yes, anybody.
Everything is clear.
OK. Any questions on the assignment, those of CNET?
No questions whatsoever, OK.
So.
You have understood everything or you have not even touched anything.
OK, you can see my screen right?
All of you.
Did I share my screen OK no, sorry.
OK. Can you see my screen now?
OK.
OK so in today's lecture we are going to discuss.
The inverse kinematics of a 2 degree of freedom planar arm. So we started with this in our lab class. Then 3 degree of freedom polar arm.
And a cylindrical arm, right, a slightly different cylindrical arm.
And an articulated arm, right. So we will start with these and we will also discuss the challenges involved. Right now, when we talk about forward kinematics.
We were given angles, right? So Theta one, Theta 2 and so on, right? And we were asked to find out.
The tool point right? So of course you know the values of the constants L1, L2, L3. Then the angles are given to you. Now from this we have to write the equation and find out the tool point. So this is forward.
Right.
Now if you look at a regular robot right now, for example.
You have an industrial robot and you want the robot to perform a certain operation, right?
Now the operation may be a pick and place operation, right? That's the simplest possible thing that you can do. Now if you want to. If you want the robot to pick an object from this location, right? So you want the robot to pick.
An object from.
Some location A and you want this to deliver this to another location B, right?
Now location a may be a conveyor where this object is being received right? So now this could be a conveyor.
Where this object can just come and fall and you have to align it to make it fall at a particular location and you have to stop it for the robot to pick it up. Right? So that means the normally you will have a moving conveyor now from a moving conveyor.
You will have what is called an accumulation conveyor. So from there you know the your object will just come and roll and stop at a point. So now so if you have an accumulation conveyor now depending on how you design it, usually for a regular.
You know cotton like objects. The accumulation conveyor will not be powered. So from a moving regular powered conveyor this will just simply roll and come and stop at a particular location. So normally if you want the object to be picked up by the robot then the object has to be in that.
Correct location and in that correct orientation. So by correct orientation what I mean is that if it is a cylinder and if the robot is going to pick it up in this orientation right, that means the cylinder kept vertical, then the cylinder cannot be lying down. So this is not good enough even though it is the same object.
You have designed the system for the robot to pick it up in this orientation. Then it cannot lie down like this, right? Or like this. Any other orientation is not OK so that right orientation has to be maintained. That right position has to be maintained. Now the reason is of course we know.
The robot cannot see, right? So you need to have the object at that location so that the robot will come and pick that object and then it is going to take the object to another location. Right now somebody might suggest you know we can have an advanced vision system for the robot to pick it up and make it intelligent. Yes, that is too much work and too much cost, right?
OK, a basic thing you you want the object to come to a particular position at that particular orientation. Now that means we know this point where this object is going to come and stop, right? So now you could have a stopper which will prevent the object from moving to the sides. Now you can have something like a funnelling arrangement.
See now while the object is coming from the conveyor, now you can have a funnelling arrangement which will actually allow the object to come and stop at that particular location, right? So something like that you have to design only then the object will come to that location. Now you know that location correctly.
Similarly, you have to place the object at a different location for further processing. It may be for another additional process, so you you know that location also. So in the regular engineering application you know the locations.
Right. So there is no need to find you know where this arm is going to go. So you, you already know that that location is already given to you. So that means P is known to you and in order to reach that particular location.
What angles are needed? Theta One, Theta 2, Theta 3 or further? You know even more number of thetas or some other variable right? So these values have to be found out right? So now this is inverse kinematics.
Now, why does it get this name? And you know now, why is this even needed? And what what are the challenges? There are so many things involved right now. Usually this target point is known to us.
Now we have to drive the arm to that target point, then pick it up and move to the other target location. So now this calculation has to be done frequently, right? So go to this point, go to this point. Then after that go to another point. So like that, this inverse kinematics problem.
Has to be solved by your software multiple times, probably in a minute or even in a second, right? So that is possible. So now what are the challenges involved in this? Of course we can write a program, or we can write, we can derive some equations to find the solution and all that everything is possible.
Now, what are the challenges involved? Now you can have multiple solutions now. First thing is multiple solutions. Now to reach this point, the robot can do this also. Right now if this is your target point, the robot can actually do this.
Right now this is also a solution.
So you need not have this angle and you know this angle.
But instead you could have this angle B, you know Theta 2 could be negative, Theta 3 could be positive. That is also possible right now. This is for 3 degrees of freedom. Now imagine if you have 6 degrees of freedom reaching a particular point. You could, you know, you could have even more number of solutions.
Right. So that means if you want only one solution, the other solution has to be either restricted. Right now I have to restrict this in my software so that this solution is not chosen right? So that is possible. You can restrict that in the software. Now the fundamental problem is you have multiple solutions, right?
So that is one problem. In some cases you might, while solving your trigonometric equations right now I showed an example earlier. Now if there is a point in the 3rd Quadrant, X is negative, Y is negative.
Now, if you find out tan inverse y by X now y by X is going to be positive, right? So then it is going to give you. The calculator will actually give you a positive value of angle, whereas the actual case it is in the 3rd quadrant. The angle will be greater than 180.
Right in the calculator will actually give you or your your MATLAB or any any kind of software will actually give you.
An angle which is from zero to 90, right? So that is again fundamentally wrong. So in that case you have to write a more number of conditional statements or you know algorithms to get the correct answers. So these are some of the challenges involved in inverse kinematics. So you have multiple solutions.
Then solving a problem itself is a challenge, right? OK, now we will start with a very simple problem, right? Of course, the forward kinematics equation for this was written multiple times already.
XYZ.
Now, is everybody comfortable writing the forward kinematics equations now? Do you still have problems writing it? Can you write it?
Uh.
Comfortably.
Yes.
Are you comfortable with forward kinematics? You know we have been talking about it for the past 34 classes, including the lab classes. Yes, OK. Some people are comfortable.
Some are still practicing. OK, fine. OK, so now in this case, we know how to write this right? First, we are gonna the best option is to put this arm in the home position.
So anyway, I am just repeating it for the people who are still practicing are still not clear, or people who are not answering.
Right. So first you put the arm in the home position. Now when I say home position, Theta is equal to zero. Primarily Theta is equal to zero. Now D can be.
Almost zero or at its minimum value. Right now I can put d like this because physically it may not be possible to fully retract it. So something like this is the home position. So now you are placing the coordinate system.
At the origin.
At the first joint and you are aligning the Z axis with the axis of the joint. So rotate Z, Theta one if you are going to cross that particular joint, you have to write this. Then you are going to move.
The coordinate system to this location to the end of that joint.
Right. And you want the Xerox is to be pointing?
In the direction of extension, if you want.
Or if you if you, if you face a condition where the joint the next joint is a prismatic joint. So whenever you have a prismatic joint, you want the Z axis to be pointing in this direction. Now you have to reach that particular position. So to do that I am moving my coordinate system forward.
Trans. XL One then when I do that, my coordinate system will come to this particular position. Now X is going to come here. Z is still going to be here and y is going to be like this.
So I am gonna do rotate Z 90 and rotate X 90. When I do this, Z is gonna we are gonna rotate like this and X is gonna go up.
Right. And Y is gonna point backwards.
Then when I do rotate X 90.
Rotate X 90 then Z is gonna reach this position.
And why will reach this position?
Right. And X is gonna remain there.
So this is T0 to one, then finally I have to come to this point where Z axis will point in the tool direction and X axis will be like this and y axis will be like this.
So in order to do that, trans Z.
T2 right, this multiplied by the origin.
This is T1 to 2 right? So these are the equations or this is the equation for this particular arm. Now you can break this into T0 to one T1 to 2. Right now when you multiply T0 to one.
And T1 to 2. What you get is T0 to 2, right? So that means from the origin till that end that matrix is T0 to 2, right? OK, now I showed you how to expand this and do the matrix multiplication right in the last class. So.
If you expand this and multiply then you will get a set of equations nowhere. We have the link coordinate diagram.
And the DH table. Now the DH table, I can just remove this and put it here, right? That itself is easy. OK, so now these are the equations. Now we have to solve these equations to get.
OK, now we have PXPY&PZ from forward kinematics. We just got these equations. If you expand all the matrices and do the multiplication, you will end up with these equations now.
If you know PXPY&PZ and if you have to find out Theta one and Theta 2 or sorry Theta one and D2, then what should you do? Now you have to solve these equations now. First thing if the equations are solvable, you can simply solve them.
So in this case the equations are solvable, so all I have to do is just 2 / 1.
Now if you the reason why I'm doing 2 / 1 is sine Theta / Cos Theta is tan Theta, right? So tan is something that we use very often, so I'm I'm just doing that. So in this case.
L1 + D2.
Sine Theta divided by L1 + D2.
Cos Theta.
Is equal to PY divided by PX. So I've just taken these 2 equations and I've divided them so L1 + D2 is common, so that means tan Theta is equal to PY by PX and Theta is equal to.
Tan inverse PY by PX right. Very simple straightforward solution. So we got the angle.
So we got the angle.
And from the angle we can find out the other variable. So I will take Theta and put it in equation number one or 2 right?
So once I put this in this equation.
Now the moment I put the angle then this becomes a number right? So L1 + D2 is equal to PX / Cos Theta.
So from this I can find out D2 PX / Cos Theta 1 -, L1 right.
So now the moment you know this angle, this part is going to be this part is going to become one number. So this is anyway a constant. So you can actually find out D2. So this is straightforward. So we just did up to this in our lab today, right, OK. So this is just a repetition.
For those who attended the lab class fine now, what are the challenges involved? So that is what I explained to you.
OK, now what are the challenges if I take my calculator OK? Anyway, let me open a calculator here. OK, so now this calculator has got.
OK, this is even worse. OK now anyway.
Second, OK, very interesting. OK, so now sine inverse Cos, inverse, tan inverse everything is here. So if I do for example, if I find out 45, if I find out.
Tan inverse sorry, sorry. Tan inverse of one for one. If I find out tan inverse.
Yeah, it gives 45 right now tan inverse of one is 45. So I can simply.
Uh.
OK. Why is that one coming now? Somebody might ask a question. OK, nowhere we have.
Let's imagine this is a graph sheet, right X&Y. So now this is the first quadrant, second Quadrant, 3rd Quadrant, 4th Quadrant. Right. So it goes in the anticlockwise direction. Now when we start, we may be at a point. So now this is how we start from zero. So now this.
This is your zero right now. If you have a point here. So now imagine a robot is turning like this from the home position. Now this may be.
The home position, right? So the arm might be facing in this direction. So now Theta is positive in this direction.
Now, if you know this particular point, then you know what is X.
You know what is y right? So Theta is equal to tan inverse y by X, right? So when you are in the first quadrant, there is no problem, right? When you are in the X axis, y is equal to zero, so tan inverse.
Of zero right when when you are aligned with the X axis, y is equal to zero, so tan inverse zero. Your calculator will give you the correct answer. Now imagine you are at a point.
Where X is equal to y right now, for example, X is equal to 300, y is equal to 300, right? So 300, 300. Now if you find out tan inverse 300 by 300 right this is tan inverse one.
Right, so nowhere Theta is equal to 45. Very easy. It just gives you that value now.
OK, here at this point.
Now X is negative.
-300 y is +300.
Right now in this case.
I would expect the calculator to give me this angle.
Right. So in this case this angle is 135, right?
Yeah, 90 degrees +45135, right? So I would expect the calculator to give me 135, but what will it give tan inverse y is 300, X is -300.
So tan inverse -1. Yeah, you can check your calculator and tell me what this is tan inverse of -1.
OK, good. So your calculators are working faster. Good. So tan inverse of -1, it is saying -45, it is equal to minus C Now so far we never thought about this because it was just mathematics. We were writing an answer on paper and we just walked out of the.
Exam hall now. Now if it is -45, where will this go?
Plus -45 it is this angle.
This is -45 for me, this is -45.
Now we are not not even close, right. We are 180 degrees apart.
Right, so this is not what we want. So even though this is what the calculator will give you, right, so that's that's how intelligent calculators are or you know for that matter MATLAB or any other software, right. Because for the calculator, it is only the -1 that matters. The calculator did not see.
This 300 and -300 right. So if you had given 300, -300 or -300, 300 then it would have found out in which quadrant this particular point is located and accordingly it would have given.
The answer, but what we fed is just this -1 and for that -1 the only thing it can do is give -45, right? So this is a problem. This itself is a problem the moment you cross first quadrant. It is a problem now coming to 3rd Quadrant.
So 3rd Quadrant is even worse, right? Because it is -300, -300 whatever it is now, whatever point that is -300 the both are minus right? So in this case tan inverse -300 / -300.
So what is it?
Tan inverse of one.
Tan inverse of one. Now if you ask it will give you Theta is equal to 45.
So actually theatre should have been what?
Later, should have been this.
So if you go all the way here, 180 plus another 45, it will give you 225, right?
Yeah, Theta should have been 225, but instead we got 45.
So again, a problem now coming to the 4th Quadrant.
OK, coming to 4th quadrant.
Now the point will be X is positive, so 300 come on -300 y is negative right? So in this case what is it tan inverse of again?
-300 / 300 or tan inverse of -1, so again Theta is equal to -45. To some extent this is OK, right? But again this will not be consistent. Now lets try a point which is closer to the X axis.
Right, So what is it now? Let me take it as.
X is equal to 300.
Y is equal to now let us say 50 y is -50, right? OK, now 300, -50. What answer will it give me?
Tan Inverse -50 / 300. Now let us see if it is giving me a consistent value. What is it?
Yeah.
Tell me the answer.
It is giving you.
Mine is 80.5. Wow, it is giving you this angle. This answer. Now if you go with mine is 18 or where will where will you go? You will actually go here right.
It is giving you something again wrong, right? So now -80, you will go close to the Y axis, but here we are closer to the X axis.
Again, you can't take this as correct value right? So overall whatever you get, everything is sorry is it is it right or sorry.
Anyway, I should not have anyway. Let me check is it is it 9.46?
Sorry, let me check.
50 divided by 300.
Minus OK inverse tan tan inverse.
OK, it is giving only 9.46, so at least it is consistent.
OK, so this is not right. So this is Theta is.
-9.46. So now if I let me try just just to verify. So this is at least consistent in the 4th Quadrant. So let me try something like Y is equal to 300.
X is equal to 50 OK, 50, -300. Now let's check this 300 -, 300 / 50.
Uh.
What is it giving me?
OK, so Y is -300.
Inverse tan. OK, so now it is giving me 80.53.
And again, -80. So now this is consistent. So now if you see it is consistent in the first quadrant and the 4th Quadrant, but 4th Quadrant again the direction is not correct, right. The only thing is here it is giving me.
A number which is OK which is giving me the correct position, but again what it should give me is this.
It what it should give me is this angle. I should go like this and I should come to this particular position but instead it is giving me the angle in the opposite direction but that value is OK right? So at least position is correct right?
So these are the problems that we are dealing with. So nowhere what we should do is now we should write conditional statements whenever you are dealing with a point like this. What you should do is.
First of all, you find out in which quadrant the point is located. Now that is very easy, right? Positive positive first quadrant. Now back in 10th standard.
Our maths teachers probably taught us right, so if it is positive, positive.
It is positive and positive.
First quadrant now if it is negative positive X is negative, Y is positive.
It is second quadrant, negative negative is 3rd quadrant. Now X is positive, Y is negative right its 4th quadrant.
Right, so this is something we know now what we should do is we should write the algorithm accordingly. Now if you get a point, first you find out where it belongs, whether it is on the first quadrant, second, 3rd or 4th.
If it is first quadrant use you can use the angle as it is Theta is equal to just Theta whatever angle you get. That is your final angle right or Theta final is equal to just Theta whatever you get.
Second Quadrant Theta final is equal to what now it is giving you.
180 sorry, whatever Theta you get.
What do you have to do? 180 plus Theta, right?
Now we have to check.
Once more.
OK.
Now for the second quadrant, if X is.
-15 and y is 300.
Right. What is the angle if the angle is small then yes you can go by that. Yes, the angle is going to be small, right or no.
See nowhere. What is it doing?
Yeah, lets find out. Sometimes it will not be consistent. So tan inverse y by X 300 by -50. OK, it is going to give me a bigger number so this is not correct right?
So 300 / 50 minus inverse tan it is giving me minus C nowhere it is giving me for the second quadrant. It is giving me. For this it is giving me.
-80.53 OK.
So the angle is smaller, so 180 plus Theta is correct.
So this angle is going to be smaller, so the angle is anyway negative. So if I add 180 to this.
I am getting 99.46 so in this case I am getting 99.46.
So this is correct. 180 Plus Theta is correct for the second quadrant, right?
Any comments, any questions so far?
No, no, it is not 90 plus Theta because the angle is negative. You are getting negative right? So that means 180 plus Theta and you are getting only a smaller angle.
OK, now let's look at the 3rd quadrant.
OK, for the 3rd quadrant, you take a point which is closer to this.
Anyway, if you already know how to calculate it, it is probably a repetition. But anyway, I just want to clarify this part. So 3rd Quadrant Theta final is equal to. What is it? Nowhere in this case this point.
Uh.
X will be.
-300 Y can be my Y can be 50. Sorry. OK, Y can be -50. Both are negative. Y can be -50. So in this case, what will you do? The angle is going to be positive. So in this case again it is going to be 180 plus.
Theta. So what is it y by X? So 50 / 300. So again it is going to be that 9.46 so it is 180 plus Theta.
So this is going to be the angle for the 3rd quadrant. It's going to go all the way and come like this and it's 180 plus Theta for the 4th quadrant.
For the 4th Quadrant, we already saw this right? It was.
Closer to.
So in terms of negative, it was giving the correct answer. So all you have to do is 360 plus Theta.
360 plus Theta will actually give you the correct value for the 4th Quadrant, so lets check that so 300, -50 it gave us.
-9.46. So if you add it with 360.
360 + 9.46 minus, right? So that is 360 minus. So this is going to be.
350.
0.54. So if you add it with 360, this is what you are going to get. So 350.54 is the correct angle in the anticlockwise direction.
So if you have to interpret the angles in the correct way, this is what you have to do to the existing.
Values that you get from your calculators and from your software, right? So this is fairly straight forward. Now again you have the axis.
Now, apart from this you have the access.
OK, so now if the robot is aligned with the X axis, this is easy because y is equal to zero.
And Theta will obviously be zero. So for the first quadrant or for the X axis it is not a problem. Now for y axis what will it give?
Right. If your arm is aligned with the Y axis, then what will happen? X is equal to zero, so now Theta is equal to tan inverse y by X. Now if X is equal to zero.
Tan inverse Infinity.
Now will your calculator give you an answer? Tan Inverse Infinity is again 90. We know that from our trigonometry, but will the calculator give you anything like that? If I do 1 / 0 in my calculator it is giving me error. Moment I do this it is showing error then I cannot do tan inverse of error.
Right. I cannot even calculate this using my calculator, right? So even though I know tan inverse Infinity is is 90, tan 90 is Infinity right? So now if I use MATLAB, will I get anything here?
So these are some simple things which can actually confuse your software. If you are going to write a software. Finally whatever we are learning here in this particular course will end up in your software. Now if I do 1 / 0 in MATLAB, what is it giving me? See it is giving me Infinity.
OK, now I I am doing a ton D.
Of the answer. Lets see if it is intelligent enough. OK, Matlab is giving me an answer. Not bad, but my calculator will not give me right? So MATLAB is actually giving me 90. So yeah, if your software is this intelligent, then at least you know.
You will not end up with some.
A robot crash or something. OK now.
If it is in this position.
If it is in this position again, Y is equal to 0X is equal to negative right? So again tan inverse.
Uh.
Tan inverse of.
Zero, right? It is just going to be tan inverse zero, so it is going to give you zero so that 180 plus this angle it is fine now whatever correction we made earlier that will work here also. Now again in this position.
If it is in this position, your X is equal to zero and so it is going to give me.
For this condition X is equal to zero.
Y is equal to some positive number, so tan inverse again Infinity. If I can get Theta is equal to 90 then I can add it with this 180. So I have to take it to this quadrant.
So I have to consider that it is belonging to the end of the 3rd quadrant, then after add it with 180. If I if I get 90 then I add it with that 180 then I get 270 right? So again these things you have to check right for all the zero points you have to check.
And then write an algorithm. But anyway I am not going to do all that again. You don't have to do all that for your exam. Whether it is your lab exam or your theory exam, you don't have to write all this logic because we won't have time for that right? So I am just telling you all this because if you are going to build a robot.
If you are going to write the software for a robot, or if you are going to write any control algorithm, you have to be careful with all these issues. Your calculator and your software will totally get confused, right? So any questions on this?
There any questions so far?
Everything is fine. Shall we proceed to the next problem?
Tan 180 plus Theta you tried with your calculator.
Done.
See now what is that you want? You want an angle or you want the tonne of an angle?
Second quadrant, we want negative value of tan.
I I don't understand your question what what are you trying to achieve here?
Tan, 90, itself is becoming Infinity, then tan 180. See tan other values. Yes, it will give you. Now that number is something related to against, you know, whatever I showed you know all your.
Tan values sine values are going to give you some numbers. Then you have to interpret them in the correct way, right? So as of now, I am not going to do any research on trigonometry. The research done already is enough, right? So now this is it. We will stop here.
So here what we have done is we have found out a way to correct what the the computer is going to give us, right. So if you include this algorithm in your inverse kinematics calculation, then again for these.
Positions. If you write an algorithm, you should be fine, right? So we have that option. Now we can write conditional statements. You can put if then conditions you find out in which quadrant this particular arm is in at any particular time, then accordingly you.
You make a correction in the angle. That itself is sufficient for our case, right? OK. So now for the polar arm, lets write the forward kinematics equation and we will also solve the inverse kinematics problem, right.
So I am just taking this polar arm. I am just putting that in the home position right? So I am putting this in the home position.
So this is my home position.
OK, so the first axis is here. The second axis is here.
3rd axis.
So that is the direction of extension. So that is here.
Now with this in mind, first I want to place my Z axis aligned with the axis of the first joint then.
I have to align that with the access of.
The second joint then finally here.
ZZZ and finally here Z has to point in this direction. So these are the positions of the Z axis. Again, once I place the arm in the home position it is easy.
To visualise this right, OK, now first.
I'm placing the coordinate system like this X I'm placing in this direction X naughty Y naughty Z naughty right? So the origin is here.
Then.
After I go here once I align my Z axis with the axis of the joint X axis has to be aligned.
With the link like this I am going to align my X axis if X axis is like this, Y will point towards the top right.
Now if my Z axis is pointing like this, I can have my X axis pointing upwards like this. This is possible and I can have my y pointing like this right?
So we can place the coordinate systems well in advance if you want, if you if you know how to position it. If you can visualise the right handed coordinate system at any particular point, then you can position the coordinate systems at the correct location.
And then you can write the forward Chromatics equation. Now all your textbooks will actually do that, right? So that means you should have already taken a course on robotics. Understood. You should have understood all the concepts well in advance. And then if you take this course, then you will be able to do that.
Right from the beginning, but you know we do not have that option, right. So first you try to understand how to rotate the coordinate systems. Then you know at 1:00 point once you get comfortable, you will be able to place the coordinate systems in those locations and then write a forward kinematics equation, whichever is convenient for you.
Right. You can follow any method.
OK, so I can do this.
In this case, if you can position the coordinate systems well in advance, then you can modify the equation to fit this position. Now somebody will always ask right? Why can't I put the Y axis like this and the X axis pointing the other way.
Or why can't I put the X axis pointing out to outside and y axis downwards? Everything is possible right? So I cannot change all that. The only thing we have to follow is aligning the Z axis.
Which we have done right in the beginning, right? Once you do that remaining.
Things right, when X axis see when Z axis is perpendicular to the link, X axis has to be aligned with the link.
If it is a prismatic joint, Z axis always should be aligned with the direction of extension. That is easy tool point Z axis has to be aligned with the direction of the tool. Now any other condition?
When Z axis is aligned like this, which way should X axis be? Does not matter, X axis can be like this. It can be like this. It can be like this whichever way you want you position it. But XY relationship has to be correctly represented.
Right. So then now if I say X axis is like this, y axis should be like this only then it will it will be a right handed coordinate system right?
Any tricks? No tricks. You follow the rules. First you position the Xerox is first.
And X axis.
Along with the link if that is the condition, see now there are easy way to see is I have I have actually done this before right? If your Z axis is perpendicular to the link see now link is this way Z axis is perpendicular to the link or the joint axis is perpendicular to the link. X axis has to be aligned.
With the link.
Now, if the joint itself joint axis. Now for example if you have a twisting joint Z axis, a joint axis is aligned with the link, then Z axis has to be aligned with the link.
In that case, Xeroxes has to be aligned with the link. X can be anywhere, X can be perpendicular here here, here anywhere.
That is fine, right? So these are the only things you can do now. These are the only combinations you have right in the case of a prismatic joint.
Right, anyways, that axis has to be aligned this way, so X axis can be like this. X axis can be like this. Anything is OK, right?
OK, now if you try this with different problems you will you will understand it better. So the only way to do this is you know solve more and more problems of different types and then at 1:00 point it will just become very easy. OK.
So now I can try to rotate Z, Theta one so here.
Then prance.
ZL one. So this is L1.
Then rotate X 90 so the moment I reach here my Z axis will be like this. My X axis will be like this and y axis will be like this right?
Now if I do 1X rotation by 90 this then everything will fall into place.
Right, everything will come to that correct location. Now this can be.
X1, Y1, Z1 right? So this is T0 to one.
OK, now from here I can try to rotate Z, Theta 2.
Right. So.
Now that is a variable, so rotate Z Theta 2 you have to include in the equation.
Trans X.
L2 so this is L2.
Now when I do this.
The coordinate system will come to this location and X will be pointing in this direction. Z will be pointing in this direction and Y will be pointing upwards. This is how it will be after this.
After these 2 commands then I have to write rotate Z 90. When I do this.
X will go to the top, X will go to the top and Y will point backwards. So when I do that, X will go to the top like this and Y will point backwards.
And Z will remain in the same position. Then after this I do rotate X.
Rotate X 90 when I do rotate X 90, I have to see from the top.
Then that means it is this rotation. When I do that automatically, Z is going to come to place and Y will come here.
Right, so this is.
Rotate X 90 so now this is P1 to 2.
Then finally, all you have to do is just move Z in the Z direction and reach the tool point. For that you have to do trans ZD 3 right? So this part is D3. Then you multiply it.
Multiply that with the origin so trans that D3 will be your T.
2 to 3.
Right so that's it. Forward kinematics equation is done. Now we actually tried this in our lab class today, right? We were able to write this equation. We were able to.
Check this using MATLAB right.
Can we write rotate X -90? No. If you do rotate X -90, Z axis will point in the opposite direction.
We want Z axis to be pointing in the direction of extension. Finally we want Z axis to be pointing in the direction of the tool right? So in in both of these cases that is not OK. The only thing where you can have some.
A choices you know here if you have.
A joint axis like this right xeroxes can point inward. It can point outward like this, or it can point inward. It can point inward like this. So some textbooks have actually followed this method, pointing the xeroxes inward.
Right. So that is for a joint like this. Now that is for this condition. So if you want you can put the Xerox is pointing inward, but you should be very careful with how you rotate right. So now if you if your Xerox is pointing inward.
Now you have to go and see from this side.
Right, you have to go and see from this side, then your rotations will be clockwise.
Again, if you can visualise, you can do it, but I I find it very comfortable visualising from this side so that my rotations are anticlockwise and they are positive right. So that is why I am writing it like this. Some most of I would say some authors.
Have chosen this, for example Syed Niku book. If you see that you know the xeroxes will be pointing outwards. If you check Robert Shilling textbook, then Xeroxes will be pointing in the other direction, right? So you can choose any way you want, but you should write it correctly.
So if you put this xeroxes in the other direction, instead of rotating in the anticlockwise direction, if you rotate it clockwise then you will get the entire thing in the opposite side, right? So everything will be wrong.
OK, so this is it equation.
Now you have to expand each and every matrix and you have to do matrix multiplication. If you do matrix multiplication, you will end up with a set of equations, right? So now for this the DH table can be drawn, so we did not draw the DH table for this particular arm.
We can do that.
Right.
Data.
The.
Alpha and a right.
And 123, right?
Are the moving links 123? So the first one this is going to be Theta, one trans ZL one, so Z axis movements X rotation 90, right? So these 2 are for Z axis.
These 2 are for X axis.
OK, rotate Z Theta 2, trans ** translation L2 rotate Z 90.
X 90 OK.
Then trans. ZD 3, ZD 3. So D3 has to come here. OK, this is it. So this is the DH table. So all you have to do is just take these values and put it for X or Z axis, right? So X translation.
Z translation X rotation, Z rotation. So accordingly you put that in the correct column and your DH table is done.
So now I can take this and put it here.
Now, if you read Robert the book written by Robert Shilling, so they just take this additional angles and put it on a separate column, but that's not needed if you want, you can just put it straight away here.
OK, now after you do all the matrix multiplication, you end up with.
These equations.
You will end up with these equations PXPYPZ.
Right. So if you know how to solve these equations or if you can find a solution right, the equations are solvable, and if you can find solutions then you can simply solve for PXPY&PZ and.
You will be able to get the answer now. Let us try that.
So this is PX. So this is equation 12 and 3, right? So we have 3 equations.
OK, so now I can try to solve the problem. I'll take equation 2 divided by equation one. Then what do I get?
L1 sorry L2 + D3.
Sine Theta Cos Theta divided by.
L2 + D3.
Cos Theta one right.
Cos Theta one.
Cos Theta 2 OK looks fairly simple, so these 2 will get cancelled Cos Theta 2 Cos Theta 2 will get cancelled. So we get tan Theta one is equal to PY by PX.
Right, so this is easy. So Theta one is equal to tan inverse PY by PX. So we got the first variable easily.
So all we did was just divide equation 2 by equation one and we got Theta one. So now can we take Theta one and put it in one of these equations? We can try or we will take 2 other equations see nowhere we can take Theta one once you know Theta one Theta one.
One is a number. You put that in any of these equations it it will actually give you. It can actually help you simplify the equation. Now I will just take equation one.
Now L2 + D3.
Cos Theta 2 is equal to PX by Cos Theta one right? So I am just putting Theta one in equation one and this.
Right hand side will just be a number because Theta one is already available with us. We can put that and the right hand side will just become a number, right?
OK, then what shall we take? We will take equation number 3.
Here we have a sine Theta 2. We will just take that. So L2 + D3.
Sine Theta 2 is equal to PZ.
Minus L1, so I am just taking all these things to one side so that I have only L2 + D3, sin Theta 2 here. Now I can do equation 3 divided by equation one.
Right. So when I do that, what will I get?
Tan Theta 2 is equal to PZ minus L1.
Divided by.
PX by Cos Theta one right so I can write this as Theta 2 is equal to tan inverse.
PZ minus L1 * Cos Theta one right divided by PX.
Right.
So with this I am able to get Theta 2 also.
Right. So I can get Theta one and Theta 2 by just solving these equations.
Now if I put Theta one and Theta 2 in any one of these equations right in equation one, you can take right? So then I get L2.
Plus D3.
Is equal to.
PX / Cos Theta one Cos Theta 2 right? So now if you know the values of Theta then this is right hand side is just a number. So you can find out from this.
T3 is equal to PX / Cos Theta one Cos Theta 2.
Minus L2, right. So that's it.
I can, you know, we can simply find out the solution just by solving the equations. Right? So now you can actually do that.
Uh.
Now you can check whether we have done this correctly.
Right. So yeah.
Cost it a one cost it a 2 can be taken to the other side.
Yeah, so D3.
OK. So that's it. So in this case, we were simply able to solve these equations and we were able to get the answers directly, right? So if the equations are solvable.
Then it is easy for us, but the only challenge is only challenge is that in some cases you know in some problems equations may not be solvable, so we will come across that condition also. But as of now if it is solvable, you simply go ahead and solve it and get the solution.
Right.
Any questions so far?
Yeah, you can put these numbers now. This is given as a problem, so here a point is given 65150350L1L2 is given. So yeah, you can go ahead and put these values so 6515350. OK lets put this.
Now can anybody tell me the answers now we will verify 650 PX is equal to 650 PY is equal to 150 PZ is equal to 350. Yeah. Please tell me the answers.
Tell me Theta one, Theta 2 and D3.
Yeah.
So somebody is telling 2A 12.99. Is it Theta one?
Theta one? Yeah, do it fast. Now tell me the answer.
We'll check. OK Theta one is 12.99 good.
12.99 degrees. OK, what is Theta 2 now?
OK, now L1.
L1, L2, L3 or sorry, L1 and L2 L1 is 250, L2 is 500.
215 L2 is 500 and there's no L3 OK.
2 5500 MM, 500 MM yeah heated 2.
250.
Later to Anybodies make it fast.
350 minus.
250 OK, that's 100 * Cos Theta one OK.
Divided by.
Accepted.
OK, now tan inverse.
OK, so I'm getting something like 8.52.
Yes. Are you getting that?
Yeah. Good. 8.5 2.149 degrees. So you forgot the inverse.
That is too small.
OK. Next one find out D3 also.
Find out D3 so Cos Theta one Cos Theta 2 OK.
Minus L2 500.
So I'm getting 174.
0.52 MM.
OK, good. 174.52524.
Yeah.
Yeah, 523 or 52 is OK because, you know, I took only 2I took only 2 decimals for the angle, so that will actually cause a variation. So that's what the answer is here. 12.99 8.5 to 174.53 MM.
Right, so here we are able to get the answers again. This is in the first quadrant, right? So we don't have to do any other manipulation.
OK, now we will. We will take a break for 5 minutes when we come back, we will do the same thing for a cylindrical arm and an articulated arm, right?
5 minutes break. So if you have any questions you can type your questions on the chat window.
Welcome back.
Now many questions are.
OK. Should we do matrix multiplication? Yes. Yes, you have to do matrix multiplication if you are asked to solve a forward kinematics problem. If it is a smaller problem, right? If it is a 6 degree of freedom problem, then it will be mentioned in the problem that you don't have to do matrix multiplication.
So if it is required, it will be mentioned if you are asked to find the equations. If it is a fairly small problem like a 3 degree of freedom problem or a 4 degree of freedom problem. In such cases you have to do matrix multiplication.
In some cases, you may be given an inverse kinematics problem, right? May be a 3 degree of freedom, or a 4 degree of freedom problem. So in that case you can do the inverse kinematics only after solving the forward kinematics problem, right? So that means here.
We have these equations by solving the equations. We are solving the inverse kinematics problem. So now how will you get the equations? For that you have to solve the forward kinematics problem, get the equations. Then you go in the reverse direction you get.
The solution for the inverse kinematics problem, so now that means you have to get these equations to solve the problem. So this will not be given to you right? So if I give a problem like this for example now you have to write the forward kinematics equation, do the matrix multiplication get the equations and then solve the equations and get the solution.
Right. So that is how it is done. So that is how you have to do in the exam also. OK, so now this is inverse kinematics of a 3 degree of freedom cylinder alarm, right. So now in the previous case.
We discussed, you know, how to write the equation for a cylindrical arm, so now that cylindrical arm was slightly different from this.
On the previous case, what we saw was.
Like this?
Something like this?
Right.
Nowhere. What we have is slightly different.
Now you have a base.
Do you have?
A joint like this?
Then you have something like this.
So both are cylindrical. The only thing is if you see the joints are interchanged, the joint orientations are actually interchanged. So one should be horizontal, one should be vertical, right? Only then you will be able to cover a larger space.
So in this case, the second joint is vertical, 3rd is horizontal. In this case second is horizontal, 3rd is vertical, so that's the only difference. But both are cylindrical, right?
OK, so now this is the arm. Now we can put the arm in the home position, right? So I can.
Draw Something like this to put this in the home position.
OK, so now the arm is in the home position. Now we can place the coordinate systems, the first joint axis is like this.
Right. Second joint axis is like this.
3rd joint axis is like this right? So we know the joint axis.
And the tool point is here, right? So in the first joint I can place the xeroxes like this.
In the second joint, I can place the Z axis like this.
3rd case I can place the xeroxes like this right?
The last 1Z axis can be like this, right? So.
Based on this, I can place the coordinate systems.
Right.
So I can write the equation and then you know I will see how I how I can go forward. So I will do rotate Z Theta one so the first one is rotation here.
Then trans ZL one. Right. So the moment I do this, my coordinate system is going to go here?
And it's gonna sit like this.
X.
Y right now, at this point, what I can do is I can turn the coordinate system in order to turn this I can write rotate Z 90.
Right. So when I do rotate Z 90 I will.
Rotate like this. I can see from here.
Now this will rotate in the anticlockwise direction. So my X will go here and Y will go here right? Then I am going to do rotate.
X 19 for that I have to see from this side.
So when I see from here, this will also be anticlockwise right? So when that happens Z axis will automatically come to this particular position, X will remain.
In the same position.
And what will happen to why? Why will come to the top here?
Right.
Then I have to move this distance.
So now the frame is correctly oriented the frame the frame is in the correct orientation. After we have done.
Rotate Z 90 and rotate X 90 Z axis is pointing in this direction. Now we have to do trans.
Z.
L2.
Right. So after we do this we get.
P0 to one right, so the coordinate system will come here.
Z.
X.
And why?
Right.
OK.
Why?
For which problem are you talking about? The previous one? The inverse cinematics problem?
Why have? Why we have not put any conditional statements. See now. Yeah, we can do all we want, but we we don't have the time. Right. So now we are just going through methods to solve these equations. Now when you are actually implementing it, you have to put conditional statements.
Right. So as far as the academic requirement is concerned, you know for the exams, if you write these equations, if you solve these equations, you know if you if you write these solutions, marks will be given.
But those conditional statements will be needed if you are going to actually implement it. So that means if you are going to implement it, you be aware that your calculators and your software will deceive you, right? So that is why I have mentioned all that. But for as far as the exams are concerned, if you can derive these equations you will be you will be.
Given marks beyond that, textbooks and exams will not interfere, right? So when you are actually implementing, you be very careful. OK, so this is T0 to one. Now I have to come.
To the next joint, right. So the next one is trans Z.
D2 sorry, nowhere. Yeah, we will call it D2 and we will call this DD 3 right to be consistent.
OK, trans that D2.
The moment I do translate D2, my coordinate system will come to this location.
Z.
X&Y.
Right now I have to see from here.
Have to see from here.
And do this rotation. Now what rotation is it? Is it clockwise or anti clockwise?
Yes.
OK.
Good.
Yeah. What about the others? Only 2 people are answering.
Now when I look at it from my position, I see clockwise rotation, but the I is in the opposite direction. So then obviously it is going to be anticlockwise rotation. So rotate X 90. When I do this rotation.
The Z axis will point downwards, so Z will point downwards like this. X is going to remain in the same position. Y is going to come here.
Right.
So I can call this as.
X2, Y2, Z2 right here this one.
I can call as X1, Y1, Z1, right.
Now up to this.
It is T1 to 2.
Then from here.
I just have to go straight down, right?
Why?
And.
X right, I just have to go straight down. No challenge here. So then the last one is.
Trans ZD 3.
Right. This multiplied by the origin.
So now this is T2 to 3 right? So that's it. First one seems to be a bit complicated but nothing so complex about it. The only thing is we.
Kept X in this orientation right? So that way it took us 2 rotations here to get our frame oriented. So that is the only thing. Otherwise it is fine, right?
Now the solution is here rotate Z, Theta, one transit L1. Nowhere in this case. See now you can do this Trans XL 2 and then finally you can do this rotation. See nowhere. The way I have written it is.
Trans ZL one. Then I can do this rotation here. Rotate Z 90, rotate X 90. Now this is consistent with this, but if I.
Make the movement after this, then this. This trans will be Z.
L2, right, so that is the difference between what is written here and what is written here, but both are just the same, right? So here I have done the last rotation here. So that means this will be Trans XL 2.
Now I have done the rotation here so this last movement will be trans Z DEL 2. So yeah, this is another way of writing the solution. So both are correct.
You can you can follow any method, then trans ZD one.
Rotate.
A tanser D2. Now let's put it this way.
OK rotate X 90 transit D3 so this is just as it is.
ZD 2.
Rotate X 90 so this is T1 to 2. Then trans ZD 3. This is T2 to 3, right? So this is your solution. Now any equation is fine.
You can expand this and.
You you can expand this and you can get the equations finally. Yeah. D3. Yeah. What's?
What's your question?
Trans ZL 3? Where is it?
L3, C Now if a dimension is given like this for example, if this is given as a dimension and this is given as a separate dimension, yes, you can include all of those. If nothing is given, then whatever you have here.
Is D3 right? So if those dimensions are given in detail, yes, you have to take them.
If a particular dimension is given right for the dead portion right nowhere, they have given L2. Similarly, if there is something here you can take that. Otherwise you can straight away take D3 and solve the problem, right?
OK, so any solution is OK.
Now the DH.
Table is given here. Now instead of 90 home I'll just put this 90 here.
So that's fine.
OK, now this is the set of equations that we will get after we do the matrix multiplication.
After we do the matrix multiplication.
This is what we will get PXPYPZ.
OK, now can we solve it? We can try now. Looks like we can solve it.
So again, I can write this equation like this D1 + L2.
Cos Theta one is equal to PX.
P1 + L2 divided by. Sorry. Multiplied by sine.
Theta one this is PY.
L1 -, D2 is equal to PZ, right?
OK, so now I can solve these equations so equation 2 divided by equation one will give us.
Tan Theta one is equal to PY by PX. So now these 2 terms will get cancelled.
Theta one is equal to tan inverse.
PY by PX, right? So this will be more or less consistent for all the arms with a vertical first joint, right?
So this is for Theta one. Now once we know Theta one we can put it in equation one or 2. So D1 + L2 is equal to PX / Cos Theta one. So that means this part right hand side will become a number.
So D1 is equal to PX / Cos Theta 1 -, L2.
Then once you know, yeah. Now D2 can be directly found out you need not worry about it. So I just take this to this side. I just take this to this side so D2.
OK.
Yeah, we will. We will change it, OK.
Yeah, I changed it. So this will be D2, D2, D3, right? Yeah. Did I miss anything?
OK. Yeah. So now I have changed it. The last one will be D3 is equal to L1 minus PZ.
Right, so D3 is L1 minus PZ.
OK, so this is it.
Uh.
We can easily solve this now. This is a fairly straight forward arm because you have 2 linear joints, you have 2 prismatic joints, right? So this is a prismatic joint and this is a prismatic joint. In that case your solution will always be very easy to get.
Right. So if you are designing a cylindric alarm, it is always easy to get the solution. It is easy to control and so on. But you need to have a problem mechanism at every joint. Then it will be easy to control, right?
So here we can solve this. There is nothing challenging in this case.
Now you have to write the forward kinematic equation correctly and you have to do the matrix multiplication without any mistake. Then you are you are fine right? So again with the numbers given you can check the solution right? So some numbers are given. Tool Point is given. You can check the solution.
Here. Right. Let's see that later. Now we will go to the next problem.
OK.
Now we actually solved the forward kinematics problem we also.
Plotted this in MATLAB in our lab class, right?
So this is a very familiar case.
And we also made one clear observation in the last class, right? So if you have one axis like this.
One axis.
The other axis these 2 axis are always parallel no matter what position the robot moves to. Then we made an observation, right? So lets see what's gonna happen here.
So anyway, this colour is not clearly visible. OK.
So I am placing my first coordinate system Z aligned with the axis of this joint then.
Second Axis is pointing this way. 3rd Axis is pointing this way and the tool is pointing this way.
Right.
Now these are the positions that we have to get the first coordinate system I'm placing like this X.
Y&ZX naughty y naughty Z naughty.
So I can write the equation.
Rotate Z, Theta one.
Trans ZL one.
Rotate X 90 right so when I do this I will get this position. Now it is always better to place the arm in the home position.
So I'm placing the arm in the home position like this.
That way we don't. We won't have any confusion right?
OK so.
Instead of this.
Let me put.
The axis like this.
OK, Z&Z, so now the arm is in the home position.
P0 to 1X will point in this direction.
If X is pointing in this direction, Y will be like this right X1, Y1, Z1.
From here, going to the next position is easy.
X.
YZX 2, Y2 Z2, right. So that means you have to include the second variable rotate Z Theta 2.
Trans XL 2 so this will be T 122 then finally.
You can write rotate Z Theta 3.
Trans XL 3.
The moment you do that, you will be here.
You will come to this point with X.
Z&Y like this when you do rotate Z 90, your X will go up.
X will go up.
Right. Why will go back?
Why will point in the backward direction then? After this you do rotate.
X 90 so.
You will do this. You will see from the top right. So when you do that, Z will come to this position.
Z will come to the final position, Y will come to this position, so this will be X3.
Y3 Z3. Right. OK, so we we have seen this multiple Times Now.
The theory class the lab class.
Right. So some of you might have memorized this equation already.
Right. So it it becomes that way at 1:00 point. You know you will be able to easily predict these equations. They are fairly predictable, right? So you when you are changing from a perpendicular direction, see now when you are changing from.
This direction to this direction automatically. This rotates Z 90 rotate X 90 will come right, so that is something.
That's clearly visible from all these problems. OK, now yes, coming to this problem after you do all the matrix multiplication right, I can do all this. We have seen all these already. Now we get.
This particular equation, now this is where the problem starts.
PXPYPZ now the equation is like this.
L2, C.
L2 Cos Theta 2.
Plus L3 Cos Theta 2 plus Theta 3 * Cos Theta one is equal to PX right similarly.
L2 Cos Theta 2 + L3 Cos Theta 2 plus Theta 3.
Sine Theta one is equal to PY right 3rd one L3 sine Theta 2 plus Theta 3 + L2 sine Theta 2 + L1.
Is equal to PZ, right? So did I miss any terms? OK looks like everything is here. OK so one.
2 and 3 right? As I mentioned earlier, when you have 2 parallel axes.
On the robot, you know this axis and this axis will always be parallel, no matter what position the robot is at, right?
So in such cases you will end up with a coupled set of angles, right Theta 2 plus Theta 3. So those 2 angles can be coupled. So in that case it is not that easy to solve this problem, so it is not possible to solve it like all the previous problems that we did.
In the previous problems, we did not come across this case, so we were able to simply solve the trigonometric equations and we got the answers nowhere. That's not possible. OK, now what can we do here? So we can still.
Solve for Theta one so equation 2 divided by equation one will give us.
Tan Theta one right. So now if you see this term is the same as this term, so they will get cancelled.
Tan Theta one is equal to PY by PX or Theta one is equal to tan inverse.
PY by PX, right? So this is easy to solve.
First now after we solve this, if you put it here or here, no matter where, we are not able to get anywhere, right?
So after this we are not able to solve it now probably.
You know.
We have to find a mathematical solution. Now I am sure solutions will exist right? But as of now directly we are not able to solve it. So what should we do right now? One option is to decouple these angles.
You can separate these angles into Theta 2 separately and Theta 3. Now there is a possible there is a. There is a way to decouple these angles. We will discuss that in the next class. Now that is called the matrix inverse method. So now we will use what is called the matrix inverse method.
So that's how this name inverse kinematics comes right? So that's where it comes from. So we will use a matrix inverse method. So in forward kinematics we did matrix multiplication. So in inverse kinematics we will find a way to split those matrices. You know at the right point where this angles.
Will be decoupled and you will get data 2 on one side of the equation, 3 to 3 on the other side of the equation. That is something we will do in the next class, but now what else can we do in this particular case, we can try to solve.
This problem by using the geometry of the arm.
Right. So let us see how to do that. Now this is called the geometric method or the geometric approach. Now if you see the textbooks, they have actually used this method, but they won't.
Give you this as a standard solution. Now when there is when a particular arm is difficult to solve and a particular arm problem is difficult to solve, textbooks will actually use this method also. Now the authors have actually shown how to use this.
But it is not a standard method. Standard method is probably the matrix inverse method because you have a proper structure, you have a proper formula here, but let us look at this method as to how to solve using this method also, right? So it is just learning anyway.
Right. So now if you look at this arm, you see?
A triangle.
Sure. So this part is a triangle.
Right. So using this triangle, can we solve this problem?
OK, now the first angle is anyway available. So Theta one is already available with us. Now we just have to solve for Theta 2. So in this case this is Theta 2.
This is Theta 2 and this part.
Is Theta 3 right now this length is L2. This length is L3.
Right, OK, Now lets try to understand the geometry of this arm. So now if I place the arm in this position now from the top view, so now everybody has studied engineering graphics. So now you can actually visualise.
The top view of an object when it is given to you.
So now I am just keeping Theta one at zero. If Theta one is equal to zero, the arm is going to be in this position at that particular point you will be able to see the proper lengths of all these links, right? So you will be. You will be able to see the actual.
Lengths of these different links, right? So this is L2. This is L3. Now what else do we know? OK, now from here, zero, zero, zero is the origin right now this height is L1.
Now can anybody tell me the coordinates of this point?
Yes.
What are the coordinates of this point?
OK, no.
No.
Okay talluri is correct.
See now how did you place the coordinate system?
You place the coordinate system like this, right? It was X.
Why?
And Z right?
It is always like this base coordinate system X naughty Y naughty Z naughty. It was always like this, right? So in that case this point is going to be zero, zero, L1.
Right.
So now so far we have been talking about placing the coordinate system. See now everywhere it is like this, right? So you should not forget this, right? So.
OK, so zero, zero, L1. This point we know now from the geometry of the arm we can actually find out this point now this point.
SPX, PY, PZ right. This is something we know. OK now I am going to ask you a question. What is this height? What is this height?
Yes.
OK.
OK. Anybody else?
OK.
So you are right now, as I mentioned, we are placing the arm like this. So that means when we look in this direction, when we look in this direction or when we look in this direction, we will be able to see the actual dimensions, right, so.
This height.
This height from the ground is L1.
Right. So this height from the ground.
Is PZ so this difference is PZ minus L1? You are correct?
Right from the geometry we are able to see these things easily, right? So.
We can write that as PZ minus L1. OK now for a triangle, see now this part is a triangle.
OK, now if I take this triangle separately.
Let me see. OK. So I am going to take this triangle separately. Nowhere. This part is Theta 2. But you know this angle can be written as. OK. Now let me see what symbols to give.
OK.
Let me call this as a.
B&C. Right. Each side of the triangle. Now the triangle is like this.
The triangle is like this. I am going to call this as a M is equal to L2, P is equal to L3, then the 3rd one is C right now this angle is.
You know, I can put this angle as capital BI can put this angle as capital a I can put this angle as capital C, right?
So that is all I am doing right? So if this is C then this angle has to be capital C, right? This angle will be capital A.
This angle will be capital B.
So yeah, anyway, it is simply mathematical symbols here. OK, so from this, what can I find out?
Now we have to use 2 rules, one is called the sine rule and the other one is called the cosine rule. OK, now I was establishing the points.
Here and here. Now, why did I do that? Now I need to find out the length C small C from my analytical geometry. Whatever I learnt in back in 10th standard. I am just writing PX -0 the whole square.
Plus PY -0 the whole square plus PZ minus L1. The whole square is this distance now from this point to this tool point, this is the distance, right?
This equation must be familiar to you length equation, PX square plus PY square plus PZ minus L1. The whole square, right?
So I know this length I can calculate C, right? OK. So from this, you know, I know now I know.
You.
A because I already know L2, I know B because I know L3. Now I already know C because I can calculate that right? So from this can I find out any angles? OK, so from the sine rule?
Now you must have studied this. Of course. You have studied this, but I don't know how many of you remember so.
A by sine a.
Is equal to B is the length b by sine, b is equal to C by sine C, right? So we have this condition which is applicable.
And we also have the cosine rule.
Right, which is a square. OK. So if I want to find out C.
This C If I want to find out or if I know already from that I can get some information C is equal to a square plus B square -2 a B Cos.
C Capital C, right.
From this I can find out.
C right, so that means I know small CI know. So now this small C is known to me A is L2 square plus L3 square -2 L2 L3.
Cos C right now if I want to find out C.
All I have to do is C minus L2 square.
Plus L3 square. Sorry, minus L3 square.
Or, you know, sorry I have to do it this way. I have to take this to the other side. L2 Square plus L3 square minus C divided by 2.
L2, L3. So this side will be Cos C right? So now from this I can find out C, right? So C will be Cos inverse of this.
Right this I can do so once I know C if I know this angle, what is this angle?
This angle is Theta 3, right? So Theta 3 is equal to 180 minus C so I will I will be able to get the value of Theta 3 right?
So 3 to 3I can solve just by this cosine rule. Right now you can actually.
OK, sorry this sorry this is C square.
Did anybody tell me a correction?
Nobody noticed it.
C Square thank you. Good.
It is C square because otherwise I have to put a square root here.
So when I write the cosine rule, it should be C square, right? So it is nothing but the Pythagoras theorem.
For non for angles without angles which are not 90 right? So it is nothing but the Pythagoras theorem. If you put 90 then you know this cost term will disappear. So it is C square. Sorry so this has to be C square.
So this is C square. OK, so now everything will will fall into place, right? So this will be small C square.
Small C square. Yeah. C square is equal to a square plus B square -2 a B Cos capital C Now again, the reference is actually given here, right? So law of cosines. Now I have put the formula here so.
The square has to be there.
Now sine rule a by sine a, b by sin BC by sin C, right? So.
OK, so C square have included now. So from this I can get this. Now I can expand the C square. So if I call this as C square then I will get PX square plus PY square plus PZ minus L1 the whole square.
So this is my C square. I can just put it there.
I can just Simply put it in this particular place. I will be able to get this angle C, then from that I can find out Theta 3 right very easy. So I got Theta 3. Now once I have Theta 3 then I can put it in this part of the sine rule.
Right. So B by sine B is equal to C by sine C, right? Whatever CI get here I can just put it here then this part will just become a number so.
I can write sine B is equal to b sine C divided by small C right from this I can find out capital B.
Sine inverse of B sine C divided by small C, right?
So I got this part of the angle, so that means you know B is small here. Now I have one more part here.
I can call it as alpha.
I am just going to call the lower part of this angle is alpha. I can write Theta 2 is equal to alpha plus capital B. So now from.
The sine rule I can find out B from the cosine rule. I can find out C from CAI can already find you know I can anyway find out Theta 3. So I have already found out Theta 3. Now Theta 2. There is only one part remaining right this.
Alpha now alpha. If you see this part is a triangle.
Now let me use a different colour.
This part is a right angle triangle.
Right, this part is a right angle triangle. I can. I can make this as right angle, right? This is vertical so I can write sine alpha is equal to opposite PZ minus L1 by hypotenuse, hypotenuse is small C, right?
So alpha is equal to.
Sine inverse.
PZ minus L1 divided by small C.
That is it. I got alpha so from alpha and Capital BI can find out Theta 2 directly. Now the only thing is Theta 3 whatever you find will always be. See in this case, since you are taking it from the geometry, Theta 3 will be positive.
Now you have to put a negative sign, right? So Theta 3 whatever you get, you add a negative sign.
For your application, because in this case you have to measure the angle clockwise, so you have to add negative sign.
Add negative sign for the final answer right? So this is something you have to do.
Now this is your geometric method. See now if you have to solve by the geometry of the arm. This is what you have to do. But this is something you can do for simpler arms. Or you know for 3 degree of freedom arms this is possible.
Now, if you are going to deal with a 6 degree of freedom arm, this will be difficult, but what they typically do is when they are solving a 6 degree of freedom problem, the inverse kinematics problem is going to be fairly complicated. Again, matrix inverse method can be used.
But again, you will end up with a lot of trigonometric equations. In that case, what they do is they use the matrix method matrix inverse method along with the geometric approach. Then somehow you know they take whichever solution is appropriate for that.
Condition. So if you are dealing with a very complex arm, you have to know this method also along with the matrix inverse method. So that is why textbooks keep using this method also right? So I think I have confused you well enough.
Right, with all the mathematical concepts. But I have given the formula here. Now you can also go through your basic mathematics book and if you try to draw a triangle and find out the unknowns, if the lengths are given. So all we are doing is you know a triangle is there.
All the lengths are known and we are finding out the angles. The only 2 rules that we need are, you know the sine rule and the cosine rule. Once we know we can find out all the unknown angles. So that is what we have done. Nothing very complex actually, but it appears to be very complex, right?
It appears confusing, but in fact if you view this as a simple triangle, it is not complex, right? So any questions so far?
OK, where where else did I miss?
C square.
OK, that was added.
Sine alpha.
And sine alpha.
OK. So you you have tried to answer.
Yeah, if you have. If I have missed any terms. Yeah. Corrected. Thank you. OK. So if I have missed any term, let me know.
Yeah. Any other any questions so far we have, we will have one more lecture on inverse kinematics where we will discuss the matrix inverse method for a few problems and with that.
Inverse kinematics topic will be over and we will discuss a little bit on mechanisms whatever is.
A left with that you will be ready for the mid semester exam, right? First 8 lectures will be there for the mid semester exam, so one more lecture on inverse kinematics and one more lecture on mechanisms will be ready for the mid semester exam, right?
OK, then we will stop here. If you have any quick questions you can ask. Otherwise we can conclude.
OK. Thank you very much. Have a good night.

GLYNN JOHN . stopped transcription