Mechanisms and Robotics (Merged - DEZG561DMZG561ESZG561)(S1-25) - Prof. Glynn John-20250906_175146-Meeting Recording

September 6, 2025, 12:21PM

2h 3m 31s

GLYNN JOHN . started transcription

GLYNN JOHN .   0:04
So I just wanted to complete inverse kinematics before we complete all the lab exercises.
So.
Uh.
OK, the recording is started.
OK so in today's lecture we will discuss the inverse kinematics of a scara arm. So actually in the last class we solved the inverse kinematics problem for.
An articulated arm using the geometric method. So here we can use the same approach to solve for the inverse kinematics problem of a scara arm. Then we will discuss how to use the matrix inverse methods, right? So we will start with.
A 2 degree of freedom RR planar arm. So that's the simplest arm possible where you will get.
The angles added or you know you will get sine a plus B and Cos a plus B so or the angles coupled. So we will see how to decouple the angles in this problem and the same thing can be applied to a 3 degree of freedom articulated now.
OK, so we are familiar with all these. OK, so here we have a 4 degree of freedom scara arm. So if you write the forward kinematics equation for this arm, it is fairly straightforward. So you can put the Z axis.
Like this?
X axis like this. So in that case the Y axis is going to be here. So this point will be the origin, zero, zero, zero. So we can write this as so we can go up.
Straight like this?
Z then X&Y so we can start with trans ZL one. Then at this joint we have the revue joint. At this point we have the revue joint, so rotate.
Z Theta one. Then we can come to this point.
So.
Like this XYZ, so Trans XL 2.
And there is a small offset here.
This L2 3 so we can go up.
Trans Z.
L2 3.
And we can, you know, stop there, right? So rotate. OK, so now this is gonna be T0 to one. Then for the second one, we can write rotate.
Z Theta 2.
Trans X.
L3.
Then when we do that, see now after we complete the first.
Movement or first set?
Of movements, then we will be here. Right? So now this will be X1, Y1, Z1. So this will be X naughty Y naughty Z naughty then with rotate Z Theta 2 and Trans XL 3 will be here.
Will be here.
Then from here.
What we can do is rotate.
X 180, right? So if you see from this side?
And if you rotate about 180 degrees, your Z axis will point downwards.
So that axis will point like this. X is gonna remain at the same point Y is gonna come to this side, right?
Now with this we can.
You know right T1 to 2 right. Then at this point we have the extension, right? So the.
The prismatic joint, and we already have the Z axis pointing in the direction of extension, so trans.
ZT 3 right so with that, we will come to this point.
We will come to this point. Z axis will be pointing here X&Y.
Uh.
Now this point.
What you have here this will be X2, Y2, Z2 then this one. The latest one will be X3, Y3 and Z3 and finally you have to finish at the tool point.
So this should be a Z4, right?
Now this is gonna be your T2 to 3 and the last one is.
Uh.
Trans.
Sorry, rotate.
Z Theta 4 because you have you know a Theta 4 at this point.
Centrans ZL 4.
So with that you can multiply with the origin 0001, right? So this is going to be T3 to 4, so you have 4 degrees of freedom and you have the number ending with 4. So a last coordinate system is going to be like this X4, Y4.
Z4 right, so this is something we did earlier. So I'm just repeating it so that you will get.
The idea of how to write the forward kinematics equation right now, how many of you tried the assignment? I saw some of you had tried and you know I think.
One one person had got a total negative marks also right? So my suggestion is you prepare you work the problem, you work out the problem and then you try to answer the questions. If you simply go and try your luck, you will probably end up.
With negative marks. So did anybody try?
Is anybody sure?
By the quiz.
Was it easy? Difficult.
OK, only one person tried.
No.
Now I will discuss the solutions once the quiz is over right. Once the due date is over, I will discuss the solutions. OK, so anyway you my only suggestion here is you just try to solve the problems and then.
You answer the questions right, don't simply go and try your luck there right? You solve the problems. Just take a few hours or you know take maybe less than an hour, solve the problem and then.
Try it. OK so.
So this is a trans rotate 180 OK trans ZD 3 then. OK, so this is the solution. Now in terms of the link link coordinate diagram, you can actually start with the origin here. Now you can also start with the origin at this point.
Right now you can also start with the origin. At this point. The reason for that is you know this part is stationary, right? So you can put the origin here and also write the equation. But the only problem with that is if you put the origin here for example.
Now all your Z will become negative because you're going below the origin, right? So yes, if you wanna deal with that, then it's fine, right? So if zero is going to be here at this point, then everything below that is going to be negative.
All your Z will be negative so.
Now our entire Workspace Z will be negative. So if if it is something that you can easily visualise, then yes, you can write it that way. Otherwise, if you want to keep everything positive, you can put your your origin here and entire Z will be positive, so the height from the ground will be a Z which is.
You know, much easier to visualise, right? So that's that will be the only difference, right? So if you see some textbooks the way they have written, they will put the origin here also that is also correct. But you can avoid it. So now in our case you know the link coordinate diagram will be like this.
Z Naughty right select this. OK now when you.
Who?
Multiply all the matrices when you expand this equation and multiply the matrices. This is what you will get PXPYPZ so you will end up with an equation like this right? So again if you have to solve this particular.
Problem you have to decouple the equations or otherwise you have to use the geometric method right? You have to use the geometric approach, but if you see PZ is solvable so so now sorry D3 is solvable, so since we.
Know the value of PZ we all we have to do is just take all these terms to the other side minus L1 -, L2, 3. OK. So let me rearrange this so I have to take D3 to this side.
And PZ to this side, so L1 + L2 3.
Minus L4.
Minus PZ is equal to D3, right? So everything on the left remains in the same place.
OK, so now we got D3 directly.
Now the only thing is you know.
The equation is coupled and we cannot get Theta one and Theta 2 immediately, right? So now what we can do is we can look at this arm from the top. If you see this arm from the top, how will it look? Right. So if you look at the top view of this arm.
Just a second.
OK, now if I look at the top view of this arm now, it's gonna look something like this.
Right. So your origin is gonna be below this particular point. There your origin is gonna be below this point. Now what you will be seeing.
If you see from the top, your X axis is going to be like this. Your y axis is going to be like this so.
If you convert this into a 2D problem, you will see.
X&Y and the origin is going to be here. Nowhere. You are going to have PX&PY, right? So this is how this will look. Now this will be L2. This will be L3.
Right.
OK, L2 and L3. So that's all. Now if you see this, this is very similar to the problem that we solved in the last class. This is just the same triangle.
Just the same triangle. Now this angle is going to be Theta 2. Right now this is going to be alpha, right? So if I take this triangle out.
Then I can find out.
The angles so using the same cosine rule and the sine rule I can simply solve this problem.
Now you want me to do it or you want to try it on your own? It is the same thing. Pretty much the same thing. Now I can call this as aim.
B&C. Now, do you want me to do this or can we skip this?
What do you want? What do you want to do?
Nobody is answering.
OK, so now this is a triangle, right? And this angle is gonna be Theta 3. So this B plus this alpha is gonna give us Theta 2. So we can write.
The signed rule.
Uh.
So a by sine a.
Sine A is equal to b by sine, B is equal to C by sine C right now this is going to be small C and this is going to be capital C Now.
What is this C right now? This point is PX, PY right now this point is zero, zero. Now if you consider only the XY plane, then C is root of.
PX -0 the whole square plus PY -0 the whole square, right? So C is equal to root of PX square plus PY square, right? So?
Now we have the value of C, so we can just take this part of the equation.
So this is small C.
We can take this part of the equation and.
We can.
Keep it for now. We will take the cosine rule.
And we'll find out the value of capital C. So in this case C square is equal to a square plus B square plus OK, -2 a B.
Cost capital C, right? So when you use plus here then you can directly put Theta 3 or if you use minus then it is the angle on the inside. So I'll just use minus for now. OK, so now C square is something you can find out from here. So that means PX square plus PY square is.
Equal to a square is L2 square plus L3 square -2 L2 L3 Cos C, right? So that means you can find out.
C from this capital C can be found out from this so PX square plus PY square minus L2 square minus L3 square divided by. Ohh sorry if I rearrange it this way.
I take this to the other side. L2 Square plus L3 square minus PX square minus PY square divided by 2 L2 L3, right? So Cos inverse of this.
P is equal to Cos inverse of this so.
Yeah.
I'm since this is negative, I'm taking this to the other side. I'm just rearranging it. OK, so from this I can get the value of C so from CI can get the value of Theta 3, Theta 3 is 180 minus C. Now later on we'll add a negative sign to Theta 3, right? So.
That is something we have to do because it is going clockwise, so one variable can be found out just like that and once you know C, you can actually put that in this equation and so you can find out B.
You can find out capital B.
So sine b is equal to small B sine capital C divided by small C, right? So now we know the value of capital C we know the value of BB is nothing but L3.
And small C is root of.
PX Square plus PY square.
Right. So some once we know sine B, then we can find out capital B. So B is equal to sine inverse.
Of this, now this part is all also given to us now. Now the only thing is.
We need to find out this alpha right, so this is PY this is PX. So in this case we have PY&PX right? So we can use tan Theta.
Yeah, of course we have the hypotenuse also. So you can use sine Theta if you want, or simply you can use tan Theta so alpha.
Alpha can be written as tan inverse.
Tan inverse PY by PX.
So.
Theta 2 is equal to alpha plus capital B.
Then Theta 3 is equal to whatever value you get. You put a negative sign, so this is included.
So to take into account the clockwise rotation, you're putting a negative sign there. This is Theta 3, right? So that's it. Now we are able to solve this problem using the geometric method, right? So.
OK.
Yes. Any questions on this?
Why nobody is answering? You are already tired.
For attending classes the whole day.
OK, now this is the same thing right? Only thing is only change will be the variables, right? The variable names. Otherwise nothing is changing here. Now what you can do is you can just check the solutions, check if the answer is correct using this particular method. Right? So you can try the.
Geometric approach. Now later on, once you understand the matrix inverse method, then you can try and apply the matrix inverse method to all these problems, right? OK, so the next topic is matrix inverse method right now how can we use the matrix inverse method for solving an inverse kinematics problem?
OK, now everything is clear till now.
Everything is clear.
I don't believe you.
It is not taking for it would not take 40 minutes now. See the lecture started when.
So it took me like 20 minutes.
So anyway, it should not take you more than half an hour. Any problem, right? Whether it is, I don't know. Matrix inverse method might take anything from 30 to 40 minutes.
But if you use see matrix multiplication, you have to practice right a little bit, at least for the exam. For the lab, yes you can. You can use MATLAB, but at least for the exam you have to do a little bit of matrix multiplication that you have to practice.
Yes, any specific questions?
OK. So we will go to the next one.
Now, yeah, before we go to the next problem. Now, what is an orthogonal matrix?
You you know what is a matrix inverse, right? Of course. We have studied that back in 11th standard. So matrix inverse is something we know. And again, you have studied that in advanced engineering mathematics. Right. You know how to calculate the inverse, right? What is an orthogonal matrix?
OK, any. Anyway, most of you feel sleepy, I guess.
I don't know.
OK, now.
When you have an orthogonal matrix right, the inverse is just the transpose, right? So that's it. That's the definition. So if you want to find out the inverse of a matrix. So now for example, if I take a matrix A, and if I multiply it with the.
Inverse of A then I am supposed to get the identity matrix right? So this is something we know.
So whichever way it it happens, you know you take the inverse and you multiply it with this matrix. Then you will get the identity matrix. Right now when when you have an orthogonal matrix, the inverse is just the transpose, right?
So nowhere, normally when we want to find out inverse of a we will, you know do it this way one by the determinant of a times the adjoint of a. This is the formula that we have learned.
Back in, you know 11th standard or 12th standard. When we calculated the inverse. This is the formula that we will.
That we will typically use right now when you have an orthogonal matrix. Now what happens is a transpose is just a inverse right? So now this makes it easier. Right now the rotation matrix is that we.
Use are always orthogonal. So now what we can do is instead of following this formula and taking a very long time to find the inverse, we can fall. You know we can follow this particular rule.
For an orthogonal matrix, so now that is only the rotation part is orthogonal. So in this case this rotation part. Now if you take the homogeneous transformation matrix, the rotation matrix alone is orthogonal. So now for the remaining part we have to use another formula. Nowhere they have given a formula.
So if you are given a homogeneous transformation matrix to find out the inverse, now you take this part alone. For that you find out the transpose. You simply write it. This part is anyway zero, right? This is anyway one.
The only thing you have to do is you have to find the transpose.
RT and you multiply that with this vector T&T and you put a negative sign.
Right.
So you take this and find you just simply multiply that now this is also the inverse. Right now transpose is also the inverse, so you just take that and you multiply with this particular vector and you put a negative sign. Then you will get the transpose. Now this method will be a lot shorter.
So now you can actually find this in all the robotics textbooks. You simply take the transpose for this part and you take the transpose and multiply it with this vector. Then you will get. Of course you have to put a negative sign there. Then you will get.
That last column also right? So that way finding out the inverse will be a lot easier, right? So here it is mentioned transpose. Here it is mentioned inverse it is just the same because it is an orthogonal matrix. Right? So.
Uh.
So this is it. Transpose is equal to the inverse matrix. Then that matrix is said to be orthogonal.
Fine, nowhere we have.
A simple problem or you know a simple manipulator. But the problem is not going to be that simple. It is going to be long. It is not difficult, it is just long. OK, so now this is an RR arm. So that means here.
You have an arm like this.
Right, so this can be.
The arm construction, so if you want you can just put an end effector like this, right? So here is one.
Variables and.
Theta 2 is another variable, so that means this arm is just a 2 dimensional arm which will move along this space, right? So that means it will move along this plane alone. So that is the arm since you have.
2 joins like this right?
Where the axis of these 2 joints are parallel at all positions. So that means automatically we will get the angles coupled right? So for this let us solve the problem.
You know the forward kinematics problem. Then we will do matrix multiplication. Then we will solve the inverse kinematics problem. So I have just taken the smallest problem possible to do this. Otherwise if I take a long problem then it's going to take a long long time. So now what I will do is I will put the arm in the home position.
So I am putting this in the home position. Now I am going to put the coordinate systems.
OK so.
Xeroxis xeroxis.
And Z axis like this right? So obviously X axis is going to be aligned with the link. Here also X axis is aligned with the link. Now if this is the case y axis will obviously be pointing towards the top because it is a right handed coordinate system.
OK, so now with this we can write the forward kinematics equation right? So first 1I will call this X naughty Y naughty Z naughty. This will be X1, Y1, Z1. Now this will be Z2 because this is a 2 degree of freedom arm.
Right.
So now we can write the equation.
OK, so starting rotate.
Z Theta one.
Trans X, right L1.
OK, let me put small L1. OK then.
So this is L1. This is L2 right? Then we are at the second joint. So this is T0 to one.
Then rotate Z Theta 2.
Trans XL 2.
So that means we'll come to this point, right? If this is L2, then we'll come to this point when we come here.
We are in this position. X is here.
Z is here and y is here. So all you have to do is now we cannot do Y rotation. So rotate Z 90 the moment you do that, X will go to the top.
X will go to the top like this.
And Y will go here. Then once you do rotate X 90 so you have to see from the top.
Rotate X 19.
And multiply that with the origins.
So when you do rotate X 90 then automatically you will you will reach this location like this Y will come here and Z will go reach its location and X will remain in the same position.
Right, so this is it. So this is T1 to 2, right? So 2 degree of Freedom arm. So this number ends with 2, right? OK, so now what we will do is we will take this equation, do the matrix multiplication, right, so.
You know, small demo. Now you can take a piece of paper and do the matrix multiplication along with me or you can go ahead of me. Right. So do that. Now, finally don't tell me the exam. It was too difficult. Right? Unless you practice now.
It will be too difficult in the exam and you will make very silly mistakes while doing the matrix multiplication and you know towards the end of the semester you will write a comment saying that you know after this robotics course I learned matrix multiplication never learned robotics right. You will write all kinds of comments like that.
But I cannot help you. So you have to practice this matrix multiplication. OK, so I am going to do it so T0 to one.
So rotate Z, Theta one so Cos Theta one minus sine Theta one sine Theta one Cos Theta 100000001.
And again here 000 right then multiplied with Trans XL one so 1111. So this is just a diagonal matrix L1 zero.
000.
OK, so we can multiply these 2 and get T0 to one.
OK, so Cos Theta one.
Minus sine Theta one.
Zero.
L1 Cos Theta one right.
OK, Sine Theta one.
Cos Theta one.
Zero.
L1 sine Theta one right. OK, 3rd one is anywhere 001 and zero last is anyway 0001.
So with this we got T0 to one right? OK very simple. So next equation T1 to 2.
I can write the expansion of this equation now if you see rotate Z Theta 2, trans XL 2. Now if you see this part of the equation, it is the same as this, only the variables are changed right rotate Z.
So anyway, let me expand it for you. So it is Cos Theta 2 minus sine Theta 2 sine Theta 2 Cos Theta 200100 then 0001.
000 here and trans X.
L2. So that means L2 will come here 000.
OK, these 2 then apart from these 2 you have 2 more matrices rotate Z 90.
Now.
Rotate Z 90 is Cos 90 minus sine 90 sine 90 Cos 90001 then 000001.
000, OK.
Then rotate X 90 so I will put one Cos 90 minus sine 90 sine 90 Cos 90.
Right.
OK.
Then.
0000100001000. So that's it. So.
Yeah, I have somehow managed within the given space. So now if you multiply these 2 you will get something very similar to T0 to one. So I will just write it Cos Theta 2 minus sine Theta 2.
Sine Theta 2 Cos Theta 2.
00100 then 000, then L2 Cos Theta 2 L2 sine Theta 2 right zero one. So I've just copied this matrix with.
The symbols changed. that's all I have done because these 2 are just the same with only the same symbols changing, otherwise they are just the same. So now what I can do is I can put the values of.
Theta here, right? So if I erase this and put it Cos 90 is zero, sine 90 is one, so this becomes -1 sine 90 is one Cos 90 is zero.
OK. Similarly cost 90 is 0990 is one.
Sign 90 here is one again. Cos 90 is zero.
So with this I can get these 2 matrices. I can multiply these 2 and I can write directly. So first one is zero.
OK. Second one again zero 3rd is one, then zero right in second row one.
Zero.
Zero and zero.
3rd one is zero.
1.
Then zero, Zero last one is always 0001, so I got somewhere he wanted to.
Then then I can multiply these 2.
And write T1 to 2 matrix right? So minus sine Theta 2.
Zero.
Cos Theta 2.
Right then L2, Cos Theta 2.
Pass the tattoo.
Zero.
Sign Theta 2.
L2 sine Theta.
3rd one is zero one.
Zero, Zero, then 0001.
OK.
So I got T1 to 2 right, so I have T0 to one and T1 to 2. Now I can just take these 2 and simply multiply right. Then I will get.
T0 to 2. If I do that OK, so let me save it.
OK, so now I can.
Copy it and put these at a corner.
OK, so now I'm just gonna multiply these 2 so T0 to 2 is equal to T0 to 1 * T1 to 2 right?
So now P0 to 2 is equal to, so I'm just writing these 2 Cos Theta one -, sin Theta, one, 0L, 1C Theta one.
SC Theta One Zero L1 sine Theta one.
00100001 so I'm just writing this again so that you know I want miss a term. Otherwise if I do this I might end up missing something, right? OK.
L2 C, Theta 2C Theta 2 0S Theta 2L 2S Theta 201000001 right OK. So now I can multiply these 2.
Now what I can do is I can remove this.
Yeah, I don't need this, OK.
OK, so now again a little bit of matrix multiplication. So T0 to 2.
OK, so now Cos Theta one. So now let me do this so -. Cos Theta one sine Theta 2.
Right.
Minus.
9 Theta one Cos Theta 2 right? So this row times this column.
Similarly, I can take this is going to be zero.
3rd one is Cos Theta one Cos Theta 2.
Minus.
Sine Theta, one sine Theta 2, right.
The last one is.
L2 Cos Theta one Cos Theta 2.
Minus.
OK, L2.
Sine Theta, one sine Theta 2 then last term.
Plus L1 Cos Theta one right so.
It goes like this.
I have to do the same thing for the next row, so I will take this row and multiply it with that column. So sine Theta one minus sine Theta, one sine Theta 2.
Uh.
Plus.
Cos Theta one Cos Theta 2 right?
OK. Then the next one is zero. 3rd one is sine Theta one Cos Theta 2 + Cos Theta one sine Theta 2 right. This part is easy.
So then L2 sine Theta one Cos Theta 2.
Plus.
L2.
Cos Theta one sine Theta 2 right.
Plus the last term is there.
Plus, L1 sin Theta one, so last term gets multiplied with this one and you get this term here.
So second row is over, 3rd row has got only one, everything else is zero. So first one is zero one.
Zero and zero, right the last rows anyway 000 and one right. So with this we got this matrix done.
Now you take T0 to 2 and you multiply that with the origin 0001. Then you will get.
PXPYPZ and one like this, right. So you will end up getting PXPYPZ now when you multiply that with the origin, what you will be left with will be the last column, right? So that is what this multiplying with the origin will give you. So you will get.
The last column. So if you see this is.
Cos a Cos b -, sin a sin b. So that is Cos a plus B, right? So if you remember the trigonometry that is studied in 11th standard. So again we are going back to 11th standard. So Cos a Cos b -. Sin a sin b is Cos a plus b sin a Cos b + Cos a sin b is.
Sign A plus B, right? So from this if I write PXPYPZ?
That's going to be L1 Cos Theta 1 + L2 Cos.
I can just write it like this Cos Theta 1 + L2 Cos Theta one plus Theta 2 right. Similarly L1 sine Theta 1 + L2 sine.
Theta one plus Theta 2 PZ is zero, right? It is a planar arm, so there is no Z axis here. So PZ is zero. So this is what we get right after matrix multiplication now.
We were able to guess this right in the beginning because the 2 axes were parallel, right? So the 2 axes were parallel, so they are parallel no matter what position the arm gets to, right? So from that we could guess that.
The angles will be anyway coupled.
So if you see the same equation is what we have here. So L1 Cos Theta 1 + L2 Cos Theta one plus Theta 2L one sine Theta 1 + L2 sine Theta one plus Theta 2. So this is what we got after expanding this and.
Doing the matrix multiplication right? OK.
Any questions on this so far?
You are able to understand.
How we were able to expand this and.
This part is clear, right? OK, so now we will come back and. OK, good. At least it is clear for one person.
Uh.
So now we will see how to. So now we did the multiplication. Now we know that the angles are coupled. So now we have to find a way to solve this problem. So now in this case if you have to solve the inverse kinematics problem directly then you will not be able to get a solution.
Right. So if you have to solve this, you have to decouple the angles one way or the other. OK, so now what is this matrix inverse method that we are talking about? OK, so if you have an equation like this T0 to one.
Times T1 to 2 you multiply this with the origin. You will end up with P, right? So this is what we have been using right from the beginning.
Now if I take T0 to one inverse.
And if I multiply the equation on both sides with this I will get T0 to one.
Times T1 to 2 then origin, then I will get T0 to one.
Inverse times P like this.
Right.
Now we know that inverse times the matrix will give you the identity matrix, right? So this will automatically become the identity matrix and identity matrix multiplied with any matrix will give you the same matrix. Again, the identity matrix will disappear, right?
So now what you will get here will be T1 to 2 times. O is equal to T0 to one inverse times P right? So if you.
Multiply this side and if you multiply this side then you will find out that the angles are decoupled right? So the angles will not be coupled anymore. You will get a set of equations and you have to solve those equations. Once you solve those equations you will be able to.
You know, get the answer for PX&PY, right. OK, So what should we do now, right? The only there is nothing complicated. The only thing we have to do is we have to find out T.
Zero to one inverse, so we already know T0 to one it is already available with us. That is why I did the matrix multiplication. We have T0 to one, we have T1 to 2. The only thing we don't have is this T0 to one inverse. So now we know.
The formula right so we can just go back and find out. OK. So T0 to one is here. OK, it's here.
Right, OK. So Cos Theta minus sine Theta, sine Theta, Cos Theta. OK.
So T0 to one.
Cos Theta one minus sine Theta, one sine Theta one Cos Theta 100100. Then here you have 000 then L1 Cos Theta one L1 sine Theta one then zero and one right.
So this is T0 to one.
Right. That's all we have. L1 Cos 0101.
OK, so nowhere we have to find out T0 to one inverse.
So we will.
Follow the rule here.
Or the shortcut, whatever you want to call it. So now this part is the rotation matrix, right? So we will take the transpose of the rotation matrix and write it here, right? Cos Theta one - Theta. Say I am just converting the rows into columns. That is the transpose. You will you will remember.
So I'm just taking the row and writing that as a column sine Theta one Cos Theta one zero right then 001 right. So whatever is a row here, I'm just converting that into a column.
So I already got the transpose. Now last 3 will be anyway zeros the the corner element will always be one, so now the only thing I have to find out is just this part of the matrix. Right? OK, now what is that we have to do?
Right now you have this particular vector.
Or this column matrix. Now this is called T right?
So now what you have here?
What you have here is R inverse or R transpose. Whatever you want to call it R transpose.
Now you take this R transpose and multiply it with this T and put a negative sign. Then you will get the elements of the last column. That's all right. So all I have to do is just take this matrix, multiply it with this.
Column. Then I I have what I need right? So I have to add it, put it for the first element, second element like that. So if I do that I am gonna get.
L1.
Cos Theta one square.
Right.
Plus, L1 sine Theta one square right? So I have to put a negative sign because of this minus here. So I will put a - here, right? Similarly for the second row.
I can put a - like this L1.
So minus L1.
Cos Theta one sine Theta one.
Plus, L1 sine Theta one Cos Theta one right or Cos Theta one sine Theta one or whatever it is just the same. So now the 3rd one is just zero.
OK, so if I rewrite this matrix then see now if you see sine square Theta + Cos square Theta is equal to one trigonometric identity. Nowhere you have negative and positive. So these 2 will get cancelled and become zero right? So if I write this clearly.
This is.
Minus L1.
Cos Theta one sine Theta 1 + L1 Cos Theta one sine Theta one right so that we know that these are the same terms with opposite sign. So automatically they will get.
Cancelled so which means that T0 to one inverse.
Is going to be Cos Theta, one sine Theta, one zero.
-, Sin Theta one Cos Theta one zero.
001.
000 then here it is minus L1.
Because sine square Theta + Cos square see nowhere you can take minus L1 outside Cos square Theta plus sine, square Theta, sine square, Theta + Cos square Theta is equal to one, so that becomes.
One. So that's why you you get minus L1 then you get zero, 3rd one is anywhere zero. Then the last is one. So this is your T0 to one inverse right? So now all we have to do is just take.
All the terms that we have, so now we know T1 to 2, we know T0 to one inverse all we have to do is just put the terms here in this equation and do the matrix multiplication and we will end up with some equations which will be.
Decoupled, right? So that is something we have to do next, right? Any questions so far?
Yes. Any questions so far?
Clear. OK, good.
Fine. So we will just take a break for 5 minutes. Yeah, I think you should be tired after seeing so many sine thetas and Cos thetas right. So we will just take a break for 5 minutes when we come back, we will do the matrix multiplication again and we will solve the equations. If you have any questions, you can type your questions on the chat window.
Welcome back. Shall we start?
While practicing for the exam now all the 2 mark questions probably will take you one minute or 22 minutes if you have. If you had already studied right.
So now for for forward kinematics, if you know the procedure you can you can simply write the forward kinematics equation. The only thing where I think it might take time will be for the inverse kinematics problem.
You can spend one hour for it still won't matter because you can finish the remaining portions and less than half an hour if you know if you know the answers though.
Right, so inverse kinematics problems usually will take like half an hour to 45 minutes, that's it. But don't focus on them. You focus on the other questions. Try to get the answers right, inverse kinematics. If you get the answer correct. Yes you can take it. Otherwise just you know complete it till.
To the point, you can.
That's the way it is, right? So whatever you get, that's fine. If it is difficult for you, it's difficult for everybody. All else in your class, so don't worry about it.
Right, but solve it, you know, till what you understand right or till what you can get. OK. So any other questions?
Yeah.
OK, now let's do the matrix multiplication here.
OK, so now we are going to take T1 to 2, multiply it with the origin.
And we are gonna take T0 to one inverse and then multiply it with P, right? OK, so T1 to 2.
OK, so T1 2 we have to take it.
And multiplied with the origin.
One and this Cos Theta, one sine Theta one zero minus sine Theta one Cos Theta 10001000 -, L1.
001.
This you multiply with PXPYPZ one right? So T1 to 2. What is it?
T1 to 2 is here right this one.
OK, so minus sine Theta 20 Cos Theta 2.
A Cos Theta 20 sine Theta 2.
And 010 right 010.
010000 L1 Cos sorry L2 Cos Theta 2 right.
L2 Cos Theta 2, L2 sine Theta 2.
Zero and one.
So this is it. So all we have to do is just do the matrix multiplications. OK, so in this case you will end up only with the last column. So L2 Cos Theta 2.
Right.
L2 Sin Theta 201.
Right, left hand side.
Is very simple, last column, then the right hand side.
PX.
Cos Theta one.
Plus PY sine Theta one.
Minus L1.
Then minus PX sine Theta one right? So this is one.
Plus PY Cos Theta one.
That's all. There's no other term there. Then PZ is here, then one.
Right. So that's all we have.
So if I remove the one on both sides then this is what we have. So this is your equation. One equation 2 and equation 3. Now of course we know that PZ is zero because this arm is a planar arm on the XY plane, right?
So PZ is anyway zero now. If you look at these equations.
The equations are decoupled.
Right, so Theta one is on one side, Theta 2 is on the other side, right? So this is what we have done using the matrix multiplication. So now by by multiplying with the inverse we we were able to decouple the 2 angles.
And then now we have to solve these equations, right? So we have to solve equations one and 2.
So how do we solve these equations now? One easy way to solve trigonometric equations is that you take them, you square them and add them right? Especially when you have sine thetas and Cos thetas. If you square them and add them.
Sine Square Theta + Cos square Theta will become one so that way.
OK.
There is a question, OK.
How we are deciding up to which step it will be T0. See now you can go till the next joint and before you operate on the variable of the next joint it is T0 to one.
That way you can take it right so that that way it will be easy to remember before you include the next variable in the equation, whatever junk you have after the first variable, you put that along with T0 to one, right?
Then once you start on the next variable till you go to that next variable right the following variable whatever terms you have, you put that along with T1 to 2 like that. That is that is the easiest way right? So.
Beyond that, there is nothing much.
Whatever terms you get between 2 joins then that should be good enough.
OK so now I am just going to take these equations and solve them. So now if you see I can I can take one square.
And 2 square I can just, you know, square these 2 equations and add them right? So if I take one square, what will I get? See? Nowhere. I have 3 terms, a plus B plus C, right? I can square them and find out. You know, I can just find out the square using the formula.
So L2.
Cos Square Theta 2 or L2 square, right? If I square this L2 square Cos square Theta 2 is equal to PX square.
Cos Square Theta one right.
A square plus B square plus C square.
PY square Cos square OK sign.
Sine Square Theta 2.
PXPY square sine square. Sorry, this is Theta one, OK?
PX square Cos square Theta one plus PY square sine square Theta 1 + L1 square a square plus B square plus C square plus L1 square plus 2A B plus 2 BC plus 2 CA, right? So again we studied this formula back in high school, so we are.
I use it plus 2A BPXPY Cos Theta one sine Theta one 2A b plus 2 BC. So nowhere we have a - so.
2 BC right -2 PYL one.
Sine Theta one.
Plus 2 CA. So I have to put the - 2. CAA is PX Cos Theta one, so 2 PXL one Cos Theta one.
Right, so I have squared the first equation. Similarly, I can square the second equation L2.
Square Sine Square Theta 2 on the left. Now this is a plus B whole square right 2 terms A plus B whole square, so PX square.
Sine Square Theta one.
Plus PY square Cos square Theta one.
A square plus B square plus 2A b. So since you have - in one term, so this is going to be 2 PXPY sine Theta, Cos Theta or Cos Theta sine Theta doesn't make a difference.
Right. Cos Theta one sine Theta one.
A square plus B square plus 2A b.
OK.
Hope I have not missed anything. So now we have to add these 2.
So now sine square Theta + Cos square Theta is equal to one, right? So from trigonometry. So that means L2 square sine square Theta 2 + Cos square Theta 2, right? So this will become one.
So on the left you will be left with L2 square, so similarly here PX square sine square Theta one + Cos square Theta one.
Plus PY square sine square Theta one + Cos square Theta one right.
Plus L1 square. Now these 2 terms will get cancelled. There's 12 PXPY, Cos Theta sine Theta and there is minus PXPY, Cos Theta sine Theta. So those 2 terms will get cancelled and.
2 PYL one sine Theta, 1 -, 2 PXL, one Cos Theta one right. So all these terms will become one, so next step.
Will be easy. L2 square is equal to PX square plus PY square plus L1 square -2 L1 times PX Cos Theta one.
Plus PY sine.
Sine Theta one.
Right, so this is what we have.
So I can rearrange this equation.
So here I have only 2L one OK.
So if I take this term to this side, and if I take L2 to this side, then I can write PX.
Cos I can write you know PX Cos Theta one plus PY.
Sine Theta one.
Now if I have 2L one here on this side I'll have PX square plus PY square plus L1 square minus L2 square.
Now if I bring this 2L one to this side.
Then I can remove it here.
So this is it. Here we have something right now. Again, we still don't have a clear cut answer for Cos Theta one there is Theta one. You know. Since we have Cos Theta one and sin Theta one right.
Now you must have used a formula. Those who studied dynamics and vibrations, you must have used a formula a Cos.
Up.
A Cos Theta plus b sine Theta is equal to r Cos Theta minus alpha right where R.
Is equal to root of a square plus B square and alpha is equal to tan inverse b by a right? So now yeah, it may sound scary, but this is something that we use very often.
When we end up with this equation, right, whenever you get a Cos B plus b sine B, right, a Cos Theta plus b sine Theta. Now this is a formula that we use so that we can get everything in terms of Cos Theta. Then it becomes solvable right?
So if I use that formula here, then this particular side will become r root of PX square plus PY square, right?
Cos Theta one minus alpha.
Now on this side, it's going to be PX square plus PY square plus L1 square, minus L2 square divided by 2L one, right? This is what we'll have now. If I take this PX square plus PY square that is root of PX square plus PY square to this side.
PX Square plus PY square. Then this side will disappear like this.
So if I wanna find out Theta minus alpha then I can take the cost to the other side.
Uh.
I can put.
Cos inverse like this.
Right.
Now if I put now if I want only Theta one then I have to take that alpha to the other side. Right now I can just take this alpha to the other side. I can Simply put + tan inverse P by a PY by PX.
Right now, if you look at this equation it it looks very similar to something that we did before. Right now this equation looks very similar to the equation that we got after using the geometric after solving using the geometric.
Up, you know method.
Right now, for example, let us go back and see this problem. The previous one that we solved today.
Right now, whenever you see Theta one or Theta 2 in this case, right?
Theta 2 is alpha plus beta. Now if you see alpha, this is tan inverse B by a nowhere in this case we have used sine Theta. But if you see the geometry is very very similar, right? Of course you can write this in terms of Cos also, but.
If you see it has a very similar structure. Fundamentally it is just the same. Right now if you see it is not similar, it is just the same because you know you see this is the same triangle.
This is your alpha. This is the other angle. Now this is the same triangle when you are finding out Theta one. This alpha gets added to this and that is how you get this. Similarly, this is your.
Theta 2 in this particular problem. So if you see it is the same triangle that we have right. So finally the solution will be just the same, if not similar, right?
OK, so this is Theta one. Now again by putting Theta one in one of the equations you can get Theta 2. That is also possible, but what I am going to do is I am just going to do something different. I am just going to rearrange this equation differently.
And again, I am going to square and add right. So just to make it you know more confusing, right. I would not say interesting more confusing. So L2. So I am what I am going to do is I am going to take this L1 to this side.
Right. And I am going to square the equations and add them right? Let us see what I get. So just just for fun.
OK, now L2 Cos Theta 2.
Plus L1.
PX, Cos Theta one plus PY sine Theta one.
Right, this is equation one. So I am I am just rearranging equation one equation 2I am simply going to write whatever we have here L2 sin Theta 2.
Minus PX sine Theta one right if I am right, PX sine Theta one plus PY Cos Theta one.
So now this is easy. We can do it quickly. OK, so I am going to square and add both sides. So one square plus 2 square, so L2 square Cos square Theta 2 + L1 square. Now this is going to be a plus, B whole square plus.
2L one L2 Cos Theta 2 so 2A b this is equal to PX square Cos square Theta one plus PY square sine square Theta one.
Plus 2 PXPY Cos Theta, one sine Theta one right. So I have squared both both the sides. It's a plus B whole square. Now on this side it's L2 square sine square Theta 2.
On this side its PX square sine square Theta one plus PY square Cos square Theta 1A square plus B square plus 2A B. So here you have a - in in one of the terms, so P.
2 PXPY so I am putting the - Cos Theta one sine Theta one right. So if you add these 2 then these 2 terms will get cancelled because one is +1 is negative.
Nowhere you you can combine these 2 terms sine square Theta + Cos square Theta is equal to one, so L2 square plus L1 square plus 2L one L2 Cos Theta 2 is equal to PX square.
Because here you have sine square Theta + Cos square Theta plus PY square again sine square Theta + Cos square Theta is equal to one and that's all right. So the 2A B term gets anyway cancelled, so 2L one L2 Cos Theta 2 is equal to PX square.
Plus PY square minus L1 square minus L2 square, right? So if I take this L1 L2 to the other side, right, this L1 L2 to the other side?
2L one L2 then this gets removed from this side. Then I can find out Theta 2 like this. So Theta 2 is equal to Cos inverse.
Now that's all.
So we are able to solve the inverse kinematics problem using the matrix inverse method, right? So.
Yeah, it took me, like, how long from the forward kinematics problem doing all the matrix multiplication, including the break, probably an hour or, you know, 4040 to 45 minutes. Whatever it is, right. So this is it. Now you have to practice it.
Then you know it will be a lot easier, right? Probably I am slower in matrix multiplication. I am. I am sure some of you will be able to do this faster, right?
OK, so this is it now we were able to decouple this and all we did was, you know, for solving, we did squaring and adding. And of course, once you find out one Theta, then you can put it in the equation and find out the other Theta that is also.
Possible, but I found this a lot easier because you know simply squaring and adding, you are able to get a solution.
So this is it. Now you can actually, of course you know how to use this with MATLAB. You can. You can try this with MATLAB and see if this is true. And of course in this case for Theta 2 we have to put a negative sign because you know this equation will not give you.
A negative sign directly, but you have to try that out and see you have to you. We have to manually put a negative sign anywhere, right?
OK, so I have given you the material here, right. Whatever we did manually right? So I have given the equations.
The matrix multiplication I have shown here right the all the equations are given in the slides.
So any questions till now on the matrix inverse method?
So here, whatever we had earlier, we had the equations coupled like this. Now after we applied the matrix inverse method we were able to decouple the equations like this.
Then yeah, in one case we simply squared this squared both sides like this, and we added them.
And another case I just moved this L1 to the other side and I again squared and added and I was able to get the answer right. So.
Yes, you can actually try this out.
And I have given the solutions, the formula, whatever formula we used, right, a Cos Theta plus b sine Theta. So r Cos Theta minus alpha where R is this root of a square plus B square and alpha is tan inverse b by M.
Right. So we use this formula and finally we were able to get all these answers right.
So Theta one.
And Theta 2.
Right.
Now till what point should we multiply so that is another question that you will have right? So if we saw if you try to solve a 3 degree of freedom problem using this method then you will understand till what point you have to decouple.
Right. So any questions on up to this point?
How long do we have to do inverse on the right hand side? OK, so that's the that's the question I was trying to answer now.
So long, OK.
Yeah. Anyway, this is the problem method for inverse doing inverse kinematics, right? So for solving inverse kinematics problem this is the problem method because it takes you through a structured approach. But if you are dealing with.
A more complex 6 degree. See now what we took was the simplest possible problem, right? A 2 degree of freedom problem where the angles can be coupled. So if you deal with a 6 degree of freedom problem right where you know.
3 angles can be coupled easily, so you have to apply the geometric method along with the with the matrix inverse method and then you may have to solve the problem. But if you are doing it.
You know, as a project now, if you are going to build a robot, then of course the solutions are available in textbooks, right? And in the search papers, you know, all all possible solutions are available. So you can take the solutions directly and implement them. Now if you are doing this in the exam, of course I am not going to give you a 6 degree of freedom problem. Maximum will be.
3 or 4 degrees of freedom, right? Extreme case will be 3 or 4.
So if the angles are not coupled, then yes, you can simply solve it. The angles are coupled. Then you have to use the matrix inverse method. Now if it is a matrix inverse problem and if you use the geometric method and still solve it if your answers are correct, yes we will.
Reduce 2 marks but still marks will be given to you right? So that is something if you are. If you think matrix inverse is going to take you too much time. If you if you solve it using the geometric approach, yes I will reduce 2 marks. But if your answers are right yeah marks will be given.
Right.
OK. But anyway, if you practice, they should not be difficult.
Right. Shall we go to the next problem?
Yes.
Any question so far?
No questions whatsoever. OK, fine.
Now we'll we'll look at this problem.
Right now we solved.
This problem using the geometric method, right? We took this as a triangle and we were able to solve it. Now if the same problem has to be solved using the matrix inverse method, how should we do it and how far should we go so that is the question right?
Now if you see this particular problem, you see this.
You see this?
Access.
Is parallel to this axis.
So now this is going to be 1Z axis. This is going to be another Z axis. Starting Z axis is going to be this one. So it's going to be something like this right? Again. This problem is familiar.
X naughty y naughty Z. Naughty. So this is going to be Z1, Z2 and the last one is going to be Z3, right? So of course to solve this problem we have to put the arm in the home position and then we can put the coordinate systems.
I like this. We could put the arm and the home position like this.
Then if you write the equations, it will be a lot easy to visualise.
OK, so this is the home position for this arm. Now in this case, if you see Theta 2 is here, Theta 3 is here and these 2 axes are parallel. So that means when.
You get your final equations Theta 2 and Theta 3 will be coupled right? So this will be Cos or sine whatever it is Theta 2 and Theta 3 will be coupled right? So because these 2 axes are parallel Theta, one axis is vertical.
So that is not parallel to any other axis. So Theta 2 and Theta 3 will be coupled so that is something we can say just by looking at this arm, because these 2 axes are parallel at all positions. Now when you multiply the equations when you multiply the matrices.
After expansion you will end up with something like this, right? So now we saw that. See Cos Theta one.
This is.
PXPYPZ.
Right. Cos Theta 1 * L2 Cos Theta 2 + L3 Cos Theta 2 plus Theta 3. So this is the equation we have. Similarly Sine Theta one.
L2 Cos Theta 2 + L3 Cos Theta 2 plus Theta 3.
L3 Sine Theta 2 plus Theta 3.
Plus, L2 sine Theta 2.
Plus L1. So these are the equations we have.
Now it is clear that Theta 2 and Theta 3 are coupled.
Right. We can see that these angles are coupled. So to solve this particular problem, first we have to expand.
Uh.
We have to.
You know, write the forward kinematics equation, do the matrix multiplication. You will end up with the solution right now. If it is an open book exam, you can take the solution directly from your notes and then you know further work on decoupling.
The equations right decoupling the angles. Now if it is a close book exam, yes, you have to do everything on your own, right? OK, now in this case we have T0 to 1T.
One to 2 and so on. We can get those matrices. So to start with you will get this equation right T0 to 1 * T1 to 2 * T2 to 3 multiplied by the origin.
Is equal to P, right? So this is something you are going to have. Now we can multiply this equation with T0 to one inverse.
To start the process of decoupling, so you will end up with.
An equation like this T2 to 3 times o times T0 to one. OK, so this is equal to.
T0 to one inverse times P, right? So nowhere. These 2 will end when you multiply these 2 you will end up with the identity matrix. You take the identity matrix, you multiply it with anything else, then the identity matrix will disappear.
So on this side you will have T1 to 2 * T2 to 3.
Times O equal to.
T0 to one inverse multiplied by P, right so.
Up to this part.
You have to complete right now after this, will the angles be decoupled?
Yes, anybody after you do up to this part, will the angles be decoupled?
That's the question you asked, right? How far should we go?
Will the angles be decoupled? Yes or no?
No.
What about the others? Only one person is answering.
Yes or no.
Square on both sides. No, not too early.
Still 2 will be coupled. OK, good. Why now? What about the others? Nobody else is answering. You have already fallen asleep.
Will it be coupled or not?
You can say just by looking at the equation.
Nobody else is answering. Coupled. OK, good.
OK, now if you see this equation.
Angle 2 and angle 3 are on the same side.
Right. If you see T2 and 3 are on the same side, right? So these are the matrices which will have. See now this matrix will have Theta 2 and this matrix will have Theta 3 and they are on the same side of the equation. That means the equation.
If you multiply at this point, still the angles will be coupled right? So that means you have to do this once more to separate Theta 2 and Theta 3. So that means you have to multiply this equation with T1 to 2.
Inverse again on both sides T1 to 2 * T2 to 3 times O equal to T1 to 2 inverse.
Then T0 to one inverse.
Then P now if you see.
T.
2 to 3 is on this side.
T1 to 2 is here now. This part will actually disappear.
Right, so this will end up as an identity matrix and that part will disappear. So you will end up with this equation.
T2 to 3 times O is equal to T1 to 2, inverse T0 to one inverse times P, right?
So this is the equation you want.
So you can directly find out the inverse of this directly. Find the inverse of this, do the matrix multiplication.
And get the equations decoupled or the angles decoupled. That is one way of doing it. Otherwise you already know Theta one. See now Theta one was never coupled in the first place right now. Why do you have to work so hard? So Theta one can be.
Calculated by by using this equation you can put it in a matrix T0 to one matrix. Then that means that matrix is just a number. Now you can invert this matrix using MATLAB.
Right. So or you can find out the inverse on your own. Whichever way is, you know, convenient for you. You simply invert this matrix. This matrix is just going to be a number, so you can multiply it. The equations will become less complicated that way, and then you can solve it so that is also possible, right?
Now, if you want a proper textbook solution, then you have to follow the steps like this right? So now when you put the value of Theta one what you will end up with. See now.
This is what I was talking about T2 to 3 on this side.
And when you expand all these equations, what actually will happen is that you know after you invert this T0 to one and multiply with P, you will end up with this part of the arm.
Right. So nowhere. When you start this problem, we go from the origin till this point and we do this rotation Theta one rotation right when we start this problem. So that means rotate Z Theta one.
Trans ZL One then rotate X 90. That is something we are doing now. When we multiply that entire equation with the in with the inverse of that particular matrix, what you will get will be now this rotation.
This rotation the the Theta one rotation right this Theta one rotation will be reversed.
So that means you will end up with the arm in this position.
Right. So you will end up with the arm in this position and you will actually get this triangle after you do the first multiplication. So what essentially you will get will be you know this planar arm.
L2, L3 and this part will be Theta 2 and Theta 3. So after you multiply with the inverse of this matrix, you will end up with the planar arm. So that means just the previous problem that we solved. So after you take this.
Matrix in the form of a number and multiply it. You will end up with an arm which is like this because the initial portion is all inverted.
Right. So that is one way of understanding it. Otherwise you can simply.
Multiply with with all these matrices and find out the inverse and you can solve this problem right? So finally what you have to do is you have to.
Do the matrix multiplication for this particular case. Then you will end up with the previous problem exactly. You will end up with this problem exactly because that Theta one is inverted, that Z translation is inverted. You will end up with this problem.
So from that you will be able to get a solution. So now that is something I am not going to do. Now you have to do this as homework, right? So whatever I have shown after this you have to do and this is your homework. Now I have I have you know.
Calculated everything.
Sorry.
So this part is homework.
So that means you can continue from here and you can complete the problem, right? So any questions till now?
So I've given all the matrix matrix inverse.
After you do the inversion, you will actually look if you you can look from any view you want, but after the inversion what you get is.
After the inversion.
That translation is gone. See now you did Z translation. That is gone. You did one theater rotation, right? That is also gone.
So that means this arm will be rotated and what you will see will be this view the on the front view you will actually see the true length of all these members. So that's typically what happens.
But that is not even needed. You need not visualise this if you are going to use the matrix inverse method, you can directly go ahead and solve this problem. Now if you take this Theta T0 to one inverse in the equation, then it becomes a bit easier. See nowhere.
After multiplying it with T0 to one inverse, see what I am getting. I am getting see nowhere in this problem. The numbers are given 6520250. These numbers are given.
So if you put these numbers and if you do this multiplication, see what you are getting, you are ending up with Z0. See now this is just, you know, a numerical error that you are getting here. So that means you are ending up with Z is Z equal to zero and you are getting.
You know.
A coordinate value for for this particular point, you are actually getting this in X&YXY&Z is zero. Now if you see this 0.04 which is nothing but zero here. So you are getting X&Y values.
Now T1 to 2 and T2 to 3 are there. That means Theta 2 and Theta 3 are still left in the problem. So now you have to go one step and decouple these 2 and after this what you are seeing is just the same same old planar problem. So that's what I am trying to say you are.
Seeing just the planar problem in this case, so you can actually solve it just like the previous problem. Otherwise, if you want to keep Theta one as it is, you can actually keep it and then.
Or get the equations also. That is totally up to you, but you know if you take this inverse and if you multiply it, you are reducing the problem to a planar problem. So that is actually shown here.
That's what I was trying to say.
Proctors problems open.
OK, practice problems are all all over the place, right? In textbooks I have, I have shared as many practice problems as possible as humanly possible, but you can actually see more practice problems given in textbooks.
All your textbooks and reference books you know you have a lot of practice problems. You can see if you can solve them right there are. You can take an example problem and see if you can get the same results.
Right up. Any other questions?
See, now you have to you cannot remove T1 to 2 and T2 to 3:00 in the first step you have to go step by step remove one at a time.
Because again, matrix multiplication, you have to follow the order. If you change the order then you get a totally different meaning, right? So you have to follow the order.
OK, now a problem like this. See now the inverse kinematics problem for the 5 degree of freedom manipulator has been solved in your textbook. You can actually have a look at it, but they have used both the matrix inverse and the.
The geometric approach for this right? So they have combined the methods and they have done so. If you see this, this and this, all 3 will be coupled. So you have to decouple one at a time and then solve it right? So that is something you can try again if you are.
Comfortable solving a 3 degree of freedom problem. Then you can try anything more than that. Now to start with, I would try this problem right again here. Also the angles are coupled.
This 4 degree of freedom problem, the scara problem. The angles are coupled. You can try that articulated problem angles are coupled. You can try that right? So when you get enough practice then you can try the bigger problems right? So any questions till now.
Any any questions till now?
Since we have time I we can discuss a little bit if you have any questions you can ask. Since we have a little bit of time I can go through some fundamental concepts.
Just a second, let me see if I have.
Any questions? If you have, you can ask.
Now all of you have a mechanical background.
Not exactly right. Some of you have.
OK, now let me.
OK, just a second.
OK, chemical also, OK.
Just a second. Any any questions till now?
Because with this we will.
Conclude inverse kinematics.
And next topic will discuss.
That is, in the next lecture we will discuss.
Some topics on mechanisms, whatever is left.
Just a second, let me go through some see now how many of you know how to select bearings, right? All of you should know those with mechanical backgrounds. You should actually know now if you are going to design a robot. Of course you have to constrain every joint.
So now let me explain a little bit on robot construction. Since we have some time how to construct a typical arm then.
I'll just show you how to select bearings quickly, right?
OK so.
Now.
OK, so if you are going to build a robot, of course you have to constrain the elements properly and you have to support.
What happened? Just a second, let me.
Just a second, I have to share the screen just a second.
Yeah, yeah.
Yeah, I was just searching for a file, but anyway I have to sketch right.
Yeah. Any any questions?
Headings based on.
See, now you have to first of all design the joins correctly, right? See now I will just discuss the construction and some of the challenges involved, right. So of course, when we discuss censors.
I will tell you where to put the sensors right now. Of course, you will have a a shaft, right? So whenever you have a joint right, for example, you have a joint right now I have been talking about joints and putting the joint axis in place. So now what is this joint? How is this joint constructed?
Now those with mechanical background. Now you already know it, right? So I am just explaining it for a general understanding for people with non mechanical background right? So now if you have a rod like this right now you have to.
Support this rod right now. This is a shaft. Now if you have to support this shaft, you have to support this on bearings, right? So now whenever I am talking about a joint axis now typically a joint is going to be.
A rod like this, or a shaft which is supported on bearing. So that means when I am constructing a robot now first joint, if I say I am going to have.
A vertical shaft. So now I have to support this vertical shaft.
Right. So when I support this then I have to support this on bearings so the bearings have to be positioned like this. Now this is the shaft which is resting on the bearings. Now this vertical member is going to be your shaft.
Butchers resting on this bearing.
Now you will always have a minimum of 2 bearings to fully constrain a shaft. Right now. In this case this shaft, this is your rotating member.
The shaft is your.
Rotating Member so this the bearings have to be properly seated. So now for that you need to have a bearing seat.
Which is nothing but another.
Component.
So nowhere what we need is a bearing seat.
So this bearing has to be properly seated and supported right? So now you may have another element to support it like this.
So anyway, I'm just simply drawing it so the drawings are not going to be perfect for now, so.
OK so.
OK.
So nowhere I have a stationary element.
I have a stationary member. This is a static member, so in that static Member I have put this bearings 2 bearings are there and they are going to support this vertical shaft right? So now when I say this joint is rotating about the vertical axis that means this.
Shaft.
Is supported on these 2 bearings.
Right. There are 2 bearings which are supporting the shaft. The bearings are supported by this bearing seat.
Right, so now this is a properly machined member, right? A machined member where you where this bearing sits accurately, and then this shaft will be supported on this bearing on these 2 bearings. Then the shaft will be able to rotate freely. Now to power this shaft. What should you do?
Now you can extend the shaft further down I can.
Extend this shaft further down. Then I can put a motor here. Now if I am going to power this using a motor so I can put a motor like this I can put a pulley. Then I can put another pulley here on this on the shaft.
Right now this pulley can be.
You know a timing belt pulley, so that means you know the pulley will have teeth.
And similarly.
This pulley will also have teeth.
And I have to put a timing belt if I am going to use a belt. I cannot use a flat belt. I cannot use a V belt because the belts will slip. So if I am going to use a belt, the belt should be a timing belt. That means a tooth belt or a belt with teeth.
Right. So then if I connect these 2?
Then you know I can. I can drive this particular joint. So that means, you know, I have to define the mechanical system like this, right? So somewhere I have to accommodate this motor and.
This pulley shaft, right? The shaft has to be properly supported on bearings. Now I can drive this using the motor. Now how do I how do I monitor this particular movement? Right now I have to put a sensor right? So what I will do is I can put a sensor like this.
I can put sorry.
Yeah. Now I just gone, I've just gone to the extreme of this page, OK?
In order to monitor the movement.
I can put a sensor here. Now I can just take by an optical encoder, right? I can just connect it with this shaft.
I can connected with this shaft. That means whenever the shaft is moving, this encoder will get a signal and of course you know this encoder has to be powered. So I will need at least 4 wires right? 2 for this positive and negative and 2.
Lines right? 2 signals will come from the encoder, so this way you know you have a mechanical system. So now this bearing seat can be mounted on this mechanical frame and you have.
The entire arrangement, right, so now this is for one joint. So now you have a motor. You have a power transmission system. That means a timing belt, pulleys and timing belt for transmission and to monitor the position of this joint. I am putting an encoder here. I am just mounting an encoder, of course.
Don't ask me this. The encoder going to hang in the air. No, you have to have a bracket for that. So this encoder has to be supported using a bracket without disturbing the belt drive, right? So you have to design all these mechanical systems.
Right, a bracket to support the encoder like this without disturbing the belt drive. And of course you need to have some kind of adjustment to move the motor so that the belt is properly adjustable, right? So all these adjustments are, you know these are.
Fairly straightforward for a typical mechanical engineer, because they must have seen a lot of arrangements like this. If you are new to this, if you have an electronics background or an electrical background, then a simple basics on you know, a simple study on the basics of mechanical design will actually help you.
Right. To understand, you know how to come up with, you know, basic components for an arrangement like this. Now, once you design the first joint, then everything else is just a repetition. But you know, the orientation will change, right? So.
You can just take this arrangement here.
Now your bearings will be like this, right? The second joint will be like this. So that means here you have to have a bearing seat. Your bearings will be here for the second joint.
And your shaft is going to be horizontal, your shaft is going to go through like this.
For the second joint, now the second link is going to be mounted to this shaft, right? So yeah, whatever component is moving.
That will be mounted to the second shaft like this.
And when you of course you can power this with another motor like this. Now you can have a second motor like this. You can have a pulley to the shaft and you can have a belt drive. Then again, you have to put an encoder here.
To monitor the position of this particular joint and you have to support the encoder. So you need to have a bracket there. Similarly for the 3rd joint you have to.
Have a shaft, again a bearing. To drive this you have to have another motor. You can position the motor anywhere you want, so another belt drive.
Another encoder on this joint. So like that, whatever we did for the first joint, same thing has to be repeated. Now you have to be very careful with how much weight you are adding at every point because that is going to influence the dynamics of the arm right now. Of course, we will discuss robot dynamics later.
So depending on where you put your motor, depending on where you put how much weight you put in in all these links will influence the dynamics of the arm, right?
OK, so now yeah its its time already. So if you want to learn how to choose a bearing right, you can simply download a bearing catalogue right? So now if you if you look at a typical motor you can actually see where the bearing is located.
Right. So now if you look at a typical motor now you know this is your bearing, right? So this is your shaft, right?
So if you download a typical bearing catalogue, so now in this particular case the bearing seat will will be on the cover of this motor, so that is removed. So the bearing is simply exposed in this case. So once you put the cover, the cover will have.
The bearing seat and that is not shown here. So now you can simply download the bearing catalogue of a typical bearing manufacturer like SKF and the bearing selection procedure will be given. You have to perform a calculation like this. You have to know.
How much radial load is going to come? How much axial load is going to come from that? You can use the formula given now. Of course the terminology. See now from the axial load and radial load you can actually get this ratio from the ratio. You have to choose these constants.
Right. So whether the axial load is more or the radial load is more now the last column will give you this e value. Whether the ratio is greater than this E or less than this E then based on that you have to put all these in this formula.
From the formula you will actually calculate static load factor and a dynamic load factor right? Then you of course have to select a life of the bearing, then static load factor dynamic load factor. Then based on that.
You have to go to a catalogue and pick a bearing, right? So once you perform the calculations for the static load factor and dynamic load factor, then depending on what your shaft diameter is, your static load factor has to be.
Less than the rated value for the bearing, right? So based on your loading conditions, whatever load you get, that should be much less than what is given for the bearing, right? So in this case this is your static load factor.
This is your dynamic load factor. So if your calculated value is let us say less than 1/4 of what you are selecting then you are very safe, right? If it is less than half of what you are selecting then you are fairly safe. So like that.
You can actually find out the life of what you select right now. If your probability of survival sin out here, you have a lot of these bearing life formula.
So if you want 99.99% reliability, then your static load factor of your so your dynamic load factor that you are selecting should be much higher than your calculated value, right? So based on the kind of reliability you want, you can choose a much bigger bearing than what you actually need.
And your reliability will increase, right? Of course, your cost will also increase, but you have to decide which is good for you. So now what you have to do is you have to download a bearing catalogue. You will see all these mathematical formulae or you can take any basic book on machine design right where they have.
Take in all these values from bearing catalogues and they have explained how the formula all these are empirical formula based on experiments, right? So they will actually explain how to use this mathematical formula right? So.
Yeah, now the screen is shared right? You can see my screen right?
OK. Yeah. Yeah. So now if you don't have a mechanical background, all you have to do is just take a basic book on machine design, go through the bearings selection page and parallel. You can download a bearing catalogue and you.
All these mathematical formulae will be explained there and you can actually perform calculations and select bearings. Right now a little bit of understanding of mechanical engineering will be needed, but it is not that difficult.
Right. So that is something you can try, right, so.
Just information, not a part of the syllabus. Right. I am just explaining it because a lot of people are here without a mechanical background. And what is, you know, commonplace for us may be difficult for them, right?
Now when we discuss embedded systems, you know, Mechanical Engineers will find everything difficult, but for them it will be very, very easy, right? So robotics, you know, is a multidisciplinary field, so you need to understand mechanical engineering. You need to understand mathematics.
Then you need to understand the electronics and you need to understand basic programming also, right? So when I explain controls they will understand quickly, whereas Mechanical Engineers will find it difficult. So for their benefit I am just explaining this part.
Right.
OK, now with that, we will stop.
So this part of bearing selection and shaft design is not there in the syllabus, right? That is something we studied in B.
OK. Thank you. Thank you.
Thanks for your wishes.
OK, then. See you the next class. Thank you very much.
Have a good night.

GLYNN JOHN . stopped transcription