Mechanisms and Robotics (Merged - DEZG561DMZG561ESZG561)(S1-25) - Prof. Glynn John-20250726_175119-Meeting Recording

July 26, 2025, 12:21PM

1h 59m 51s

GLYNN JOHN . started transcription

GLYNN JOHN .   0:14
OK, so you must have seen the course handout for this particular course. So the course handout is available in the eLearn ortal so you can just download it.
And let me share my screen.
OK, can you see my screen now?
OK, thank you. OK, so if you look at the course handout, so this is A . masters level Robotics course. So now this deals with.
Some of the fundamentals of Robotics and of course the main part of the course is the mathematical methods. So how to model an actual arm, A . manipulator mathematically and based on that how to perform calculations and how to.
Control the arm. So how to?
Program the arm. So overall this covers the entire spectrum of topics design, you know, right from the mechanical design, mathematical modeling, controls and of course programming. So controls and programming, you know.
They're just, you know, one Lecture each, but the main part is the mathematical modeling of the manipulator. So if you look at the textbook for the course, so this is the textbook is Robert Schilling, Fundamentals of Robotics.
So.
There's another textbook here, Introduction to Robotics by Said Niku. So this is.
Also A . good book. So this covers most of the topic, most of the topics. Now if you if you actually see the first book, I think there was some difficulty in getting this right. So Fundamentals of Robotics. That's also A . very good book which covers pretty much all the topics now.
Last time I heard it was A . bit difficult to purchase that book, so in that case you can go for textbook #2. So again which deals with all the topics and there are some examples also and for reference you can use Industrial Robotics by Groover so.
That is again A . basic BTEC level book which covers some topics really well, some history, fundamentals and applications. This book covers really well. Then reference 2, reference 3, some topics we'll use reference 3, some Mechanisms related topics.
We'll actually use reference three. Also reference 3. Some versions can be downloaded, so those SPECIFIC topics you can actually download and use and reference two focuses more on control sensing. So if you're.
Interested in learning more on controls? You can actually, you know, get reference to also, but all these are reference books. Now for textbook it's better to use textbook #2.
If textbook number one is available, that can also be used, right? OK, now what will you learn in this course? So if you look at the objectives, you'll get an introduction to the.
The applications basic necessity, then application of mathematical methods. So you know we'll deal with, you know, forward kinematics, inverse kinematics, then how to derive A . Jacobian and how they are.
Useful in calculation of forces and torques, right? Then the dynamics of A . manipulator. So how will the dynamic forces act when the manipulator is moving at A . particular velocity when it is accelerating? So you know those effects will be.
You know, dealt with in the course, so that's you know those, you know those topics will be covered in the course. Then again, sensors, actuators, you know the of course you must have studied some of the basics of sensors in your mechatronics course also, but we'll see the sensors related to Robotics and how they can be used in A . robotic arm and.
How the robotic arm can be constructed. So then we'll also deal with controls and programming, right? So those are the learning objectives. Now in terms of outcomes, after you complete the course, right, let's imagine that you study really well and you know, complete all the assignments.
Write the exams really well. Then after that, what will you learn? So you'll be able to identify manipulators. And if you see A . robotic arm, you'll be able to say what arm that is right, what manipulator type that is. So that is 1 fundamental.
Piece of knowledge that you'll gain, right? So moment you see A . manipulator, you'll be able to say what manipulator it is, right? Then if A . manipulator is given to you, you'll be able to mathematically model the manipulator. So that means you'll be able to create A . mathematical model of A . given manipulator. Any given manipulator, you'll be able to mathematically.
The model and find out where the tool is gonna go if you turn the manipulator by A . certain angle, right? Then if you understand the topics really well, then you'll be able to calculate the static and dynamic forces and torques right at the joints.
Then certain level of.
You know, so you'll be able to gain certain level of knowledge in terms of controlling A . manipulator using digital control. So I'll introduce you to, you know, A . control board, right? A basic control board, something like Arduino, with which you'll be able to control A . joint so.
If you work more on that method, you'll be able to control multiple joins at A . time and you'll be able to control A . manipulator. So at least you know from the application part we'll be able to do A . control using A . digital control board, right? Then we'll introduce you to programming.
So after that, if A . programming system is given to you, you'll be able to program A . robot by understanding the fundamentals. The fundamental concepts are same no matter what robot you're using. Then depending on what software is used, you have to learn that software and you'll be able to program the robot. So these are the learning outcomes.
Now, if you look at the topics in the first lecture, we'll deal with fundamentals of Robotics technology. So we'll talk about history, some basics of links and joins, then what we expect in the future, and human arm characteristics, if time permits.
Then in the second Lecture we'll talk more on links, joins, degrees of freedom and so on. And we'll also discuss ARM configuration. So how can you come up with different ARM types using A . combination of?
Links and joins.
And then after that we'll start discussing more on the mathematical methods. Forward kinematics 2 LECTURES and inverse kinematics, you know, 2 lectures. So.
After the end of the 8th lecture, you'll have your mid-semester exam, right? So you'll have one assignment before that. And after that, after your mid-semester exam, we'll discuss sensors. We'll discuss Jacobian, the concept of Jacobian and how it can be used for.
Calculation of forces in talks, then robot dynamics, 2 LECTURES and one topic, one Lecture on trajectory planning, then one Lecture on control and one Lecture on programming. So that's all we have. So the course handout is available at the eLearn portal. You can download it.
We'll have A . lab for this course, so.
A virtual lab using MATLAB. So in that we'll do experiments on forward kinematics and inverse kinematics. Whatever we learn in class, you'll be able to create functions using MATLAB and you'll be able to analyze.
Arms, robotic arms or manipulators and how they move based on the given input. So here we have A . list of the exercises, right? So when it comes to the evaluation conference, so we have 30% for.
One 30% for EC2 and 40% for EC3, right? So you'll have 20% for the lab, 20%, so 2020% for the lab. So in that you'll have lab assignments, lab quizzes.
And final lab assignment, right? So final lab exam or lab assignment. So total 20 marks for the lab, then assignment class assignment one 5%, class assignment two 5%. So the approximate dates are actually given here.
So after the end of the 4th lecture, most probably you'll get your assignment one on e-learn portal or it might be given as A . quiz. Also assignment 2 again after the end of the 12th lecture, right?
Right.
OK. So any questions so far? So that's all we have on the course handout.
OK. Any any questions so far?
Yes.
Till now, any questions? Everything is clear.
OK.
So how many of you have a mechanical background or how many of you have non mechanical background? How many lab sessions? You know, four lab sessions typically if you're going to, if you're too slow, then we'll probably have one additional, OK.
One additional lab session non mechanical, OK, electronics, non mech. You know what? What is your background? Electronics or electrical?
This electronics. I see one electronics. Biomedical. Oh my God. OK. Electronics instrumentation. Aero. OK.
OK.
Yeah, I know the majority is mechanical, but anything else?
What is ENTC?
Those who are from design engineering, I know you have a mechanical background or probably, you know, related to mechanical embedded systems, mostly electronics and some electrical, right? OK.
OK, so I think most of you are from some of you are from embedded systems. OK, fine. Anyway, so you know those with non mechanical backgrounds you might find some of the topics slightly challenging, but I'll.
Give you some reference books you know at that point, and you can actually I'll just tell you the names of those reference books. Then you can refer to those for reading some basic topics on mechanical, right? For example, when we talk about kinematics, mechanisms and dynamics.
Now there there are some topics which are mechanical, but you know B tech level mechanical. You know you need to understand some fundamentals to understand some of the topics. For that you have to read.
Some books on basic mechanisms and, you know, dynamics. So I'll just give you the names of those books, you know, when we discuss those topics, right?
OK, for those with mechanical background, I think most of the topics should be familiar, right? But again, you know these are masters level topics, so you have to spend time practicing what we learn in class, right?
Prerequisites. You know there are no prerequisites per SE right now. See if you look at the topics now, for example, if you look at the course handout.
See now if you are someone with a non mechanical background, see now if you look at the topics right now if you see degrees of freedom.
Right. Of course, I'll explain what we mean by degrees of freedom from a robotics perspective. But what is this degree of freedom, right? You should be able to understand, you know what it means. So that means some basic reading will be needed, right? So of course I'll explain all these terms now. Anything related to robotics I'll explain during the lecture, but there are.
Some basic topics. Now what do we mean by a mechanism, right? What is a spatial mechanism? I'll explain, but it's always better to read all these. You know, mechanical engineers already have studied this four bar mechanism, spatial mechanism and all that. They at least know what it means, right?
But beyond that, you'll learn in this course, but just go through these names so that you're familiar with these topics right now. For example, when I say kinematics, right? So what do we mean by kinematics? So you can just read a little bit.
Take a mechanical, you know, engineering book. Now any book on theory of machines, right? Or kinematics of machinery, things like that. You know there are books with with those titles. You can just, you know, read the first one or two chapters, just go through them, then you'll understand what we mean by kinematics.
Again, you know, I'll explain during the lecture. You need not worry. So it's not like I'm just going to straight away jump into the topic. I'll explain the basics, but it's always better to read if you if you want to be familiar with these names, right? What do we mean by dynamics?
Right. So what is a dynamic force? Things like that you could just read right a little bit. You need not. You need not be afraid. They're not so difficult. Things you know that you've studied in your 11th standard or 12th standard. Same thing.
And put in a different way, right. So that's all I would say. You've already studied what we mean by, you know, statics and dynamics and so on. But mechanical engineers already have studied some of those concepts and calculations, so they'll have an advantage.
But if you spend a little time, you should be able to catch up. OK, OK, now in today's lecture, we'll talk about some of the history, how this area evolved, right? So how robotics evolved, and we'll discuss a little bit of robot analysis.
Not to me, right? OK.
Now if you look at a mass production of automobiles, it actually happened in the early 1900s. So now at that point, in order to speed up manufacturing and to bring down the cost, special machines were designed, right?
So they actually design special machines to manufacture specific parts. Now, for example, one machine will be designed just to produce washers, right? One machine will be designed just to produce one particular fastener. So like that, special machines were designed and these machines were automated.
That means one person will just load a long rod from one side and just this raw material will be fed from one side and the components will be made in thousands and thousands right now.
The way these machines were designed, you know, they were purely mechanically controlled, mechanically operated. So they did not have an advanced, you know, circuit or a control system here. So they were operated using cams. So you know, the rod will be fed in after.
One particular operation gets over, then you know again the part will be made, you know the same part will be made again and again, right? So purely mechanical control, but it was also automation, right? So now this particular type of automation is called hard automation or it's also called fixed automation.
So automation is happening. Parts are produced automatically, but if you see the way these machines work, they were pretty much hard to change, right? So if they're designed for one particular part, they'll just, you know, produce that part again and again.
In thousands, right? So that is hard automation. Now the fundamental problem with that was, you know, if you want to manufacture a different part, then you have to alter the machine, right? You have to alter the tooling. At least you have to alter the machine a little bit.
So flexibility was not there. Now if you want to alter the tooling also now that's going to take some time. At that point you have to stop the machine, you have to load a different tool and then continue with production right now. Similar automation even now exists right now. For example, if you take a power press right?
So now in a production facility, you'll have a power press, which is, you know, operated by a motor. So that means this press will continuously be stamping out parts, right? You keep loading sheet metal from one side and it's going to produce washers. It's going to keep stamping parts. Now if you want to make a different part, then you have to change the tooling.
Then you can still make different parts and the automation will be very fast and you can produce thousands and thousands of components. So now that is good for very large volume production. If you're not going to change the component, if you're not going to change the design then.
That kind of automation is very, very good in terms of cost, right? Because you'll get thousands of parts for a minimal amount of time, right? Very minimal time and very minimal energy. Production volume can be very, very good. So in terms of cost, that's good, but in terms of flexibility, it's gonna be very.
Very poor, right? And again, while changing the tooling, you have to stop the machine. So that's another problem, right? If you stop production, that means you are losing time, you're losing money, right? OK, so later on what happened? They introduced electronically controlled machines. So that means you could program the machine.
You change the program, then you can actually produce a different part, right? So again, now this kind of automation is also available today. So that means your CNC machine, right? Computer numerical or computer numeric control, right? So CNC, so numerically controlled machine tools, right? So in this case.
You just have to change the program. In some cases you just have to change the tooling also. Now the machine will have an option to change tools. Multiple tools will be available and the machine automatically can go change the tool and do the manufacturing right.
Now this kind of automation is called soft automation, right?
Now again, the soft automation is also called programmable automation, right? So you change the program and you can produce a variety of components just by changing the program. A little bit of tooling change will also be needed. Now in soft automation you have.
You have a similar problem. Now in some cases I mentioned the tooling has to be changed, right? So at that particular point the machine has to be stopped, the tooling has to be changed, right? And in some cases the machine can automatically change the tooling. Now if you have a particular automation where you don't have to change anything.
Right, simply change the program and if you can write the program offline, right? So that means you don't have to stop the machine at all. You write the program using a computer and you can simulate the working of that particular program in your computer itself. Then you can simply load the program in your machine.
It can start manufacturing the part, right? So again, this comes as a part of programmable automation or soft automation, but here no change is needed. So that means physical stopping of the machine is not needed. Physical change of tooling is not needed. So in that case, this particular automation type is called flexible automation.
Right, so you don't change anything, simply change the program. It becomes flexible automation. Now in some cases you'll be able to change the process itself. In some cases the process will be more or less constant, but a different component can be made, right?
Now when I say process this, for example, if you take a robot, the robot can actually do spray painting. The same robot can be made to do pick and place. Same robot can be made to do, you know, welding or any other operation. So in that case your process itself is changing.
Now in the case of machines, CNC machines, you can change the program and get a different part made. And in that case again, the process pretty much remains the same, right? For example, if it is a CNC milling machine.
Milling is the process, so that process will be more or less constant. Now in some cases you may have multipurpose CNC machines, so one machine can do multiple operations. So in that case that can come under flexible automation, right? So if you are simply able to change the program without stopping the machine and get a different part made.
Then that is flexible automation, right?
OK, so there's one question. So programming in Matlab we'll be using, we'll be using the basic script files and function files, so we'll not be using Simulink, right? So basic creation of function files. Using those function files you'll be able to.
Erform calculations on forward and inverse kinematics. So that's what we'll be using in the lab. OK, now here if you compare.
Fixed automation or hard automation and programmable automation. So in terms of production volume, if you see hard automation or fixed automation is doing really well. Now here these numbers are pretty outdated.
So in today's standards this will be much, much higher in terms of programmable automation, right? If you are ready to change the tooling a little bit, then you will get many different parts. Right now the part variety can be increased if you are ready to change the tooling a little bit.
A little bit. That means if you're able to stop the machine, change the tooling, then again do programming, then the variety will be very, very high, right? You can get many different varieties, but if you want to stay in the range of flexible automation, that means you're not gonna change any tooling, then you're compromising.
On the volume as well as the variety, you're staying in the middle, right? So that is flexible automation, right? So you are somewhere in the middle. Now if you see robotics, it can come here, right? So.
It can be a part of programmable automation as well as flexible automation. So this is where you have robotics. So you'll get fairly good variety at the same time, fairly good volume, right? So you're somewhere in the middle.
OK, so going to history, right? So how did this field evolve, right? Why do we get this name right? So now this came from a novel, right? So.
Novelist Carol Kepek actually wrote a novel by the name Rawson's Universal Robots. So in that novel there were characters, right? So of course, very similar, very familiar story, right? So somebody creates a robot.
Then at one point, these robots become intelligent and start taking over. And so that's the story now, very similar to all our movie stories, right? So this was written in 1921.
And the word came from Robota, which means forced labor or slave, right? So initially the characters in this novel are mechanical beings which are used as slaves. Then later on they become intelligent and start taking over, right?
OK, so now overall this was the start and based on science fiction and based on novels, this particular field actually evolved, right? Even before technology could catch up, you know?
Movies and novels had very advanced robots, right? We can see in movies. The robots are really advanced in movies, but in the actual industrial scenario, it's very hard to see such advanced robots, right? So overall, this field evolved through novels and movies.
So now, in the 1950s, they had a movie, right? A Day the Earth Stood Still. So there's a robot there. In the 70s, we had Star Wars, right?
Some robots in the 80s. Again, Terminator, right? Of course, there were many other movies. I'm just showing some examples. So only through these movies and novels, this field became very popular. The technology was very slow to catch up, right?
OK, so now of course the robots in Terminator or in Star Wars, they're very, very advanced. We don't have such robots even now, right? OK, so how do you define a robot, right? If you have to define a robot nowadays, you know, if you watch TV, there are programs which show, you know, robot wars.
Where some remote control devices are actually fighting with each other. Now will they qualify as robots, right? So when we use a language just like that, we actually call everything a robot, right? But how do we define a robot?
So now here there are some definitions. Robotics Institute of America defines a robot as a reprogrammable.
Multifunctional manipulator, right? So now you have to be very careful with the terms used here. Each one has a meaning. Now design to move material parts or specialized devices through variable.
Programmable motions for the performance of a variety of tasks. So this is the definition given. So what do they mean by reprogrammable, right? So that means you need to program this device and the program should not be loaded just once, right?
Every time you should be able to change the program and reprogram this particular device. So that's what they mean by this particular term. So that means a remote controlled device will not qualify as a robot if you have a remote controlled vehicle or a remote controlled arm.
Then that will not qualify as a robot according to this definition, because it has to be programmable and reprogrammable, right? In some cases you load a program in an electronic circuit and with that it's done right? So it's not that way here. You should be able to reprogram it and.
Term manipulator is used now. When we use the term manipulator, this means an arm.
Right, we have an arm, right? So when we use the term manipulator, it means arm. So that means this device has to have an arm and the arm has to be reprogrammable. So these terms are used and multifunctional. Of course it has to be able to perform multiple functions, right?
So if you have a device which meets all these requirements, then it will qualify as a robot, right? So now of course this part is very simple. Move material parts, specialized devices through variable programmable motions, right?
Now somebody might ask, you know, will an AGV qualify as a robot, right? Automated guided vehicle. According to this, according to this definition, it will be reprogrammable. It will be multifunctional, but it won't have an arm. Now if your AGV has an arm, then you can call it a mobile robot, right?
But if it does not have an arm, then it's just a vehicle, right? Probably you can call it a vehicle or a programmable vehicle, but it's not a robot, right? So it has to have an arm reprogrammable and then it has to be multifunctional, right? Any remote control devices? Now what you see in Comedy Central, this robot?
And all these things, they will not qualify as robots because they are not reprogrammable. They are simply remote controlled vehicles, right? OK, so this is the definition. Now an industrial manipulator will typically fit this definition, right?
This definition, right?
OK, now here another definition. Robot is a software controllable mechanical device. These are. Now this is a dictionary definition, right? Software controllable mechanical device that uses sensors to guide or you know, one or more end effectors through program motions, right? Again, you have pretty much.
Everything that you need, right? So software controlled, software controllable. It's a mechanical device. Of course sensors will be used. Now there are end effectors. Now end effector is your hand right now in the case of a rope.
But what is equivalent to a hand is an end effector, right? The gripping device through programmed motions. So that means it has to be programmable and reprogrammable. So this is a dictionary definition, but still it pretty much mentions everything, right?
So if you uh.
Fixed automation. Those will not see now they're not reprogrammable, right? So according to this definition, fixed automation will not come under robotics, so you cannot call that a robot, right? Autonomous mobile robots. Now they see now autonomous mobile.
Well, robots, if they have an arm, yes, they will qualify as a robot, but otherwise that's a misnomer, right? So that term itself is a misleading term. Then you have to call it an AGV, right? Automated Guided Vehicle, right?
So yeah, these terms are used everywhere, but unless the device is reprogrammable, unless it has an arm, you cannot call it a robot, right? But we keep using the term pretty much everywhere.
Right. And now if you look at all these characters in movies, right, in science fiction movies, you see robots which look like human beings, right? So now those are called androids.
Androids or humanoids, right? So anything that looks like a human being is called an android or a humanoid. So industrial robots are not androids, right? And they're not humanoids. So most of the industrial robots are anthropomorphic, which means that they resemble A.
Human arm, right? So they resemble a human arm. They don't look like human beings, right?
So normally when we think about robots and now we think about a humanoid robot or, you know, an Android, but industrial robots don't look like that, right? So a typical industrial robot.
Looks like this.
Right. So it looks something like a human arm, right? So now here there is an example of Panasonic VR5, right? So this is a typical industrial manipulator or an industrial robotic arm. So you can actually see now here this is.
A welding torch here. So in this particular case this is a MIG welder, right? So a welding robot, so with the welding head mounted on the front and similarly you can use.
You know an end effector and you can use it for pick and place right? So that means here you can see all the applications material handling. So that means if you put an end effector which can grab objects then it can be used for pick and place operations. It can be used if you put a welding head here.
Then you can do welding, plasma cutting, spot welding, all kinds of welding can be done, right? So you simply change the tool and it is ready to perform a task, right? Of course, the software has to support the kind of programming, so.
If the software is, you know, versatile, then one robot with the software can do multiple tasks and you know it can do many different things, right? So like welding, pick and place, material handling, spray painting and so on.
OK, so if you look at the applications now, where do we need robots, right? In terms of industrial applications, wherever it is difficult for a human being to work right now, any particular task where it is very difficult for a human being to stay inside and work for long hours, right? For example.
The spray painting booth. So in this particular case, of course there are experts who can do spray painting really well. But imagine staying in this paint booth for an entire day, right? So it's very, very difficult, right? It's a hazardous environment and you can't inhale this paint spray, this aerosol for a long time.
And solvent will be there. So it's not a safe environment to work for, especially for long hours, right? But that's how people were working in the past and they had health issues because of that. So in such environments, it's always better to employ robots, right, in terms of.
Productivity, they're just as good as human beings are much better. So in this case they can work for 24 hours without getting tired. So you have many advantages. So wherever it's difficult for a human being to work. So that's where automatically robots.
Will be very very useful right now again welding applications like this right if you if you see.
Arc welding or any kind of welding, right? Any kind of arc welding or even resistance welding. You know, staying in this personal in the case of arc welding, you'll have fumes coming, right? You see fumes.
Coming and again, it becomes very difficult for people to stay in the work environment and work for for long hours, right? So that's one issue. The other thing is productivity. Now welding requires a lot of skill, so if somebody has to Weld a seam which is curved.
So that means the welder should be skilled and if he's going to work for eight hours a day and it's going to be very, very difficult for him to focus and do that task, right? So he might start making mistakes and because of that, the product quality will be affected. So if you automate this particular process.
The robot can actually do without getting tired and the accuracy will also be very good, right? Overall, your productivity will improve, your quality will improve and.
Your overall output will also increase. Your production volume will definitely increase.
OK, now here in this particular case, this is spot welding. Now you can see this spot welding tool. So that means you know two pieces of sheet metal will be joined together. So one electrode will press from the top, one will press from the bottom.
And the sheets will be joined together. Now this is a process which is used for making vehicle body structures, right? So if you take your car or your Jeep, right? So the sheet metal parts will be welded using spot welding.
Again, if you mount the proper tool on this robot, then it can actually do this task really well. Our typical car body will have about 10,000 spot welds, right? So imagine doing this manually, right? Again, even doing this manually will not be very easy because you have a spot welder.
The welder will not be easy to handle right? Because the welding machine itself will be fairly heavy, right? You see this spot welder here. This itself will be heavy. A person cannot lift it and turn it and so on right to do the welding. So in this case you have a much bigger robot which can lift this welder and.
This welding operation, right? So the output is gonna definitely increase and it can. So now you have to design fixtures and supporting equipment to turn this body, to turn this chassis and present it in the right position. So that is also needed.
The robot alone may not be able to do everything, so you need to have supporting equipment, fixtures which can lift the body, turn it so that the robot can reach into all the nooks and corners into the welding, right? So all these things are needed.
Then there are some medical applications, right? Of course, doctors won't allow you to automate an operation completely, right? So the way this is performed medically is that a doctor will operate a, you know.
A robotic arm remotely. So that's the way it is performed now. But a typical robotic arm, a medical robot can do a stitching of skin, right? Can actually perform surgery. But at one point, you know, the doctors have to allow it.
So as of now, it is mostly remotely operated, right? So robotic surgeries, even though they call it robotic surgery, but they are actually remotely operated. A doctor will be operating on a patient using a robot, but the doctor will be in a different city, right? So that's the way it is happening even now.
But at one point, if you have enough information that you feed in the software, the robot will be able to perform the surgery on its own. Even now it has got the capability. But again, when you try something on human beings, you need to get a lot of clearances from governments.
and from medical societies and so on. So medical applications are also there. Now Now here you have other applications like search and rescue, right? Military applications.
So in this case you can allow remotely controlled vehicles. In this case this vehicle has got an arm and this is operated using a remote control. So this will not qualify as a robot according to the definition because it's not a programmable device instead.
It's a remotely operated device, right? So you have an arm, it's remotely operated by a person sitting far away, so it's picking up a shell which is unexploded. Or in this case, there is a suspicious device placed in a car they're sending.
You know, a vehicle with an arm and they're picking that device, right? So bomb disposal, military applications. So now this is called Parkbot. So this is made of plastic and a soldier can actually carry it.
And it is remotely operated. So all these devices are also there, right?
So this is what you see here. So the soldier is carrying a pack pot on his backpack and this can be thrown into an enemy territory and this can be remotely operated. Of course a camera can be mounted and you can actually see people.
People movement. Then you can also ick U you know, unexploded shells like this, right? So all these things are possible. Of course there are, you know, more applications.
Right. OK. So these are, you know, current trends and what is happening now, right? Of course you have more things happening now coming to industrial robots, right? So any questions till now?
Do you have any questions?
U to this point.
Drones. Again, drones cannot be called robots because most of them are remotely operated, right?
Yes, any other questions?
Clear. OK, fine.
OK, now industrial robot history, right? So if you see Unimate, so this is the name of the first industrial robot. So this was developed in the 1960s.
Right. So initially there was a person by the name George Duvall, right? An American inventor by the name George Duvall. So he was working with magnetic tapes, right? So, you know, several decades ago we used to have audio cassettes and video cassettes. They used to have a magnetic tape.
And information used to be recorded on that magnetic tape. Using that we used to play movies or songs, right? So similar technology was used earlier. So he was an expert in these magnetic tapes and how to store and retrieve information from that.
Based on that, he actually came up with an idea of using that information stored in that tape to move an arm, right? So he thought, you know, a program can be written and stored on that tape, and then using that program you could move an arm. So that was the idea, and he actually got a patent for that.
So a design of a mechanical arm controlled by an electronic system with the information stored on that tape, right? So this patent was given in 1954, right? It was filed. So it was filed in 1954, granted in 1961, right?
So and of course this was an experienced person, so he had the technical knowledge. Then of course somebody has to invest money and work on building a product, right? So that was done by Joseph Engelberger.
So this person was, you know, an investor come, you know, a manager. So who was interested in this particular idea and he invested in this particular company. So he went into partnership with George Duvall and he invested in this particular venture.
And Joseph Engelberger is called the Father of Robotics, right? Because he was able to come up with a product using that, right? So they started a company called Unimation, right? Unimation Corporation in the US. And now this was the patent, right? So the patent was programmable.
Programmed article transfer. So the two people are here. So this is George Duval, the older person, and Joseph Engelberger is here. So now here you have a robotic arm serving drinks, right?
So they had the first robotic arm developed in the 1960s, early 1960s.
And so that's how the first industrial robot was created, right? A similar patent was filed in the UK at that particular point. See now here you have a very similar device patented in the UK, but you know this came into production much earlier.
Right. So this was Unimate robot, right? So now what you see here is a modern version of that particular robot, but the older version also looked very, very similar, right? So this was used by General Motors, right? So.
Automotive companies in Detroit, they started using these robots for machine tending, right? So if you are loading a part in a hydraulic press or in a forging machine, so then the operator has to stand next to a very dangerous machine, right? So somebody has to.
To load it and unload it. So that is called machine tending, right? So attending to that machine, loading and unloading it. So that operation was performed by this particular robot. So this was a hydraulic robot, so the movements were.
Made using hydraulic actuators. So this was able to lift heavy objects, right? So the same company came up with, you know, more models of robots. So now this is another model called Puma.
Universal machine for assembly, right? So a slightly different version. Now of course, when we discuss robot configurations, we'll actually be able to understand what configuration this is and what configuration.
This one is so that we'll discuss in the next class.
Right. So again, this model Puma was able to perform assembly, right? It was faster, fairly accurate. It was able to handle light materials and it was able to do inspections. So this was another model.
Then other companies, you know, started developing robots. So now here you see an example, Cincinnati Milochron. So this is again a hydraulic robot, you know, a much more flexible and you know, versatile robot.
OK, so that's about the evolution. Of course, you know, many other companies started investing, and after the American market was more or less saturated, Japanese companies wanted to develop robots. At that point, Joseph Engelberger actually helped a lot of Japanese companies develop robots.
Said, you know, of course, you know, Japanese models have become very popular.
OK. So what can we expect in the future, right? Robotics trends and future prospects. So if you see that now we come across this term very often, Industry 4.0, right? So what do we mean by that?
So the first industrial revolution happened quite some time ago, right? Some 200 years ago. So you know that revolution happened because of the steam engine, right? So once they were able to operate all the machines inside a factory.
Using a steam engine. So that means the steam engine will be sitting outside the factory and the machine will be running a very long shaft and from that long shaft multiple machine tools will be powered using belt drives, right? So that means you know one long shaft will be.
Running across the shop floor and there'll be one lathe or any machine, so that will get its drive from this machine. Another machine will be here that will get its drive from this machine. So like that, with one steam engine, they were able to operate multiple pieces of machinery, right? So they were able to perform manufacturing like that.
So that mechanization with steam power. So that was the first Industrial Revolution, right? So that happened about 200 years ago. The second Industrial Revolution happened.
About 100 years ago, right? So when automobiles were mass produced and at this point they started using electricity, right? So electricity was used and of course individual machine tools could be powered, so you did not need one long shaft to power all the machines. So electricity was there.
Mass production was happening with specialized machines. There were assembly lines, right? People do, you know, doing assembly in a sequence and making production faster, right? So that was the second industrial revolution.
3rd Industrial Revolution is happening now, right? So with the use of computers, we are able to automate so many things, right? So we have a lot of electronic systems, automation systems, all your PLCS were used in the 60s. Nowadays we have more advanced systems.
So overall with the use of computer and automation, we are able to improve production, right. So that is the third industrial revolution which is happening now, right now. The 4th industrial revolution is what we mean by industry 4.0.
So the 4th industrial revolution of course has not happened yet. So we probably expect that to happen maybe in the next 100 years or maybe longer or maybe happening in the next 10 years. We don't know, right. So what we expect here is that you have cyber physical systems that means.
Systems which can work on their own, so more or less autonomous. So you'll have a shop floor where robots can work without the help of humans, right? So that means you don't have to go to a factory. You can probably sit at home and.
Monitor production. And at one point, probably you have to supply the raw materials and you'll have finished products coming out without a lot of intervention by people, right? So that means I don't know what's going to happen to people.
But overall, it appears to be a scary situation. But that's the 4th Industrial Revolution that we are talking about, right? So if you start automating things and with artificial intelligence getting integrated into systems, at one point this is definitely possible. See now.
Teaching this course 10 or 15 years ago, this did not seem imminent, but now with the development of artificial intelligence and the common use of artificial intelligence shows that this can happen very soon, right? 4th Industrial Revolution is gonna happen.
Happen very soon. That's something we see, right? OK, so this is what we expect, right? OK, so if you look at progressive advancement in terms of robotics technology, where are we, right?
This is for Industrial Revolution per SE. Right now in terms of Industrial Revolution, we are in the third phase. We are moving towards the 4th in terms of robotics technology, where are we right? So in the first generation robots, right, the early robots developed in the 60s.
They were able to perform repeating tasks, right? They were non servo, so that means they did not have a feedback control, right? So that sensors were not very popular at that time or sensors were not well developed at that time, so they were non servo, so you did not have proper position control.
Proper feedback. You had just switches to show the starting point and the end points and they were typically used for pick and place operations. So these were the first generation robots now after that.
With the development of sensors, with advances in sensor technology, sensing devices were used for feedback.
Right. So every joint you'll have a sensor. The sensor will tell you where that particular joint is or you know what angle that particular joint is and you'll get that information and that'll go to a software and the software will be able to monitor the robots, right. So in this particular case now you'll be able to.
Do not just pick and place. You'll be able to perform. For example, if you want to make the robot follow a particular path in three dimensions, a task for spray painting or welding, that is possible, right? So with this particular feedback and the software control.
You can actually make the robot follow complex paths, right? So that's what we see in second generation robots. Now third generation robots we still have not seen, right? So what can we expect in 3rd generation robots?
You'll actually have artificial intelligence integrated, right? So now artificial intelligence development and artificial intelligence is happening now, so this is possible very soon. So the robot will be able to learn from its experience.
So that's what we mean by self learning. So this is possible using artificial intelligence and based on its experiences, the robot will learn and it will be able to work intelligently, right? So that's third generation robots. Now as of now, we are here in second generation, so this is where.
We are. So second generation is where we are as of now. So soon we can expect third generation robots, right? Maybe again in the next 10 years or maybe in the next 50 or 100 years, we don't know. OK, now 4th generation.
Robots. These are, you know, what we see in the movies, right? So maybe an Android, right? In some movies they actually have what is called a biological robot. So they try to integrate some kind of human like intelligence with a brain.
And you know, something like that may be possible at some point in time. You may have an electronic circuit functioning as a brain with artificial intelligence. That is possible. Or maybe something related to biology also may be possible. Again, we don't know. We are simply speculating as of now.
But you know, many companies are actually developing humanoids or, you know, androids which can, you know, function just like us, right? So you know that technology is what we.
What we may see in 4th generation robots, right? So after you see very intelligent robots, then you may see more advances in design and you might actually see a more advanced robot which will function like human beings, right? So.
Again, after second generation, everything else is just speculation, right? So this may happen, may not happen. We have to wait and see. OK, any questions to this point?
Till this point, any questions?
Yes.
Are you listening? Are you awake? Is anybody sleeping? Yes.
Not getting any response. OK, clear.
Uh, yes.
Yes, pick and place. Yeah. What's what's your question on pick and place?
The back in, back in the 60s, they did not have advanced sensors. So in that case they were handling metal components. In that case you will have the robot picking a metal part, placing it on the machine. Then after the operation is performed, it'll just pick that metal part and you know, load it in.
In a tray, so pick and place. You know you do not have force control back then, so you'll have one open close kind of arrangement in the end effector. So you'll have one maximum force, 10 force, right? So we'll just open.
And when you close, you'll apply the maximum force, whatever is set. So back then they did not have that force control and they did not have advanced sensors, right? But in today's robots you can actually.
Program a force control. That means even if you ask your robot to pick an egg, it can actually pick without breaking it, right? So that is possible today.
See now.
Now according to.
Yeah, people can always mention we are in industry 10.0, but based on this information what you see here, we don't have autonomous factories till date right the moment you have totally automated factories.
Without human intervention, then yes, you are at Industry 4.0. We are actually gearing towards Industry 4.0. We are not yet there. Three is where we are because we are able to do automation using a computer or computer like devices.
Right.
Yes.
Waiting to see. Yeah, you will find out very soon. Cyber physical. See again, you know what technology will be able to do this. That is something we have to wait and see, right? IoT is something we have now, but.
See now the level of usage and the reliability and the autonomy that it's gonna provide. So that is something we have to wait and see. But IoT in terms of monitoring a machine in terms of getting information.
Farmers actually use IoT for monitoring their pumps, right? So all these things are happening, but running a factory like this and having totally autonomous systems, that is something we have to wait and see.
Yeah, we may. Yes, we have to wait and see.
See now. Yeah, we'll we'll wait and see.
For the actual reports, whether that is happening.
See now robots are not that expensive now, not as expensive as they were 1015 years ago, right? So nowadays with 2 1/2 lakhs we can actually get a robot, at least maybe import at once.
But you know, for 10 lakhs you can actually get robots which which are.
Good for, you know, industrial application. So the price is coming down. So as more and more Indian companies manufacture robots, the price should come down.
EOT cranes definitely not.
See EOT cranes, you know.
They cannot be called robots. Again, you have an operator sitting on the grain for most applications. Yes, they are automated. If you have something like an arm. See now a regular robot can be mounted upside down and it can be made. It can be made to move along a slide like that.
If it is programmable, reprogrammable, all these, so you have to apply that definition and see.
See now you can give any name 4.0 and all that. They'll say we are gearing towards or somebody will claim to have reached you know 4.0, but we have to wait and see right till till it's widely accepted.
See, now again, you're saying phase one, right. So what is the final phase? When is it, when, when is it universally, universally accepted? So we have to wait and see all that, right. So overall, yeah, we are moving faster. That is something very clear from what we were, you know, from where we were 20 years ago or 15.
Years ago from there, if you see now we are moving faster. Again, everybody around the world is working towards it. So we're moving faster, but we are not yet there.
See now we'll go through some of the basic mechanisms for design, right? But gripper design you will actually see in a in a basic textbook. It's there's nothing complex there, right? So if you're going to design A gripper, if you refer a basic textbook on robotics, you'll actually see some basic designs and force calculations now, for example.
The reference books. One of the reference books by Grover, right? That shows you some basic gripper designs. So that is something you can do if you have a basic mechanical.
Design knowledge. You can actually design A griper. There's nothing complex there.
Yeah, legal complications. Yeah.
Legal complications, they are totally different, but our main challenges, you know, using the technology, right? Even now we are dependent on some countries for magnets, right? If people can't make magnets and they're dependent on some country for a magnet, then why?
Why talk about legal blocks here, legal issues here? Technology, basic technology has to be developed. Basic production capabilities have to be developed. Then we can worry about legal issues, right? OK, so now we'll just take a break, right? We'll just take a break for 5 minutes.
And so time is 6:56, right? So by 7:00 or 7 two, we'll just come back, right?
If you have any questions, you can type your questions on the chat window.
Hey, welcome back. Can you hear me? Shall we start?
OK.
OK. So now what can we expect in the future, right? So we were discussing the current trends. So in the future we may have more robots in the military, right? Public security, healthcare.
You may have co-workers, so that means, you know, humanoids who work alongside others, right? As of now, they have actually come up with some robots, now customer service robots in Japan and even McDonald's came up with a customer service robot.
You know, some robot which looks more like a human being, of course, you know, humanoid.
So we may see more of them in the future. Robots at home, right? There are humanoid robots which can actually perform basic chores at home, right? Honda had actually developed a model called a SIMO.
Which can actually walk around the house, right? So, you know, do some basic things like picking up a newspaper and things like that, right? So robots in education. In the future, you might have a robot teaching a class like this, right? So robots in education, entertainment.
We have remote controlled devices and entertainment. Now in the future you may have an actual robot entertainment. So all these things are possible in the future, right? And you might actually see more designs of end effectors, more advanced end effector designs in the future, right? So these are some of the things that you can expect.
So this is the robot I mentioned, Honda ASIMO. So this has been around for quite some time now, so almost more than a decade this has been around. And here you have Atlas from Boston Dynamics.
Right. So this, you know, robot can actually walk with two legs. You know, even Seymour can actually walk with two legs. They actually started developing this technology right from the late 80s and and it was able to work well, you know, in the recent years.
Again, Atlas has been able to do a lot of things. Now you can actually check YouTube videos and Atlas. You'll actually see a lot of things that Atlas can do, right? So these are some of the things, you know, some of the trends. And of course there are more.
Such models which you can see right.
So this is something that's happening now. These robots can be used in defense and of course for various other things. Of course, Boston Dynamics is actually a defense supplier for the the US military, so they are actually developing such.
Robots. So you'll definitely see them in the future in military, right? So for military applications, they'll definitely use these robots.
OK, so coming to some of our basics, some of our fundamentals, right? So of course these are things that we have learned, but I'll just run through some of the basics right now if you take coordinate systems.
Now if you have a graph paper, you have an XY coordinate system, right? So you have an X axis like this, AY axis like this, right? A typical graph paper you have two axis. Now if you add a third axis here.
Then this becomes your Z axis. So in this case, this coordinate system is called a Cartesian coordinate system, right? So that means if you have to plot a point here in this Cartesian coordinate system, you have to specify XY and Z coordinates, right?
So if you specify XY and Z, you'll be able to plot a point in the Cartesian coordinate system, right? OK, so next one. You know, before even going to cylindrical, now we are familiar with what is called a polar coordinate system, right?
If you take A2 dimensional space and if you have to plot a point using the distance.
And the angle.
Right. So that means if I have to plot a point here if I know the distance from the origin.
R and if I know this angle Theta then I'll be able to plot this point in two dimensions, right? So this is called a polar coordinate system. In 2 dimensions this is a polar coordinate system. Now I can make a three-dimensional coordinate system using this polar coordinate system. Now I can have.
Of a reference axis like this.
Now if I plot a point, I can specify R Sorry, I can specify this distance R and I can specify this distance Theta. Now if I add a third axis to it, if I add
A third axis to it, which I can call as H or Z, whatever I want, right? So that means I'll be able to plot a point along the polar coordinate system, and if I add a vertical axis to this, then it becomes a cylindrical coordinate system.
In dimensions it is polar. In three dimensions it becomes cylindrical, right? So this becomes your cylindrical coordinate system. So that means if you have to specify a point on the cylindrical coordinate system, you have to specify R, Theta and your height.
Z.
So if you specify these three, you can plot a point anywhere within this cylinder, right? So the same polar coordinate system can be used to create a three-dimensional sphere, right? For example, I have a polar coordinate system.
I have.
A point here. Now I can specify RI can specify Theta. Now instead of secifying the height, I can rotate this about an angle here. So I can rotate this by another angle.
So I can rotate this by another angle.
And this angle I can give the name Y right? O then so I'll be able to move along.
This sherical space, right? So I'll be able to move along this axis. When I change data, I'll be able to move along.
This circle when I change Phi and I'll be able to move in and out if I change R right? So this becomes a spherical coordinate system. So if you have to plot a point, you have to specify R, Theta and Phi right?
So by choosing a particular robot design or arm design, depending on how your joints are positioned, depending on how this arm is going to move, you will use any of these coordinate systems. You will you might actually use a Cartesian coordinate system.
A cylindrical coordinate system or a spherical coordinate system or combinations of all these things, right? OK, so any any questions on coordinate systems?
Now we have used a Cartesian coordinate system and polar coordinate system right from school, right? Right from 8th standard, probably 11th and 12th standard. We started using a polar coordinate system. Yeah, it's clear, right?
See, now we can combine multiple coordinate systems again, you know.
We can. Of course, you'll understand it better when when we discuss ARM configurations.
So as of now, we'll stick to individual coordinate systems. Now we'll see combinations or, you know, the actual work envelopes when we discuss ARM configurations. OK, now coming to robot anatomy, right? So if you have an ARM.
How we can label each arm or how we can label each part. So if you look at an arm, any particular mechanism or any particular device will have.
Links and joints. Now if you have a flexible device, then the device will have links and the joints will be connecting multiple links. So that's how a typical mechanism is constructed, right? So in this particular case of.
A human arm. Now, for example, if you take a human arm, this has got a shoulder joint. It has got an elbow joint. Of course it has got a wrist, right? In between we have bones which function as links, right? In some cases we have two bones, in some cases we have just one bone.
Between the joints right O they act as links and deending on how the joints or links are designed, you will get different motion.
You'll get different movements and you'll be able to operate this particular mechanism. Now the study of the physical construction of a robot is called robot anatomy. So whenever you use the term anatomy, it deals with physical construction. So when you want a movement between the links.
Right then you have to have a joint between the links. So I have links here which are marked in red. So if I want some movement then I need to connect the links using joints, then I'll be able to get movement right?
So a joint provides relative motion between the links. So that's what is a joint. So that's the purpose of a joint. It's going to provide relative motion between the links. Now in the case of a human arm, you have a shoulder, you have an elbow, and you have a wrist. Now here you have.
Uh.
A simple robotic arm. Now the base is actually called the shoulder and here the in between joint you can call that as an elbow and whatever is close to the end, that's called a wrist, right?
Now the arm is actually called a manipulator. This part of the arm, right? So something that looks like a human arm. So that part, the yellow part in this particular case, this is a manipulator. The technical term for the arm is a manipulator.
Now.
This part of the human arm see now here.
This is what we call a hand, right? We have, you know, a palm and we have 5 fingers and that's a hand. So in the case of a robot, this particular device is called the end effector. So the equivalent of a hand is an end effector.
Now in our case, we have, you know, an arm, right? So we have a forearm, we have an upper arm. Put together we have an arm. So that is called a manipulator in robotic storms, and the equivalent of a hand is called the end effector.
Right, so the end effector again can be a gripping device. In this particular case, what you see here is a gripping device. Now, instead of a gripping device, you might actually see a magnetic device. You might actually see a pneumatic device. You might see a simple hook.
You might see you know a welding tool, right? You might see different tools, a spray painting tool. So all these things will be your end effector. Your tool will actually be your end effector in this particular case, right? So depending on your application.
You can have a gripping device or you can have any particular tool, right? OK, so this is your robot anatomy. So here we are just introducing the terms. I'm just introducing the basic terms used in robotics, right? So you have to keep this in mind because we'll be using these terms very often, end effector, manipulator.
Right. So these are the basic terms that you have to understand.
Point.
OK.
Now Mechanisms now trying to understand mechanisms.
OK, so when you take a mechanism, A mechanism will have links and joints connected to each other. But just because you connect a link and a joint, you cannot call it a mechanism. Now for example I can take.
A link like this, I can connect it to another link, right? O I'll just go through some fundamentals here.
I can take one link like this. Now a link when I mean it is a rigid object, right? So now again, link can be a rigid link or it can be a flexible link or it can be a fluid link, right? But here I'm just talking only about rigid links, right? Something that is very stiff.
Solid that is called a rigid link. When you have something flexible like a rope, that's called a flexible link. Now if you have a fluid filling a tube, then that is called a fluid link. You can actually transmit a force through this fluid, right? So now in a hydraulic system you will have oil in a tube.
So you will be transmitting a force through this oil, right? So that is a fluid link in that case. So here in the case of robots, we are talking only about a rigid link. Now here we have two links connected by a joint.
So here in this case, if I hold this link, for example I'm just holding this, then I can move this right? So there is relative motion between the two links, so I can similarly hold this particular link.
So when I use this symbol, that means you know that it's stationary. Now I can move this, right? So that is relative motion. So in this case, this is called a kinematic pair, right? So you have relative motion, you have two links connected by a joint, and there is relative motion.
Between the links, then this is a kinematic pair, right?
OK.
Now if I have a link, then I have a joint, then I have another link.
I could put another join here. I could still connect another link. I can just go on like this, right? I can just, you know, go on and on, connect more and more links and join them using joins, right? If I if I do this, then this is called a kinematic chain.
So now in this case, each one can move with respect to the other one. If I hold this one, I can still move this, right? I can move this. If I hold one, I can move the other right? So you have relative motion between the links then.
This is called a kinematic chain. Now if I have something like this, now for example I can have a.
A mechanism now uh.
Or I have an arrangement where the last link is attached to the first link, right? So in this case this is a closed kinematic chain, right? So now I have.
So now I have links connected together using joins, but the last link is joined to the first link so that I'm forming a loop right now. This is a closed kinematic chain. Now what I can do is I can fix.
Or ground one link so I can just ground this. I can just fix this and if I turn this.
If I turn this, then this will definitely move, right? When I move one link, the other one will move, but in a different way. In the sense I could move this faster and this will move slowly, right? So depending on how much angle it is turned so.
But there'll still be movement, right? So in this particular case, if I fix one link of this particular close kinematic chain, then this becomes a mechanism, right? So now in this particular case, there is relative movement.
And one link is grounded, then this becomes a mechanism. Now if I attach a powering device or if I attach an actuator right, in this case if I attach a motor to this particular joint or.
You know something to turn this right? Then the moment it is powered, it becomes a machine, right? So you have a kinematic pair, a kinematic chain you can have. In this case, this kinematic chain is open.
In this case, this is a closed kinematic chain.
Now when I ground one link of this closed kinematic chain, it becomes a mechanism. When I power this mechanism, it becomes a machine, right? So this is something, you know, mechanical engineers must have learned in their second year of kinematics.
So I'm just repeating that now for further reading. You can actually take any book on kinematics of machinery or theory of machines. You can just read some of the basics first few chapters and you can try to understand this right? So just.
Links and joints, how to create a mechanism and how to create a machine. All right, So the moment you power a mechanism, it becomes a machine. OK, so that's what we are discussing here. Mechanisms consist of connected parts with the objective of transferring motion or force.
From a power source to an output, right? So in the case of many robotic arms, they follow this open kinematic chain. They have an open kinematic scene. That means I can ground.
One link and it is everything else will move. Then this is an open kinematic chain.
OK, so one part is designated the frame, right? So now one part is called the frame. So now whatever is grounded, that is called the frame. So in this particular case, I'm grounding this, that is your frame. So here I'm just grounding this, that is your frame, right? So.
Right. So this is the part that exhibits no motion. So links are individual parts of the mechanism, right? They are connected using joints, right? OK.
Now we have degrees of freedom, so depending on what joint you're using, right? Of course here I just mentioned joint, right? I did not mention what joint type it is. So depending on what joint you're using, you're going to get.
A different kind of movement or a different flexibility that articular joint will offer O. Here you have the term.
Degrees of freedom to describe how a particular joint is going to work right. So now if you see degrees of freedom is often used to describe the number of directions that a robot can pivot or move a joint. Now here from the robotics perspective, they've mentioned what we mean by degrees of freedom.
Now the general way of understanding degrees of freedom is, you know, if I take a particular object. Now, for example, if I take an object.
Now I'm just taking a three-dimensional object. Now this object is floating in space. Now let's imagine this object is floating in space, right? There's no gravity. This object is floating in space.
So in this particular case, how many in how many ways this object can move, right? Any guesses how many ways this object can move?
OK, I'm getting a lot of answers. 666.
Yeah, OK, that's correct. So this object can move along the X axis.
Right.
So it can move along the X axis.
This is 1 movement. Now it can move forward and backward. So forward and backward is just considered as one movement because it is moving along a particular axis, right? So it is moving along the X axis. You get forward movement and you get backward movement. So that is counted as one.
Imilarly, it can move along the Y axis.
And forward and backward O. This is another one movement. Imilarly it can move along the axis uward and downward again.
Both directions, right? So this now here we have 3 movements, 3 translations. Now similarly we'll have three rotations. That means this object can rotate about the X axis. This object is floating in space. It can rotate about the X axis.
It can rotate about the Y axis. Now again clockwise and anticlockwise rotation. When I say rotation, that means both should be included as one rotation right forward. That is clockwise rotation, anticlockwise rotation that is considered as rotation about that particular axis. Similarly, this object can rotate about the Z axis.
Right. Clockwise rotation and anticlockwise rotation. So totally 3 translations.
Then three rotations, right? So put together you have 6 degrees of freedom. So for each translation about an axis, that's called 1 degree of freedom, one rotation about an axis, that's called 1 degree of freedom. So you have 3 translations and three rotations.
And you have 6 degrees of freedom. So this is what an object which is floating in space will have, right? So the moment you take this object and place it on a plane, right? I take this object now and I'm placing this object on a plane.
Now I'm just giving one restriction that this object should not be lifted from this plane.
Right, so this object I'm just taking it, which is floating in space. I'm just taking it from space. I'm placing it on my table. Now I'm just giving one restriction that it should not be lifted from the table. Now how many degrees of freedom does this object have?
Yes.
Yeah, I'm getting three, three, three. Any other answers?
Any other answers?
So only a few people have answered. OK, somebody's saying 4 now, so if I wait, I'll I'll get a higher number. So if I wait for some more time, I'll get a bigger number. Is that right?
So people are giving, you know, bigger numbers. Why?
Now I'm just giving that condition. The object should not be lifted off the table. It should not be lifted from the table. It should always maintain contact with the table. So that condition has to be satisfied, right?
Why so much confusion?
Again, 11 translation, 2 rotations. You know I'm getting very interesting answers here. OK, now this object can move along the X axis, right? X axis movement is possible both forward and backward, right?
Similarly, Y axis movement is possible.
Forward and backward right now Z axis movement is not possible. Z axis movement is not possible because I cannot lift it off the table. I cannot push it below the table because it is resting against it. So that means Z axis movement is not possible.
So no Z axis movement. Now I cannot rotate it about the X axis. This is not possible because if I try to rotate it about the X axis, I have to lift the object, right? So that means no rotation about the X axis, no rotation about the Y axis. If I have to rotate it about the Y axis, I have to lift the object.
From the table, so I cannot do that.
Now the other thing I can do is I can rotate it about the Z axis without lifting it, right? So Z rotation is possible, X translation is possible, Y translation is possible, Z rotation is possible. So the moment I place this object on my table.
And I put a restriction, I'm getting only three degrees of freedom, right? So now you understand what we mean by degrees of freedom, right?
OK, so yes, some of you are trying to understand it. OK, good. So now this is what we mean by degrees of freedom. Now let's look at some of the objects right now. This is the condition I mentioned right now. Whatever I discussed just now is this condition.
So planar joint. So in this case this object can move in One Direction, it can move in this direction, then it can rotate about the Z axis. So this will have 3 degrees of freedom, right?
Now let's take for example this particular arrangement. You have a square hole and you have a square rod going through that hole, right? So the hole is a machine.
And it is non circular, right? You can put it that way, square or rectangle, it doesn't matter. And the object fits that hole and there is movement. Now how many degrees of freedom does it have?
In this articular case, now the box, yes.
And I see 111.
OK, I see a lot of ones here. I'm not seeing any other number. Come on, make it interesting. Yeah. Any other answers?
OK, good. So this has got one degree of freedom. So that means this rotation is not possible, right? The only movement that you can get is you can move along this axis, right? Whether you call it X or Y, it doesn't matter. So forward movement.
And backward movement along that axis is possible. Any other rotations, you know this rotation is not possible, this rotation is not possible, this rotation is not possible and sideways movement is also not possible. So this has got only one degree of freedom. So this joint can be called a prismatic joint, right? A linear joint is called a prismatic joint.
OK, now let's take this one, the next one.
Cylindrical. Now I have a circular hole and I have a circular rod going through it. How many degrees of freedom do I have?
OK, I see 222. OK, any other answers?
OK, I see a lot of twos here.
Yeah, I'll I'll come to that. I'll come to that. Any other, any other answers?
So whenever you get this linear motion, that's called a prismatic joint. Not necessarily one degree of freedom, whenever you have a linear motion, but normally we restrict it to 1 degree of freedom. So that's a prismatic joint. I'll I'll when we discuss the different joint types, you'll actually see.
OK, so now in this particular case, you can move this forward and backward about along this axis. You can move this rod forward and backward. So that's one movement. That's one degree of freedom. The other thing you can do is, since the hole is circular and the object is also circular, you can rotate it.
You can rotate it about this axis, the same axis, right? If it is X axis, then you have X translation and X rotation. So now here you have two degrees of freedom. Now when you have a hydraulic cylinder or a pneumatic cylinder, normally what we do is even if you have a circular rod, we'll try to put a restrict.
Restriction on it and so that we avoid this rotation. So if you want to restrict this rotation, see circular rod is easy to manufacture. From the manufacturing perspective, it's easy to manufacture. That's why you have circular rods everywhere. But if you think it's going to give you an additional movement and create confusion, you have to restrict it. You could put a key wave.
Somewhere. Or you could just put any kind of projection somewhere and you can restrict that rotation and you can use it as a one degree of freedom joint. That's possible. This typical the square hole and square rod that's very difficult to manufacture in terms of.
Manufacturing. It is fairly a complex process, right? At least more expensive, time consuming process. Not that complex, but yeah, expensive and time consuming. In terms of creating a circular hole, all you have to do is simply drill it, right? Drill it, ream it and turn this rod and then use it.
So in that case, cylindrical joint will give you 2 degrees of freedom. So if you don't want the additional rotational degree of freedom, you have to put some restriction and use it as a one degree of freedom joint, right? OK, now we'll come to the next one.
OK, now what is this screw joint? How many degrees of freedom do we have?
OK, I get answers like 1-2. Any other answer?
I see a lot of twos here.
I saw only 111. OK, I see too many twos here.
OK, now coming to a screw joint. See now when you turn the screw, when you turn the screw, the screw moves forward.
Right. If you want to move the screw backwards, then you have to turn it again in the opposite direction, right? But if you want to move it in a linearly, if you want to make a straightforward movement, then it's not possible without turning it, right? So that means you have even though it appears that.
You have two degrees of freedom. Both are dependent on each other. So one degree of freedom is dependent on the other. Now, without using the rotation, you cannot get translation. Without using translation, you cannot get rotation.
Right. So that means you have degrees of freedom dependent on each other. So now that is not that's considered a foul, right? So that's not considered an independent degree of freedom. So that means when you have a degree of freedom, that degree of freedom has to be independent.
From the others. So here, even though it appears to be two, you have only one degree of freedom.
So that means if you want a linear movement, you can use the screw to produce a linear movement. Or if you want rotation, then yes, you have to rotate it directly, but you cannot get one without the other, right? So that means this is considered as one degree of freedom. So normally if you want a linear movement, right?
Like in many of the mechanisms that we see in machine tools, right? For example, you have a lead screw on a lathe where you use a screw mechanism to move the carriage, right? Similarly, now you can have a screw like this.
Right now, of course, there are people here who don't have a mechanical background, so I have to explain this. So those who already understand, you can just, you know, bear with me a little bit. OK, now here I have a screw and a nut.
Right. So that means the nut is mounted on the screw. Now if I want to get linear movement, what I should do right? I have to first of all restrain the screw, so I have to.
Prevent the movement of the screw in the linear direction. That means I have to put bearings here. So I'm putting the screw on bearings, right? So that means I'll prevent the movement of the screw in any direction other than a rotation.
So now I have to guide the nut.
So that means I'll put some kind of guide here so that the nut doesn't rotate right? So now if I turn the screw.
If I turn the screw, what will happen to the nut, right? Depending on how the threads are made, if I rotate the screw, the nut will move either forward or it will move backward.
Right. So if you want to get a linear movement, then you can use the screw and nut arrangement and you restrain the screw and you turn it. So that means you can connect this to a motor. You can connect this to a motor. You can connect the screw to a motor. Simply drive the motor this.
This nut will move forward or backward so that you can get a linear movement. So if you want to get a linear movement using a lead screw mechanism. So this is how you do it, right? So that's where this mechanism can be used, right? So.
Yeah, of course you can modify this mechanism. Now you can introduce balls between the screw and the nut to minimize friction. All these modifications are possible, but the fundamental mechanism is just this, right? OK, now here you have one more joint. Now let's go to the next one.
So here you have a revolute joint, so that means you have a.
One arrangement like this and another one like this. These two are connected using a pin. So that means here you have one link, you have another link. These two are connected using a pin and what you have done here is you have made.
See now if I draw the other view of this particular arrangement, it can be something like this. Two plates joined using a pin and the pin will have heads on both sides. So that means this axial movement is arrested, right? So I can.
You get this rotational movement, but any other movement is arrested, right? So that's the arrangement that you have here. That's a revolute joint right? Now, how many degrees of freedom does this have?
A revolute joint. OK, since we have arrested the axial movement using the pinheads here, right? So no axial movement is allowed. Only rotational motion is allowed. So that is a revolute joint, right? So that's a revolute joint.
So it's a very common joint that we see pretty much everywhere, right? In robots or in any kind of linkages, even your folding chair will have revolute joints, right? So it's a common joint that you see everywhere. OK, now the last one.
Uh.
A spherical joint. How many degrees of freedom does this have?
Yes, spherical joint. I see three.
OK, I see all threes. Any other number?
Any other number?
OK, now now this is the joint that we have on our shoulder, right? So here when you say spherical joint.
OK, so uh.
How long did I disappear?
Now can you see my screen now?
Yeah, the window went missing.
5 minutes. OK, sorry.
So you didn't see me for 5 minutes. So now I was getting answers for spherical joints, right? OK, so you were following till spherical joint. OK.
OK, shoulder analge. OK, so not many minutes. OK, don't worry about it.
OK, OK, now this is something that we have in our shoulder, right? So now if you take your shoulder joint, now I can, I can do this right? So I can make this movement. I can move my shoulder up and down, right? So that means I'm rotating my shoulder.
About the horizontal axis, right? So I'm just doing this. This is 1 movement. Now if I try, if I take this as a vertical axis, then I can move my shoulder like this, right? So I'm able to move my shoulder about the vertical axis. I'm able to move my shoulder about the horizontal axis.
Then I can do the third movement. Say I can do this. I can rotate my shoulder about the axis of this particular arm, right? So I can just do this. So one right two. Then I have 3.
Right, so I have three rotations, but translation is not possible.
OK, so you have three rotations.
Rotation about.
OK, so in this particular case I can rotate about a horizontal axis, right? I can rotate about a vertical axis.
I can rotate about the axis of the link, right? So I have 3 rotations, but I'm not able to get any translation, so translations are not possible. So this is your spherical joint. So the the joint that is familiar to us is the ball and socket joint, right?
OK, just a second.
OK.
OK, so these are some of the examples of the joints and their degrees of freedom, right? So how you can visualize the joints and the degrees of freedom? Of course you can have modified versions of all these joints, but these are the basic types, right?
Understand these joins and their degrees of freedom. Then that should be sufficient for this course, right? So anything beyond this, yes, things are possible, but those will be modifications of these joins, right?
Joystick. Now you can have a joystick with two degrees of freedom. You can have a joystick with three degrees of freedom also. So you can actually make a joystick which is usual. Joystick will have only two degrees of freedom. You don't need that rotational degree of freedom in a joystick.
But there were devices where you'll have a ball rotating, right? So in that case you can actually have 3 degrees of freedom, right? So very similar to a mouse where you have a ball and you can just turn that ball and use that as a.
You know, more like a mouse pad. So all those devices were there. But yeah, depending on how it works, you can actually have two degrees of freedom or three degrees of freedom.
OK, so now most of the time we'll actually use a revolute joint and a prismatic joint. So in this particular prismatic joint, we'll restrict the degree of freedom to just one, right? So using this we'll have a joint notation scheme right now, basic notation.
For revolute joint it is R. For a prismatic joint or a linear joint, the other name for it is linear joint. You will have L right now. This revolute joint can be further classified into different types, right?
So let's see now. A typical pin joint or a revolute joint is going to look like this, right? So you have two links joined with the help of a pin, right? So this is the arrangement that you see in your folding chair.
In some cases you will you will actually see this arrangement, right? A prismatic joint can be like this also. A prismatic joint can be like a slider in a slot, right?
Or in some cases, it can actually be something like your motorcycle shock absorber, right?
Something like this is possible, right?
And in some cases, yes, yeah, front suspension, rear suspension, you'll have, you know, a rod moving in a cylinder. So the degrees of the rotational degree of freedom will be restricted. So you have you, you can actually call it as one degree of freedom system. So just a sliding joint or a prismatic joint.
Pin joint or a W joint, right? So these are the basic joint types. Then beyond that you can further classify them and you can represent them using joint notation schemes right now.
Since we're almost at the end, so I'll just stop here. Remaining parts for the classification we'll discuss in the next class, right. So any questions till now, whatever we discussed now those with non mechanical backgrounds.
Now, were you able to understand?
Oh, you think you need more reading? Yes.
So, so far we have discussed only some of the basics, right? Any questions till now? Anybody then, especially those with non mechanical backgrounds?
OK, you've understood as of now. Yes, it's better to do some reading, but beyond that, I'll explain all the fundamental concepts also before discussing, you know, other concepts. So you need not worry, but you start reading a little bit.
Some basics of kinematics, some basics of mechanisms, right? Yeah. Any other questions?
In kinematics, we'll we're not going to. This is not a kinematics course. We'll we'll go through some of the basic mechanisms needed for building robots. Then after that we'll just move forward in in robotics topic.
Yeah, yeah, PowerPoint slides will be uploaded in Microsoft Teams. Now you you know where to access them, right? So.
Textbooks. Textbook 2 is available. You can simply buy it online, right? Or at your nearest bookstore.
Right. You can just buy it at a regular bookstore or online textbook one. As as of now, I heard that it is very difficult to get it, but if you can get it, it's fine also.
Right. Yeah. Any other questions? No, no doubts on any of the topics. Everything is clear.
OK, OK, then we'll stop here. See you in the next class. Have a good night.
Yes, Niku is good.

GLYNN JOHN . stopped transcription