- Explain Ship Navigation with a necessary block diagram. Also list the critical and non critical tasks. Provide whether open loop or closed loop system is used and justify your answer

Answer (short explanation + block diagram + tasks + closed/open loop): Overview & block diagram (conceptual): A ship navigation control system is a closed-loop feedback control system that uses sensors (GPS, compass, gyro, AIS, radar), a navigation/planning module, a controller (autopilot), and actuators (rudder, thrusters, engine throttle). Data from sensors → fused in Navigation system → desired heading/speed computed by Planner/Guidance → Autopilot (controller) computes control commands → Actuators change ship motion → Motion measured by sensors → feedback to Navigation. [Sensor Suite] (GPS, gyro, compass, AIS, radar, speed log) ↓ [Sensor Fusion / Navigation] ↓ desired course & speed [Guidance / Path Planner] -----> setpoint (course, speed) ↓ [Autopilot / Controller] <---- measured state (heading, yaw, speed) ↓ control commands [Actuators] (rudder, thrusters, engine) ↓ Ship motion (→ measured by sensors) (feedback loop closed) Critical tasks (must meet deadlines / safety-critical): - Collision avoidance & obstacle detection (radar/AIS processing + emergency maneuvers) - Rudder/thrust command generation & autopilot control loop (tight timing for stability) - Engine/thrust control for propulsion/speed regulation - Sensor sampling & sensor-fusion for immediate state estimate (heading, yaw, speed) - Emergency alarms & bridge alerts (real-time alerts to crew) Non-critical tasks (can be lower priority or deferred): - Voyage logging and telemetry uploads - Crew notifications that aren’t emergency - Route planning UI updates, weather-forecast downloads - Entertainment / non-safety communications Open loop vs closed loop: The ship navigation system is a closed-loop system. Justification: the controller (autopilot) uses measured variables (compass/gyro/GPS-derived heading and speed) to compute corrective commands to the actuators (rudder/thrusters). Without feedback, disturbances (wind, currents) would cause drift — closed-loop corrects for those disturbances in real time.

- What is the makespan (maximum flow) and utilization of the following set of 4 tasks T(e,p): T1(2,6),T2( (3,12) T3(1,24) and T4(3,5). Also determine hyperperiod assuming period = deadline time.  Also determine whether they can be rate monotonically scheduled

Answer :

Makespan, utilization, hyperperiod, RM schedulability

Tasks: T1( e=2, p=6 ), T2( e=3, p=12 ), T3( e=1, p=24 ), T4( e=3, p=5 ). (Assume period = deadline.)

Step 1 — Hyperperiod (LCM of periods 6,12,24,5):

- lcm(6,12) = 12

- lcm(12,24) = 24

- lcm(24,5) = 120
⇒ Hyperperiod = 120 time units.

Step 2 — Number of job instances in one hyperperiod and total execution (makespan / total CPU demand):

- T1: 120/6 = 20 instances ⇒ execution = 20 × 2 = 40

- T2: 120/12 = 10 instances ⇒ execution = 10 × 3 = 30

- T3: 120/24 = 5 instances ⇒ execution = 5 × 1 = 5

- T4: 120/5 = 24 instances ⇒ execution = 24 × 3 = 72

Total execution demand in hyperperiod = 40 + 30 + 5 + 72 = 147.

(Interpretation: the total CPU time required by all jobs in one hyperperiod is 147 units.)
This total-execution-over-hyperperiod is sometimes called the total demand (makespan over that interval).

Step 3 — Utilization U (sum e_i / p_i):
Compute per-task utilizations and sum:

- U1 = 2/6 = 0.333333...

- U2 = 3/12 = 0.25

- U3 = 1/24 ≈ 0.0416667

- U4 = 3/5 = 0.6

Sum: U = 0.333333 + 0.25 + 0.0416667 + 0.6 = 1.225 (exactly 147/120).

Conclusion about feasibility / RM schedulability:

- U = 1.225 > 1.0 → Total required CPU > 100% ⇒ impossible to schedule on a single processor (misses deadlines).

- Liu & Layland RM bound for n=4: ULL=n(21/n−1)=4(20.25−1)≈0.7568U_{LL} = n(2^{1/n}-1) = 4(2^{0.25}-1) \approx 0.7568ULL​=n(21/n−1)=4(20.25−1)≈0.7568. Since U > 0.7568, even the sufficient RM bound is violated. But the primary blocker is U>1 which immediately rules out schedulability.
Therefore: They cannot be Rate Monotonically scheduled on a single processor.

- What are controlled and controlling systems. Explain a Radar System to explain the same

Answer:

Controlled vs Controlling systems; Radar system example

Definitions:

- Controlled system (plant): The physical system whose behavior we want to regulate (e.g., an aircraft, a ship, a motor). It responds to control inputs and disturbances.

- Controlling system (controller): The algorithm/hardware that measures the plant state, computes control actions, and sends commands to the plant to drive it toward desired behavior.

Radar system example (to illustrate):
Consider an automatic tracking and fire-control system (simplified):

- Plant (controlled): The target relative motion or the turret/gun system that must be pointed. If we focus on tracking, the "plant" is the turret pointing mechanism (or the target track state — the thing whose angle we want to make match the setpoint).

- Sensors: Radar provides measurements of bearing, range, elevation (noisy, sampled).

- Controlling system: The radar signal processor + tracking filter + controller. The tracker (e.g., Kalman filter) estimates target state from radar returns; the controller (servo controller) computes actuator commands (pan/tilt) to keep aim on target.

- Feedback loop: Radar measures aim error (measured bearing vs desired), controller computes corrections, actuators move turret, radar measures new error → loop closes.

Why this clarifies controlled vs controlling:

- Radar itself provides measurement (sensor) and signal processing (part of the controlling system). The turret (or the target’s position being compensated) is the plant (controlled system). The controller uses radar measurements to minimize pointing/tracking error.

- What are Sporadic Jobs ? Explain . Also list the major differences between sporadic and aperiodic jobs with special reference to acceptance test and also provide at least 2 examples for each

Answer:

Sporadic jobs, differences from aperiodic jobs, acceptance tests, examples

Definition — Sporadic jobs:
A sporadic job model describes tasks that arrive irregularly but with a known minimum inter-arrival time (a guaranteed separation). Each job has a worst-case execution time and a minimum interarrival time (or rate). They are like periodic tasks but with nondeterministic phase; they can be analyzed by assuming the minimum interarrival as a period.

Definition — Aperiodic jobs:
Aperiodic jobs arrive irregularly with no guaranteed minimum interarrival time (i.e., they can arrive arbitrarily close). They do not have a known minimum separation; they are usually handled using special servers or background processing.

Major differences (with focus on acceptance test):

- Arrival model / boundedness:

  - Sporadic: bounded by minimum interarrival time → can upper-bound worst-case arrival rate.

  - Aperiodic: unbounded arrivals (no minimum gap), cannot guarantee arrival rate.

- Acceptance test & schedulability:

  - Sporadic: can use periodic task/arrival analysis. Acceptance test: check whether adding the sporadic task (modeled as periodic with period = minimum interarrival) keeps total utilization ≤ schedulable bound (or perform response time analysis). Because minimum separation is known, exact schedulability analysis (e.g., response-time or utilization test) is possible.

  - Aperiodic: cannot directly be accepted by hard real-time schedulability tests; instead use servers (e.g., Polling Server, Deferrable Server, Sporadic Server) to allocate budget for aperiodic work. Acceptance is performed on the server configuration (server capacity and replenishment period) not on each aperiodic job.

- Handling in scheduling:

  - Sporadic: scheduled like periodic tasks (or converted to periodic equivalents).

  - Aperiodic: scheduled within slack or using servers; may only receive soft real-time guarantees (or bounded response via server).

- Examples:

  - Sporadic job examples (minimum separation exists):

    - Emergency stop button triggers on a machine but with mechanical debounce ensuring at least 200 ms between distinct presses in practice (treated as sporadic with min interarrival).

    - Crash-detection interrupt from a sensor that by physics cannot occur more often than some min interval (e.g., certain vibration sensors).

  - Aperiodic job examples (no guaranteed minimum gap):

    - User requests (HTTP requests, UI events) — can come arbitrarily close.

    - Logging or analytics events produced by many sources — bursty and unbounded.

(Also note: examples depend on system specifics; the classification rests on whether a known minimum interarrival can be guaranteed.)

- Explain the concept of preemptive and non-preemptive scheduling using Shortest remaining time first and FCFS algorithms respectively for set of 4 tasks with release time and execution times as provided Job; Release Time; Execution Time. J1; 3; 2. J2; 2; 4. J3; 0; 6. J4; 1; 4.

Answer:

Preemptive SRTF vs Non-preemptive FCFS schedules for given jobs

Jobs:
J1: release 3, exec 2
J2: release 2, exec 4
J3: release 0, exec 6
J4: release 1, exec 4

We produce timing diagrams (Gantt charts) and compute start/finish/wait/turnaround.

(A) Preemptive — Shortest Remaining Time First (SRTF)

Algorithm: at any time choose job with smallest remaining execution among arrived jobs; preempt when a newly arrived job has strictly smaller remaining time. If equal remaining time, continue current job (tie-break: continue running job).

Simulate step by step (careful arithmetic):

- t=0: arrived {J3(6)} → run J3 from 0→1. J3 remaining after t=1: 5.

- t=1: J4 arrives (4). Compare J3 remaining 5 vs J4 4 → switch to J4. Run J4 from 1→3 (we will check arrivals at t=2,3).

  - At t=2: J2 arrives (4). At t=2 J4 remaining = 3 (ran 1→2). Compare 3 vs J2 4 → continue J4.

  - At t=3: J1 arrives (2). At t=3 J4 remaining = 2. Compare remaining: J1=2, J4=2 → tie; we continue current job (J4).

- Continue J4 from t=3→5 → J4 completes at t=5.

Now arrived but unfinished: J3 (rem=5), J2 (rem=4), J1 (rem=2). Choose shortest remaining next → J1 (2) is shortest.

- Run J1 from t=5→7 → J1 completes at t=7.

- Next choose J2 (4) vs J3 (5) → J2 shorter.

- Run J2 from t=7→11 → J2 completes at t=11.

- Finally run remaining J3 from t=11→16 → J3 completes at t=16.

SRTF schedule (Gantt, time intervals):

t=0 ──1: J3

t=1 ──5: J4

t=5 ──7: J1

t=7 ──11: J2

t=11 ──16: J3

Completion / turnaround / waiting:

- J1: arrival 3, start 5, finish 7

  - Turnaround = finish − arrival = 7 − 3 = 4

  - Waiting = start − arrival = 5 − 3 = 2

- J2: arrival 2, start 7, finish 11

  - Turnaround = 11 − 2 = 9

  - Waiting = 7 − 2 = 5

- J3: arrival 0, start at 0 (ran briefly), final finish 16

  - Total execution = 6 (1 at 0–1, 5 at 11–16)

  - Turnaround = 16 − 0 = 16

  - Waiting = turnaround − execution = 16 − 6 = 10

- J4: arrival 1, start 1, finish 5

  - Turnaround = 5 − 1 = 4

  - Waiting = 0

(You can verify total CPU busy time 16 units: 0→16 no idle.)

(B) Non-preemptive — FCFS (First Come First Served)

Order by arrival time (ties by lower job id): arrival order: J3 (0), J4 (1), J2 (2), J1 (3). Non-preemptive means each job runs to completion once started.

Schedule:

- J3 runs 0 → 6 (exec 6), finishes at t=6

- J4 runs 6 → 10 (exec 4), finishes at t=10

- J2 runs 10 → 14 (exec 4), finishes at t=14

- J1 runs 14 → 16 (exec 2), finishes at t=16

FCFS Gantt:

t=0 ──6: J3

t=6 ──10: J4

t=10 ──14: J2

t=14 ──16: J1

Completion / turnaround / waiting:

- J3: arrival 0, finish 6 → turnaround 6, waiting 0

- J4: arrival 1, start 6, finish10 → turnaround 9, waiting 5

- J2: arrival 2, start 10, finish14 → turnaround 12, waiting 8

- J1: arrival 3, start 14, finish16 → turnaround 13, waiting 11

- Find the effective release  time of each of the jobs if the release time,  and execution time are given

It is mandatory to consider the execution time while calculating effective release times

Explain the procedure in detail

| Job | Release Time | Execution Time |
| --- | --- | --- |
| J1 | 3 | 2 |
| J2 | 2 | 4 |
| J3 | 0 | 6 |
| J4 | 1 | 4 |

Answer

Effective release time (with execution times considered) — procedure + compute for the same job set

Interpretation & definition used here (practical, typical classroom interpretation):

Effective release time (ERT) of a job = the earliest time that job can actually start execution on the processor assuming a non-preemptive FCFS handling of jobs already released earlier. In other words, you must account for earlier jobs’ execution that will occupy the CPU before this job can begin. If the CPU would otherwise be idle at the job arrival, ERT = arrival; if earlier arrivals occupy the CPU, ERT = completion time of those earlier jobs.

(This definition matches the instruction “It is mandatory to consider the execution time while calculating effective release times.”)

Procedure (step-by-step):

- Sort jobs in non-decreasing order of their release times.

- Maintain a running variable current_time which is CPU free time (initially 0).

- For each job in arrival order:

  - If current_time ≤ job.release, then there is idle time: set ERT = job.release and set current_time = job.release + job.exec.

  - Else (current_time > job.release): earlier jobs occupy CPU, so set ERT = current_time and update current_time = current_time + job.exec.

- Continue for all jobs — the ERT values produced are the earliest possible start times assuming the earlier arrivals execute before.

Apply to given jobs (same set):
Jobs sorted by release: J3(r=0,e=6), J4(r=1,e=4), J2(r=2,e=4), J1(r=3,e=2).

- Initialization: current_time = 0.

- J3: release 0. current_time (0) ≤ 0 ⇒ ERT(J3) = 0. Update current_time = 0 + 6 = 6.

- J4: release 1. current_time (6) > 1 ⇒ CPU busy; ERT(J4) = current_time = 6. Update current_time = 6 + 4 = 10.

- J2: release 2. current_time (10) > 2 ⇒ ERT(J2) = 10. Update current_time = 10 + 4 = 14.

- J1: release 3. current_time (14) > 3 ⇒ ERT(J1) = 14. Update current_time = 14 + 2 = 16.

Final effective release times (startable times):

- J1: ERT = 14

- J2: ERT = 10

- J3: ERT = 0

- J4: ERT = 6