Real Time Operating Systems (Merged - ESZG524MELZG524)(S1-25)-20250824_082025-Meeting Recording

August 24, 2025, 2:50AM

2h 16m 40s

Nilanjan Mukherjee . started transcription

Nilanjan Mukherjee .   0:49
OK.
OK, so basically in today's lecture we'll be starting about how to model a real time system, followed by analysis of scheduling tasks and nature and task nature study.
well So, these are the agenda for today's lecture.
So we started with the.
So we started with the idea of modeling real time systems in the last class, right? So we'll take it forward from there on, right?
The question now is that why actually need to? Why actually do we need to model real time systems? Why? What is the usage? What is the motivation and the objective of modeling the real time systems, right? So therefore it can be stated that.
The the modeling is basically.
Is the task right or the art of representing a real time system abstractly before deploying it right?
Now, what do you mean by representing a real time system? I'm struggling. It means that you basically have to find out some kind of mathematical equation, some kind of mathematical formulations, right? That would represent the the relationship, right? That would represent the functional relationship between the.
Different parameters and the different components of the real time systems, right? So once we are able to model those relationships between the different components, between the different parameters of a system, right? So then it becomes quite easy.
To model those relationships, right? And if you are able to model those relationship by using some kind of mathematical formulation, some kind of mathematical equations, then we can easily deploy it right in the in actual real time cases, right? Not in simulation environments, right?
So let's take an example. So for example in the Delhi Metro that what are the different types of parameters right that that that basically do we need to consider that the parameters are the the different type of sensors, the different components. The parameters are there are different sensors which are used to feed data to the.
The door status, the breaking distance and the signals of the track right now, these parameters right are all temporal in nature. Means they're all bounded by the time they're all bounded by the timing constraints, right? So if the breaking action delays by even a fraction of the.
2nd right, what will happen? A fatal consequences will be will be will be encountered. So that is why it's very important is very necessary to model the timing constraints right in order to ensure that the safety is maintained.
OK, another example is the Indian Railways automatic train protection systems. Right, so now this automatic train protection system depends heavily on the real time guarantees. Means the properties of the real time systems right that we have already discussed. The most important among them being the timing constraint, right.
So and what do and what is the exactly the timing constant, the deadline factor, right? So if the braking system doesn't exactly respond within a particular defined deadline, the train will overshoot a signal, right? And it's quite unacceptable, particularly in local trains, right? Like in Mumbai corridors, the Mumbai local trains.
Right, which are very much high. The high density corridors, right? So therefore it's very much important to model the timing constraints, the temporal factors, right, that exist right between the different.
That that exists with respect to the different parameters, the different components of the real time system. And once we are able to model that, the timing constraints right apart along with the other different parameters, we will be able to design a very much predictable and optimised.
Real time systems, so therefore the major purpose, the major goal of modelling of real time system is that to understand how much predictable the system is right, that is, it will ensure us that whether a system will be.
Responding within the deadline, right, whether it is a hard deadline, whether it's a soft deadline or whether it is a hybrid of these two fact or these two kind of deadlines, right?
Another is that optimization, right? So what is optimization? The optimization is basically the balancing of resources against the different costs, right? So you all know that whenever we are try to model the relationship, the functional relationship between the different parameters, right? But taking into consideration the timing constraints, right?
So obviously in that case the timing becomes a cost function that needs to be optimised, right, right? Because we have to minimise the time within which a particular system will respond, right? So that means this timing factor, this timing constraint is one is 1 function.
That we need to minimise or that we need to optimise so these can therefore be considered as some kind of a cost function. Timeliness is basically the same.
Oh.
So therefore, the core purpose of the important purpose of modelling is therefore to ensure, as I said, #1 predictability, that is, we know that whether a particular system will respond right, that is obviously the important thing.
Another important factor is the optimization right which which we just discussed that whenever we are trying to model any any real time systems and we have to find out, we have to explicitly find out the relationship between the different parameters, right, different parameters of the different components of.
Real time systems, right? We need to satisfy some kind of constraints, right? So what are the constraints in most of the real time system, the constraints that we need to ensure is the timing constraints or the temporal constraint, right? So this is basically the cost function that we can ensure that we can consider so in most.
Of the optimization problem, we have basically have to achieve some objective while satisfying the constraint in this real time systems. Also the task is what the task is we have to achieve some predefined objective depending on one particular problem.
But one optimization constraint 1 cost function that remains constant is the timing constraint that we have to finish that particular task within a given time. For example, the signalling system of a railway right, that whatever may be the situation.
We the train should always obey the signal. The signal should always be generated within a particular instant of time, within a particular deadline, right? Because if the signal is not generated within the particular deadline, the train will miss it, right? And it will lead to some kind of fatal or dangerous consequences, right?
So we have to balance the resources against different types of cost, right? One of the important cost in this case is obviously the timing constraints. Fine. Next is what next is the timeliness, right. So what is timeliness now, timeliness is basically.
That, as I said, that every task has to be completed within the deadlines, right? It cannot exceed the deadline, otherwise it leads it will lead to the degradation of the overall performance of the system, right?
And and final one is that validation validation means that once we have modelled the relationship between the different parameters of a real time system between the different components of a real time system as well. By taking into consideration all the constraints right, particularly the temporal or the timing constraints, we have to make sure.
The relationship that we have formulated is perfect and in real time also the different components, the different parameters follow that same functional relationship and for that what do we have to do? We basically have to carry out extensive validation, extensive simulation, right of the model.
In order to ensure that the relationship that we have formulated are giving us the desired results right, it is not giving us even if even if it is giving us some erroneous result, even if if it is giving us some errors, we need to correct it, right? We need to modify those errors by.
Manipulating the relationship, manipulating the values of the different parameters right before it is been deployed in a real time actual real time situations, actual real time system, right? So that is basically the idea behind the modelling of real time systems, particularly the motivation behind modelling a real time.
Right that we basically have to explicitly find out the relationship right between the different components, different parameters of a system, right, satisfying some constraints. For example, the temporal constants or the timing constraints. Right? And then.
Deploying right those relationships in a a first we have to validate those relationships in a similar environment and once it is giving us the desired results, we basically can deploy that particular model in a real time situation or a real time environment in actual real time environment. OK.
Fine.
Now if I go to the purpose of modelling right, particularly taking some Indian context into account, right? So some of the some of the, some of the very classic applications of modelling, particularly in Indian applications are.
Air traffic control in Mumbai airport, right? But the planes are landing, which which is one of the busiest airport in the world as well, not only in India, right? So the planes are landing every three to five minutes.
So the planes are landing every three to five minutes, right? And there are different types of tasks, right? That needs to be done before a plane is allowed to land. For example, communication radar tracking, right communication with the ATC, Air Traffic control, right. Tracking of the radar of the plane.
You need to clear the runway also right. You need to clear the other planes. You need to clear the landing. You need to give the landing clearance to the plane right? So therefore there are different type of tasks right? That needs to be done before a particular plane is landing, right?
And all these tasks have to be done in such a manner that they meet the deadlines, right? Why? Because if a plane is about to land and it does not create the landing clearance or or is getting the landing clearance in a delayed fashion, then what will happen? It can it can lead to some kind of damage. It can lead to some kind of a problem.
So all these tasks has to be made in a strict deadline order, right? So therefore, if I if I'm able to model the logistics, if I'm able to model that, how many planes will be landing in in a particular time zone right in a particular?
Time interval right? What are what are? What are the runways that mean that? I mean that can be allocated to a particular plane right?
Within what interval of time can we make the runways clear for the landing of the plane? Right? So all these factors right are very much essential in order to ensure that the collision free operation takes place in one of the India's busiest airspace.
OK, another example is a traffic signal controller Bangalore Silk Board, right? So we all know that Bangalore Silk Board is one of the busiest. I mean, I mean areas, right?
Well, there is an very significant amount of traffic congestion, right? So in in that case in the in such a area, if there is a system failure, right and that the particularly with respect to the traffic signal, right, so that will lead to a significant increase in the amount of congestion.
Right. So that that is why we need to understand that within a one particular time interval, what can be the maximum number of cars that can be passing through that particular area, right? What is the number of cars that we should allow?
Right to to to pass what is the number of cars that we should be stopping at a particular signal? What should be the time interval for particular red signal for a green signal? Right. So all in order to ensure that there are no hassles in order to ensure that there are no extra congestion, right? So all this thing requires a perfect modelling.
Right. That is the relationship between the different parameters. What are the parameters here? One is the clock signal the the the traffic signals. Right. The time interval between the different traffic signals, right. That greens, the Greens, green light, red light, yellow light. Right. The time interval between these lights. This is one.
What can be the maximum number of cars right that we can expect to to pass through the particular area, right? So what is the maximum number of time interval that we can? I mean allocate right for for for a, for a for a particular period of time, maybe in the busiest hour of a day the the the rates.
Signal we can we can allocate the rest signal for maybe for five seconds if it is even more busier then we can go for one minute, right? But during the night time when the traffic is less OK, we can reduce it to 30 seconds or maybe 10 seconds, right? So depending on what is the time of the day, what can be the maximum number of cars? Right. So.
All these things will basically ensure us will basically rather help us to ensure a proper synchronizations across the different intersections of the road in the congested traffic junctions, right Bangalore Silk Board being one such example.
Right. So in case of a healthcare systems ventilator systems is one of the classic example, right. So it must respond within a within within a given within within very few seconds, right? But within I mean within milliseconds I can say.
Right.
If it does not work in that fashion, what will happen? The predictability will be varying, right? And that would basically be causing some kind of disturbances with with with different patient loads under critical conditions and it would therefore directly impact the survival of the patients, right? So in so so.
The modelling basically will ensure the predictability in this situation under different kind of patient loads, right? Otherwise, if the ventilator system right, if the different types of control signal that gets delayed, the patient may die, right? So survival of the patient is one of the important factors that we need to take.
Into account while we are modelling the ventilator systems, OK.
So.
You already seen what are the different modelling parameters. That is execution time, the time taken by a task to finish the execution right? For example, in an Indian automotive system airbag systems, what is the minimum amount of time right in order to deploy an airbag in case of an accident, right. So that is basically the execution time.
Period is the time interval within which that particular task will repeats right. For example, a pacemaker will basically stimulate the heart every 800 milliseconds, so therefore it will. It's like a basically a periodic signal. What is the time period of the occurrence of the same?
Task right? So the same task is occurring at an interval of 800 milliseconds, right? What is the deadline? Deadline is already, as I said, the maximum time when which a task must finish right? So for example, in an UAV surveillance for a disaster relief.
The image are stabilisation right, so that must process within 50 milliseconds, right? So in this case the deadline is 50 milliseconds, right? The analogy with the help of the the the Chai right that the the T seller vendor the T vendor example that we have already discussed in the.
Last class.
OK.
Now, what are the different performance measures right in real time systems?
So the different performance measures are, I mean how will you measure? So what were they? Those were the different types of parameters, right that we need to consider while modelling a real time system that is execution time, deadline and period. So these were the different parameters right of a real time system that we need to model.
In order to find out the explicit functional relationship of between the different components right of a real time system, right? And you also need to find out the relationship between these three parameters as well that how much is that out of the total period, right? What is the amount of time that is actually?
Taking taken by the task to finish right. So in that case execution time can be considered as a direct fraction or a direct of the period as well right deadline. So whether the period is greater than the deadline or period is lesser than the deadline, right? So in this way you also have to find out the different relationship.
Between the OR I should say the functional relationship between the different parameters of the systems as well, right? OK. Now how will you measure that whether a particular system is being able to satisfy right the the, the the performance criteria?
So there are some kind of metrics that we need to consider, right? So in case of real time systems, what are the metrics timeliness, what what do you mean by timeliness that whether the system was able to finish the task before the deadline? That is one parameter as sorry that is one performance measure.
One metric, the second metric is predictability means whether for if, for, for the same input for the same input that we are applying, whether we are basically getting the same response or not, right? And what is the response time?
Number two, number three is reliability and availability. That is how often the system is operational, right? Whether it's operational for a particular period of period of time or whether it's operational all throughout the day.
Right. So these are some of the matrix that will give us an idea about the performance of the real time systems.
Yeah.
Now let us find out a comparison right between the two systems. Right. So if you, if you can see the the, the the figure on the left hand side.
OK, so if I can see that there are two systems, right? One is the system A and one is the system B. So what are we trying to compare? We are trying to compare the predictability, right? What of the two systems A&B?
And how we are doing, we are basically plotting the probability density, right? That is means.
Data the.
Peak of the system right means the the the nature of the response of the system, right, and that we are trying to compare so we can see and that response is basically modelled as a distribution, right? So we are trying to model.
The response time of the two systems as a probability distribution, right? And from that probability distribution of the response time we are trying to compare the performance of the two systems right? So for the system A we can see it is the the distribution of the response time that is quite narrower.
Right. That's obviously very narrow. But then again, it's it's occurring at a very sharp peak, right? So that means it has got a high predictable response time, right? Highly predictable for for for every input, right? It will almost give you the same output every now and then, right, but then?
Again, since it is very narrower, it does not mean that it will be very much faster, right? So the output in response to the input can be delayed in some cases, right? So it can take some time for the output to generate in response to a particular input. That that is case for the system a.
Now for the system we we can find that the distribution is quite broader, right? So which we basically gives us an idea that it's very much faster, right, but it's less predictable since the peak is much.
Lower as compared to the system A so that means the system response time right or I should say the system response is not as predictable as the first system. However it is very fast, right? So almost.
The response the output response will occur immediately, right with respect to the input.
So.
This can be also given as examples from Indian analogy, right or from any real time systems that suppose we have got two systems right? Let's say Delhi metric system a.
And a private bus service as a system B. So Delhi metro, right? It's arrives almost exactly every 5 minutes, right? That means it's predictability is is is very high, right. So it will almost our every 5 minutes.
Right on the, On the contrary, the system B. That's our private bus service, right on local bus or state bus, right? Sometimes it will arrive at an interval of three minutes. Sometimes it will arrive at a interval of 15 minutes.
Right. So therefore we can say that system A which is more predictable, right is much preferred. What the system B because of the fact that it is more consistent, right, which is quite valuable in case of the timing constraint, right? In case with respect to the timeliness.
So whenever we're considering the timeliness as a matrix system, A has to be much more preferred as compared to the system B, right?
No.
So therefore, in critical real time applications.
Like railway signalling, industrial control, so the predictable timing is much more important, right? That way. What is what exactly the time, right? What exactly the output response that we are getting with respect to every inputs, right and what is the time interval within which the output is occurring, right. That is very much important particularly.
In Industrial Realty systems. OK, so now if I go for an automotive control, right? So let us say take the electronic engine control unit or ECU in case of a.
Maruti Suzuki car. Right, so there are different types of parameters, right? Input parameters, some output parameters. There are some computational parameters as well, right? For example some gonna system parameters, right?
So what are the input parameters? The input parameters are, for example the oxygen sensors, right? The crankshaft sensors, the throttle positions, and the temperature readings. So these are the inputs for control loop here. We already know what is a what? What do we? What exactly is meant by the control loop we have discussed in detail.
It's right.
So then comes the computation, right? So what are the computational parameters computational parameters is also the system parameters right? So what are the system parameters? The fuel injection rate, right? So what is the amount of fuel that the car is?
Is recurring right for its operation, right? What are the different engine conditions right? And how does the car is adjusting itself? The varying engine conditions, right?
And how does it processing processes the different kind of sensor input that it is receiving right? So these are the modelling parameter or you should say system parameters right? Because engine conditions, fuel injection rate, right, processing of the sensor input. All these are inherent to the.
System, right? So that is why these are called the system parameter for the computational parameters as well. Right? Next comes the output parameters, right? So what are the output parameters the actuators right? How the actuators are performing? Right? So therefore how the actuators are?
Injecting the air fuel mixture right, what is the spark timing control means the spark plug how many times it is sparking in a in a particular interval of time, right? What is the speed regulation, particularly in the ideal cases of the motor?
So so as we said that all these parameters which are which are pertaining to a control loop can also be modelled with the respect to the timing constraints, right? So what are the different constraints?
That that we that that we can have that we have to take into account what are the different timing parameters. One is obviously the execution time then we have got the periods and we have got the deadline as well.
So that means.
That execution time, let us say it is five milliseconds. That means the ECU should always process the sensor data and calculate for example the computational parameters, right? The fuel injection rate. It is basically taking around 5 milliseconds, right?
Now what is the total period? Total period means that what is the time interval after which the inputs are coming to the system right? So what is the so, so so basically the the data from the oxygen sensors, the throttle positions, right? The temperature readings? All these are basically occurring at an interval of.
20 milliseconds, right? So that means even within within one second these are repeating 50 times, right? So therefore the frequency is 50, right per second. Now what is the deadline that basically the control loop should process all the data and come up?
With the output within a maximum time limit of 15 milliseconds each should not go beyond 15 milliseconds, right? So this is how the control loop parameters can be modelled with respect to the real time parameters, right? That means execution time.
The period and the deadline.
So now.
How can we model how can we map the performance of the system right with respect to the response time right? How can we basically classify right the performance of a real time system with respect to the time?
Within which it is responding, right? So therefore we can say that.
Obviously it is imperative for a for a successful system we should be having such kind of response time. That should be very less, which means as soon as we give the input, right.
The output should occur immediately and these are very much critical. These are very much essential, particularly for tasks like signalling control, right, automotive control, right industrial process control for this type of task, right? This response time or timeliness as we discussed earlier is very much imperative.
Right. However, with the gradual passing of time, right?
Wearing and tearing of different components of a system will occur right and the system will inherently tends to become a little slower, right? I mean, many cases also, even inherently the system is slow as well due to some kind of delays that are occurring within the system, right?
So.
What are those delays? So 1 can be time bound delays. Time bound delays means that the delays are dependent only on the time, right? They are not dependent any on the parameters of the system and there can be some kind of parameter bound delays, right?
The delay will be depending on the different parameters of the system, so these delays can Causeway some kind of lag of the output response with respect to the input, right?
So therefore there are three types of performance that we can obtain with the respect to the response time. One is perfect. That means the response is immediate, right? There is no delay. So when whenever we apply the input, we get the output as soon as possible.
That is one what is the next thing next thing is the degraded. That means the performance is degraded, which means that the response time has increased, right? The output is not occurring immediately as the input, right? And there is some kind of lag that is occurring which is.
Appreciable, but obviously the system can still be used even with that amount of delay. And finally, the final one is the useless, which means the delay is so significant that the output is occurring so much later after the application of the input.
But practically, there is no reason that we should use that system.
Right. So therefore in case of an automotive system, it's saying that that whenever we pedal the, whenever we press the acceleration pedal right it should, it should immediately start to accelerate right within a time interval of maybe 5 seconds or 10 milliseconds, right?
Now, if I find that we are going on pressing the acceleration pedal, but the car is accelerating very slowly, maybe it is taking about one minute or two-minute to reach the desired speed that we want the car to follow. That means.
The safety or and the comfort of the car both is being compromised, right? So that is why it is very essential to have such a.
Real time system whose response is perfect. That means it should be as much immediate as it as it can, right? But obviously in practical system there should be always some amount of inherent delays which will Causeway some amount of lag, but that should be minimised.
As far as possible.
OK.
So what is the utilisation factor that we have already discussed that is sum of CI by TI? What is what is CI that is execution time and TI is the total period so.
Now we have already discussed that a particular system is schedulable. If only the RMS scheduling right, that is less than one, right? So that is an important thing that whether a particular system is schedulable or not, right? If it is greater than one, it means the system is not schedulable, right? That means it is a high.
Only delayed system.
OK, so these are some of the examples for Metro signalling systems where we basically can check the schedule ability and find out whether a particular system is schedule level under rate monotonic scheduling or not, right?
So some of the modelling examples that we have already discussed have been explained in details, right with respect to Indian analogies, so.
Now coming to the end of the modelling, we can say that modelling is just not for the purpose of academies, right, but it is a very critical mission that we need to achieve, that we need to satisfy while we are actually designing a real time systems.
Therefore, there are three parameters. We call it as the CTV parameters, right, which are considered to be the main component of a real time system, right? That is execution time, the total period and the deadline. And these are very much essential.
As far as mission critical applications like satellite control, railway signalling, automotive safety systems, health care monitoring are considered the real time system should always meet the deadline. It should be periodic in nature, right?
And it should be.
Responsive right? It should be. There should be timeliness, right? And how can we find out we have to basically find out the utilisation factor and check whether the utilisation factor is less than one for rate monotonic under the idea of rate monotonic scheduling or not. Right? So what happens if the so basically?
Why? Because if I find that the utilisation factor is is is greater than one and it's not.
Scheduleable under rate monotonic scheduling. Then we basically have to manipulate the timing constraints. We basically have to model the timing constraints in such a manner so that the total utilisation factor comes down one and we are able to schedule the system which means.
Which we will be able to model the different parameters with the respect to the timing constraints, right? So the system will be able to function in a seamless manner, right? So that is the objective of the modelling of a real time system.
Yeah.
Now there is another point is that the feasibility analysis right now, what do you mean by feasibility analysis? Feasibility analysis means that.
We have to find out that the tasks right that we have been given right, the tasks that are given to a particular real time systems, whether it is feasible or not, whether the scheduler will be able to perform those tasks in a ordered manner in a timely manner or not right.
So if the tasks are schedulable, the scheduler will be able to generate a schedule and complete or execute the task. If if it is not. If it is not schedulable, that means there are there is an overload situation and we basically have to manipulate the timing parameters. We basically have to.
Take care of that of those timing constraints in such a manner so that there is no such failure, right? So whenever we get some of the raw data, raw data means a different type of tasks, their utilisation factors, right utilisation factors, we can get directly, but sometimes also.
So we can get the execution times the total period Z. So from that we can calculate the utilisation factors, right? And those will basically enable us to determine the feasibility by calculating the utilisation factor.
Right. So that means once we can understand whether a particular task is not schedulable, what we have to do, we basically have to manipulate the timing parameters, right? So that is where we need to apply some kind of predictive analysis analysis as well.
That means.
That suppose if the deadlines are missed, then what will we do? Right. What? What? What is the Plan B that we have got? How can we manipulate it? Right now it's not for a complex system. Sometime it is very sometime it becomes very difficult.
To take into account all the different parameters with the respective timing constraints, right and model the system, right? So in that case it is very difficult to predict the failure with such a complex system with such complex parameters.
Then that case, what do we have to do? We have to based on.
Some kind of analytics, some kind of machine learning based algorithms in order to discover the failures, right? Even without the help of modelling. Right. So that is what is an area where a significant amount of research is going on when analytics, data science, machine learning.
It can all be integrated in order to find out that how the performance of a real time system can be improved, right?
The failures can be reduced even without actually modelling the system right just from the raw data. Whether we are able to solve these challenges or not. So that becomes a very critical part of research as far as modern real time operating systems or real time systems are.
Concerned. OK.
So this is a flow chart, right which basically shows.
That how to approach the scheduling task if I if we have got multiple processes right? So first what do we have to do? We have got a.
Allocate the task we need to allocate the tasks to different processors, right? And then each we have to and then each processor should generate an individual schedules based on the allocations, right? We have to schedule each.
Processors, right? And find out the utilisation factor. Now based on those utilisation factor right, the value of utilisation factor we have to find out whether a particular task is schedulable or not. If it is schedulable fine that means all the red lines are made. If it is not schedulable that means it is not.
Feasible. So we have to take some kind of preemptive action, right? We have to take some, perform some manipulations, take some actions so that all the tasks, right, the utilisation factor comes down. One, all the tasks are able to satisfy the.
Deadline criteria and the modelling and basically the execution takes place in a seamless manner, right? What is the stopping criteria? Stopping criteria means that at what point at what utilisation factor right? Because in many cases it's gonna happen that we can get a utilisation factor.
1.1 we can get an observation factor 1.5 we can get an observation factor just one right? So.
All of them are beyond the.
Marginal value. The optimum value right? That is once right.
But.
The three cases are not actually the same, right? Because they have got different values.
Now 1.1 or one is less critical is less severe as compared to 1.5. So that means that 1.5 has to be taken care of immediately as compared to 1 or 1.1. So in that case this 1.5 right?
Has to be declared a failure much earlier than one or 1.1, right? So that is for the stopping criterion. That is, what time interval should be declared that a particular system is a failure? Right. And we have to take the necessary preemptive actions, right?
So this is also an important consideration while we are trying to model the different parameters right of a real thing system.
Yes.
Yeah.
No.
What are the task models that we have got?
So the once we are able to model the different tasks right, it basically does what it basically provides us. The parameters that are needed for the schedule ability tests, right? So for a periodic tasks right we can find out the period, the execution and the deadline, right? And it always occurs at a fixed interval of time.
That's the task models, right? We have got periodic task, we have got the aperiodic task, we have got the sporadic task and now this type of task model can directly dictate right that which utilisation test we should apply.
Right for periodic task, since it is always periodic, we should go for utilisation bound test. We should check that what is the utilisation factor and depending on the utilisation factor we can decide on its schedule ability decide on its whether the system is a successful or whether the system is a failure and can take the necessary.
Connect directions.
But a sporadic task. However, there is no.
Meaning there is no such significance to find out the utilisation bound. Why? Because it is not occurring at definite interval of time right? It is occurring at.
Random intervals and that also the mean inter mean the inter interval time is also very less right.
So it is very difficult to find out utilisation for this type of product this task. So what matrix should we go about in order to decide whether it is a successful or failure? We have to use the concept of response time that what is the time interval?
Right when we are getting the output after the we are giving the input right? So that time response analysis is therefore or or response time analysis or time response analysis. Whatever we say is a performance metric that we should apply for sporadic task.
For a periodic task, right, we should have we should be dependent on the servers, right? For example UPI payment request processing, right? So we do not know that whether the server will be busy or or or not at the particular interval of time. It has got unpredictable arrival times.
Right. It can occur at any time, right? And also it must combine within a stipulated time as well, right? But in many situations you will find that it says the server is busy, server is not able to connect, right? So therefore.
It does not depend upon the task as well, right? It depends upon the server. So we have to apply some server based approaches right in order to find out the scalability in order to find out the performance analysis of this type of aperiodic tasks.
Right, fine.
So now if I go for an example for a task set.
So this says that there are different type of tasks. All of them are also given the task ID and the name of the task. So we have been given the three things. That is we have got the periods. We have got the execution time and we have got the deadlines as well.
So that means we say from here the task Tau one right, that is the first task, the real altitude sensor.
That is the highest frequency task, right? It's occurring at 20 milliseconds. So that means within one second how it will occur in around maybe 5 * 20 milliseconds, right?
Now the second is adjust the engine thrust right the task two that is also important, but its frequency is quite less, right? And which is the least critical task as far as the?
Period is concerned that is a checking Kevin pressure, right? It's occurring 500 every 500 milliseconds, so it's occurring at a greater interval. So therefore the inter interval inter interval time we can see is gradually increasing. So based on this inter interval time we can also decide on.
That what particular task is critical. So the altitude sensor, which basically means.
The sensor that will read the altitude of the plane will provide the feedback to the pilot, right? So the pilot is able to make corrections as far as the height or the altitude of the aeroplane is considered right? So therefore it is quite critical.
Right, because at different flight levels, right, different kind of situations can occur. So in some flight levels, the turbulence can occur. In other flight levels, there may be other aeroplanes that have been allocated so that that is why it is very important that in what flight level, right, you're flying your aircraft.
Right. So in order to determine the flight level, we have to get the idea of the altitude right and that we can only obtain by means of a altitude sensor and that is why in order to overcome the challenges of collision with other aeroplanes.
Right turbulent, whether we constantly have to feed this altitude data to the control system, right. That is why it is the most critical task now based on those altitude data based on at what height right? The aeroplane is flying.
We can adjust the engine thrust. Why? Because if the aeroplane is flying at a very high altitude, right? So that means there will be less friction, less air, right? So less air means it will have less friction. So that means the engine will be burdened less.
So therefore, less thrust will be required. So that is why we will observe that in most of the cases, the aeroplanes generally fly at very high altitudes like 300000 metres, 36,000 metres. The higher the altitude, less fuel is required. That means less engine thrust is required, right? But if it is flying at lower levels.
There is a lot of turbulence, lot of disturbances, right? So that means aeroplane has to counter more frictional resistances, right? And therefore more amount of fuel is needed, which basically translates into higher engine trust.
So that is why depending on what altitude the aeroplane is flying, we can adjust the amount of engine thrust, right? The force exerted by the engine if you just flying at higher altitude, less force is required from the engine. If it is flying at a lower altitude, higher engine force is required.
So that is why depending on the height, the engine thrust is being adjusted right? So that is why even from these factors of time period we can determine that which tasks are critical.
Right.
No.
Precedence constraints now precedence constraints means that when one task is basically dependent on another another task, for example in the previous case in this aircraft flight control system that adjusting.
Engine thrust is a task which depends upon the altitude, altitude, sensor data. Why? As I said, if the aeroplane is flying at a higher altitude it will require less engine thrust. If it is flying at a lower altitude, it is requiring more engine thrust.
Right.
So therefore, depending on the height right, depending on the altitude we can balance, we can manipulate the thrust of the engine, so therefore controlling the thrust is a task which will occur only after reading the altitude sensor data.
So that means altitude sensor data. Reading the altitude sensor data should occur earlier than adjusting the engine thrust. So that means that is how the precedence of the task is maintained, right? That one task has to complete before the other task.
Right. And this task precedence is basically represented with the help of a directed acyclic acyclic graphs. Means every tasks are represented are nodes right and arrow from one node to another node represents that which tasks are complicate first, right?
So for example, if I just draw.
So.
So if I just draw.
So that means.
This is.
So.
So in this graph you can see that there are four nodes like ABCD. So what is a A is 1, task B is another task, C&D are other two tasks. So this graph, this directed graph basically indicates.
Right. This is that the task a right has to be completed before task B right? B has to be completed before CC has to be completed before D.
Right. So in this way and obviously there is a connection between A&B as well, but if I just remove.
If I just remove this air.
So now it becomes a directed acyclic graph. Why it is called an acyclic graph? Because there is no cycle. So this means that the task A has to be completed before BB has to be completed before C&C has to be completed before B, right? And these arrows basically indicate that.
Which task is having higher precedence, so therefore a is having the higher precedence as compared to BB is having a higher precedence as compared to C&C is having a higher precedence as compared to B right? So that is how we basically can.
Model the precedence characteristics of different tasks, right? So is this clear?
OK.
OK, so this is an example of a task precedence graph for the example of a automated tea making machine, right? But the first step is that we have to dispense the water. Then we have to add the tea leaves.
Add sugar, add milk, start boiling for increase the temperature after the monitoring the monitoring it and we have to final stir the mixture and then serve. So that means that some of the task has to be completed before the other task.
And some of the task can also run in a sequence, right? While others some of the tasks can run in parallel while others must strictly follow a sequence that is, 1 task should occur before the other task. For example the task T1 and T2.
Right, we can add tea leaves only after taking the water from the tap, right? But T2 and T3, like adding tea, water, adding sugar, adding milk. These three can all occur simultaneously. We can Add all these three things simultaneously into the water, but we have to take the water initially.
So what are T1 has to be done before T2T3 and T4 right? But T2T3T4 can be done simultaneously. OK, so that is why we say that the first task T1.
That has to be occur initially. No other tasks right can occur before this T1, but T2T3T4 adding T adding T leaves sugar and milk, right? This can happen after start and this can be taken to the occur and this can be occur considered occurring simultaneously as well, right?
T5 that start boiling so this can occur only after T23T and T4 has occurred right? Because for making tea only after adding the tea leaves, sugar and milk only the tea the the water should boil right? We cannot make the water boil before adding all these three things right? So it has to occur after T2T3 and T4.
Then monitor the temperature so monitoring the temperature will obviously will occur after T5 because unless and until we start boiling, we cannot find out what is exactly the temperature is right and then the final steer and serve. This should occur only after finding out the temperature. If the temperature is high, that means.
That the tea has all has been made, and now we can steer it so it cannot occur simultaneously, or it cannot occur after monitoring the temperature, it can only occur after we have been able to find out what is the temperature that the mixture has reached right.
So therefore, so we can find that there are some some tasks, so therefore the scheduler has to to queue the task it has to model the task taking into fact taking into consideration these dependencies. That is, it cannot start T2T3 and T4 before completing T1.
It cannot perform P5.
Before T2T3 or T4. Similarly, it cannot perform T7T8 before T6. It has to occur after T6, whereas T2T3 and T4 these three can occur simultaneously, right? So therefore?
These are some of the constraints. These are some of the factors that needs to be monitored while we are modelling the precedence of a task with the help of a DAG that is a directed acyc graphs. OK, so now.
OK.
Yeah.
Yeah.
Play trailer of the song.
Yes.
OK. So next one is.
That fixed versus EDF that is always deadline 5 deadline first scheduling scenarios right, let us with the example of the slow mass lower landing case situation. Right. OK. So let us take a break for 5 minutes and then we will be again.
Starting with the last portion.
Rajkumar saying your answer your the answer to your question obviously depends upon like I'm giving the answer, it's quite simple that suppose there are two tasks, right task A&B and suppose a third task C is coming in between A&B.
Now we have to decide on the criticality of the new task, right, whether the task C that is a new task which is coming in right has got a higher priority right depending on the utilisation and depending on the utilisation factor, depending on the date.
The deadline factor, right? If it has got a higher priority right, then obviously what we have to do, we have to basically preempt. We have already discussed a bit of preemptive scheduling we will discuss today more that we have to stop task A.
Right context switching, we already have discussed in the last class that whenever a whenever we are carrying or whenever we are doing one particular task right and a higher priority task comes in right, what do we have to do? We basically have to preempt the already existing task, right, allocate the scheduler to the new task, complete the execution of the new task and.
Then again comes back to the old task that we were using, so that is why we basically have to store the state of the last task that we were executing right while we are leaving it right? Because after completing the high priority task that is that is coming in that is coming in currently.
We do not have to start the old task from its initial situation, right, so we can only start it where we have left. That is basically the answer to your question that the higher priority task when it comes in between A&B, we basically have to preempt the current task.
Allocate the scheduler to the new task right. Store the state of the old task, carry on the new task, execute it, finish it, and then again allocate the scheduler to the old task that we are executing, right? That is how we do it.
And Senthil Kumar K that YT 6 not depends on T2.
So this your question, I mean I don't understand your question. So T6 is what monitoring the temperature, so monitoring the temperature you can only monitoring monitor the temperature after it is after the things start boiling.
Right. Because if you add just leave sugar and milk in a in a in a in a in cold water. So if you monitor the temperature, what will it give? It will always give 0°. It means the the normal room temperature that for example 15°C if it is cold water it will give 5°C.
Yeah.
Right. So monitoring the temperature is only necessary after we have add after after we have started boiling the things right after we have started, make preparing the tea. So once we have started preparing the tea, we need to decide the stopping criteria that when we should stop it and that is why monitoring the temperature is essential. Because if I just put some water leaves.
And sugar in a in a cup of water in a in a bucket full of water right in a pot full of water. And we'll find out which temperature that does not signify anything. Right. So that task monitoring the temperature is important only after we have started preparing the tea, that means we have put all the leaves, milk and sugar in the tea.
And started boiling it. So once we have started boiling it then only we need to decide that what is the temperature we should at which we should stop at which the preparation of tea will be complete, right? So that is why monitoring the temperature depends on T5. It does not depend on adding tea leaves, sugar and meal because because before that you before boil.
That with the the the the water containing sugar leaves and milk are all at room temperature, so monitoring the temperature does not have any significance, right? So that is why T6 depends upon T5. We're not on T2T3 or T4 because T3T2T3 and T4 is only just adding.
You.
These things in a normal room temperature water. So what is the necessity of monitoring the temperature at that point? It does not help us. We already know that it is a room temperature, right? So that is why.
And.
Yes, Ganesh, tell me, what is your question? Ganesh D.

GANESH D .   1:05:48
Hello so am I audible?

Nilanjan Mukherjee .   1:05:49
Yeah, yeah. Tell audible. You tell me.

GANESH D .   1:05:53
Can you share that diagram diagram screen?
So here I have a doubt Sir. So I I can able to understand this T1T2T3 in order of residence and so the execution flow I can able to understand but in the left side image figure 3.1 we have 1 equal to.

Nilanjan Mukherjee .   1:06:01
Yeah. Tell me.
Hmm.

GANESH D .   1:06:19
Null and true that how that values are like.

Nilanjan Mukherjee .   1:06:19
No, no, no, no, no, no.

GANESH D .   1:06:24
Like how the values are calculated.

Nilanjan Mukherjee .   1:06:24
That basically.
That means no. Those are the steps. Those are basically the steps. So basically SO2 means what? So basically initially we are starting that means 5 for five so that is null, right? So that is basically the state values we are indicating, right. So that means T1 which is basically that means what so?

GANESH D .   1:06:32
Well.
Cast.
Thank you.

Nilanjan Mukherjee .   1:06:47
And initially we do not have any steps or so that left hand side that 1234 basically the steps that basically what in in their I mean their occurrence right. So initially everything is 0. So that what that is means we are taking a time instant one it is at 5.
Right. Then at time instant two, what is happening? We are dispensing the water. Right. Fine now. Fine. So you can you can clearly see from this graph right. That means what at initially what we are doing we basically are taking T1.

GANESH D .   1:07:10
OK.

Nilanjan Mukherjee .   1:07:20
Right. First we are this, we are taking the water right. So then after that so this 1234 on the left hand side are basically indicating the time instant then basically maybe we are taking some time to fill up the water. So we are assuming that 234 are the time instance where basically the water has been filled completely to the pot.
Then after one, then 2-3, then 3-4, then that means what 2-3 and three four because just simultaneously they can occur. So that is why one can one has already occurred after that 2-3 and three four they can occur in any order and they can occur either any order or they can occur simultaneously.
Right then after that we right, so that is how we are getting. I mean we are we are modelling it right. So then 134 that means what it should be 5 here 1/5 is missing actually. So consider that the five is missing so then it should be 134112345 and then six that means.

GANESH D .   1:07:59
OK, OK.

Nilanjan Mukherjee .   1:08:18
We have completed up to four, then the Presidency is what, five? And then it's six and then seven and eight can occur together.

GANESH D .   1:08:27
OK, so got it. Yeah. Yeah. Like T1 starts at 0 and T2T3T4 starts at 1 and T5 like has to be reward ability. Yeah. Yeah, T.

Nilanjan Mukherjee .   1:08:27
Have you got it on there right? Yeah.
Then exactly right.
Yeah, five is actually missing, so it should be five year also. Yeah. So 565 and six and then seven and eight can offer together after six.

GANESH D .   1:08:43
Yeah, TT5 has the dependency on one 2-3 and T6 has dependency on 134. So like a flow. Understood. So thanks.

Nilanjan Mukherjee .   1:08:49
Right, right, right, right, exactly.
OK, so that is how we are going.
Yeah, sure. Just a second.
Yeah.
Play song.
Yes, Rajkumar, you tell me your question.

RAJKUMAR SINGH .   1:09:26
So am I audible?

Nilanjan Mukherjee .   1:09:28
Yeah, yeah. You're audible. Tell me.

RAJKUMAR SINGH .   1:09:29
Yeah. So actually you said 5 minute break, so I went sorry for that. So actually I missed your answer.

Nilanjan Mukherjee .   1:09:36
Hmm.
Oh, OK, OK, OK.

RAJKUMAR SINGH .   1:09:39
Actually I wanted to know that when we are talking about the periodic task.

Nilanjan Mukherjee .   1:09:43
Right, right. Yeah, yeah, it's so. Yeah, right. So basically what is happening is that suppose you're carrying two tasks, right? So basically, maybe you're carrying on one particular task, a right. And there is another particular task B that is scheduled to occur after a, right. So there is this precedence between A to B.

RAJKUMAR SINGH .   1:09:46
From ABCD.

Nilanjan Mukherjee .   1:10:02
Now suppose a high priority task comes in. Depending on the deadline criteria, depending on its utilisation factor, so high priority task comes in, let us say C so that we have already discussed in the last class that whenever a high priority task comes in what we have to do, we basically have to preempt the current task. Preempt means we have to stop the current task that we are doing.
Right, So what do you have to do? We have to stop the current task, right? State store the state of the current task that we were executing, right? We have to store its current state. Then we have to preempt it. Preempt it means we have to discard it. We have to allocate the scheduler to the.
High priority task that is currently coming in right the scheduler.
So scheduler basically will be now be allocated to the new task that is coming in right? It will execute the task. The high priority task complete the task and then it will go back to the unfinished task that it was executing earlier, right? It has already stored the IT already stored the.
State of the last task, right? So that is why while it has left, it can start from that point, right? It did not need to again start from initial value of the first task that it was executing. OK got it.

RAJKUMAR SINGH .   1:11:19
Yes, Sir, I got it. But this happens in a periodic task also.

Nilanjan Mukherjee .   1:11:21
Hmm.
Absolutely. Period. Any priority is always important, right? Whether it's a periodic task, whether it's an aperiodic task, whether it's a priority task even in case of a periodic task. Suppose a particular, because periodic task means what it will occur in a, it will occur at a particular interval of time, say at an interval of 20 milliseconds, or an interval of 10.
Milliseconds, right. So that is, that is what we mean by periodic task, right. But even within that 10 milliseconds, right? Even when that particular task is occurring?
A high priority task comes in it should. It should be always. The scheduler should always be allocated to execute the high priority task. So priority is all is important, whether it's a periodic task or whether it's a a periodic task or sporadic task right priority.
For high priority task will always be given the higher precedence, so it should be preempt the current executing task whether it is a periodic task or not execute the higher priority task and then again go back to the unfinished task.

RAJKUMAR SINGH .   1:12:23
Thank you.

Nilanjan Mukherjee .   1:12:26
Have you understood?
Hello. OK. Yeah, which graph?

RAJKUMAR SINGH .   1:12:29
Yes, Sir. I understood. Yeah. Thank you, Sir.

Nilanjan Mukherjee .   1:12:35
You're talking about.
Yeah.
Play song.
I've unmuted you, you can ask me.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:13:08
Yeah.

Nilanjan Mukherjee .   1:13:09
OK, OK.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:13:11
Hello Sir. The last graph which we have discussed.

Nilanjan Mukherjee .   1:13:12
Yeah. Yeah. Tell me.
Yeah, as I told you, so basically you see that means what? First let us it should be like like T1 like T1 is the first point after T1T3T4. Let me just draw it. Let me just draw it so that it in a simple way so that you can understand.
OK.
So first we have.
Let us say this is T1.
Right now you see, there are three tasks T2T3 and T4. All these have got the same precedence because means they can occur at any order and they should all occur after T1. So that means we can have T2 here we can have T.
T3 here we cannot T4 here, so that means they should always occur after this one like.
Now this means this is.
Yeah.
D2.
This is T4, right? So that means T1 should always occur first followed by T2T3 and T4. Now in between T3T2T3T4 there is no precedence all can they can occur at any order. So let us take for example this precedence.
That is first T one is occurring followed by T2, then T3, then T4. We can also could have done it in other manner as well because there is no precedence between T2T3 and T4 so this can occur in any order. What is the next thing T5 now T5 should always occur after this.
So therefore, let us poop.
D5.
Right. So that means.
So that means T5 should always occur T2T3 and T44 right then after T5, what do we have got? We have got T6 so let us put T6 here.
These are the nodes. Pardon my drawing a bit so.
Then after that what we have got T7T8T7T8 should always occur after T6, but they can occur at any order. So let us just put T7.
So.
Right. And since these two can occur in any order, right. So let us just consider that we are doing T7 and T8. So this is how the graph is occurring and you can clearly understand this is a acyclic graph because we are not ending and starting at the same point, right? That is AT 1:00. So that is why we can call it.
There is a directed acyclic graph. Is this clear? So this is an example of how you can draw a directed acyclic graph for how different for a for a task having precedence right you can. Is this clear to you?
Hello.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:17:23
Yes, I can go to that diagram.

Nilanjan Mukherjee .   1:17:28
The you can scream in white white board, right? Why is this clear in the White Board?

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:17:28
NBPT NBPT.

Nilanjan Mukherjee .   1:17:40
Hello.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:17:41
Yeah, this is clear. Sir, can you go to the PPT?

Nilanjan Mukherjee .   1:17:42
Yeah. Yeah. So that, yes. So this is one way of drawing that. So T1 then T2T3 and T4, then T5T6, yes. Now tell me what is your question?
Yeah. Tell me.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:18:01
Here he has considered from T6T 7T8.

Nilanjan Mukherjee .   1:18:04
NT8, after yeah, yes.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:18:08
But whereas T5 and T6 are parallel.

Nilanjan Mukherjee .   1:18:14
No, no. You're making one thing wrong. So see T5. What is T6 start boiling and monitor temperature right? So T5 and T6 are parallel, means they are not. That means they are not at parallel here does not mean that they are at same priority.
Right, because parallel means I mean same priority means whether you are getting the same arrows from the previous task. For example you can see that T2T3 and T4 right all of them are getting the inputs from T1.
Right now, between T2T3 and T4, what can happen? Basically, either you can do T2 earlier or you can do T3 earlier, or you can do T4 earlier, right? So basically you can do anyone of them so but all of them should occur after T1, right? So that is why T2T3T4 have been occur after T1 right?

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:19:08
Yeah.

Nilanjan Mukherjee .   1:19:08
And from for T5T5 should occur. What T5 should always occur after T2 and T3 and T4 right? So that is why there is this. T5 is after T2T3 and T4 right? So arrows are from there then again T6T6 means what T6 should occur after T5 right and then?

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:19:22
Hmm.

Nilanjan Mukherjee .   1:19:27
T7 and T7 and T8. But since T4 that means what T4 and T6 that means what that T6 is occurring after T4T6 is occurring after T5. This indicates that T7 and T8IS occurring after T6 only. So and T7T8 can occur in any order. So we have taken.
Straight arrow connecting T7 and T8 right? So here one thing you can. One thing you have pointed out right that we can we could have I could have given 1 arrow from T5 to T6 indicating that T6 has occurred after T5. But apart from that.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:20:03
Yeah.

Nilanjan Mukherjee .   1:20:03
That the precedence is right so that that thing I've pointed out right.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:20:09
Like. Yeah. Is it OK? Like if we point out from T5 to T6 or this is the cartoon?

Nilanjan Mukherjee .   1:20:16
No, if you if so, see if you. If you also point out from T5 to T6 that is also correct right? The only thing you have to while marking or the only thing that you have to remember is that the precedence of the graph that means which one is occurring, which one is occurring after which one. So once you're correctly pointing out the arrow because I've already said.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:20:31
Yeah.

Nilanjan Mukherjee .   1:20:35
That the tasks are represented by the nodes and the arrow should always be from the first task to the next task, right? So once that is right?
You're all good.
OK.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:20:51
OK.

Nilanjan Mukherjee .   1:20:51
Is this? Yeah. So that is the thing. But The thing is that even for a same precedence task like for example T2T3 and T4, you can give an arrow or you can you can choose to give an arrow. You can choose not to give an arrow because T2T3 and T4 are all at the same precedence there.
All have got the same priority, right? So you can add either T2 or before T3 or T4 after T3 so there is no such precedence between them, right? They have all the same priority so that is why you you between the same precedence task you choose to give an arrow you choose not to give an arrow. But I would suggest that in order.
To clear your confusion, it is always good to assume one particular direction. That is, you assume since T2T3 and T4 are all having same consequence, I saw same precedences. So it is better to choose One Direction. So it is better to follow the.
This graph that T2T3 and T4 are having same precedences but for for our confusion for for our clarity let us just put an error from T2 to T3. Another error from T3T3 to T5 just to indicate that it is occurring in that particular order. Because in Excel you need to show one particular order.
OK here.

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:22:10
Yes.

Nilanjan Mukherjee .   1:22:11
OK, any doubts anyone?
Yeah, that I've said already, no, senthil Kumar K that it should have been better if we if I could have just added an arrow from T5 to T6.
Yes, so, so so yeah, so that is the thing. So always assume the fact that the precedent should be followed that the tasks having the higher priority or occurring should always have an arrow coming outward from it and the arrow should be inward was the task that it is following right.
And for the same precedence task, it is immaterial whether you give an arrow or not, because they are all at the same positions, but obviously in order to have a clarity in order to clear your confusion, it is better to put an arrow assuming one particular direction. For example, have I shown that we are assuming that we are, we assume that.
T2 has occurred, then T3 and then T4 or either of, but you can take the error from any of T2T3 and T4, but we have assumed from T2 to T3 and then from T3 to T4, right? So that is why OK is it clear?

ADLURI VENKATA SHIVA RAMAKRISHNA .   1:23:51
Yes, Sir.

Nilanjan Mukherjee .   1:23:53
OK, now let me take a break for 5 minutes and then we'll be back, OK.
Yes.
Different.
Yeah.
Yeah.
I.
OK.
Yeah.
OK, let's start.
I.
Yeah.
So now.
So let us just compare the fixed fixed scheduling versus earlier deadline for scheduling cases, right. So let us consider the it's so much lower landing situations, right.
So there are 4 task that is to be done right and we have got a UNI processor that is a single processor. So the first task is we have to deploy the solar panels right the the period and the deadline and the execution execution time and the deadline has been given.
Right then we have to establish the communications for all, for which also the execution time and the.
Deadline is also given. We have to run the diagnostics and calibrate the camera right now. Fixed or non preemptive ordering. Non preemptive ordering means one task.
Has to be completed before starting off the another task, right? So there is no such priority. That is, we are considering, right? So what is happening so that T1 is completing at 10 minutes? Obviously because its execution time is 10 minutes right? So that it meets the 30 minutes deadline fine. OK T2 is completed within 20 minutes so that it meets the 60 minute deadlines fine.
T3 is completing its T3 is completing after again 15 minutes so that is what 10 + 10 minutes then 20 minutes that is 30 minutes. So 30 + 15. So T3 is completed in 45 minutes right? So it is also meeting the deadline.
But you see in this case T4 or Tau 4 calibrate the camera, the 4th task that is computed in what minutes, 4445 + 5 or 50 minutes. But it has to completed within 20 minutes. So that is why it has missing the deadline, right?
So in always deadline first what we are doing, we are basically giving a dynamic priority based on the deadline, right? So that means?
Which one has got the highest priority based on its execution time Tau 4 calibrate the camera right? Because it has got the lowest execution time and it has got the lowest deadline right? So therefore based on the deadline, right Tau 4 has got the highest priority, so we are.
Tau 4 first, right? That is, calibrate the camera. Fine. Then what is the next deadline that we are having? We are having the Tau one having a deadline of 30 minutes. Right. So we find that 5 + 15 is 30 minutes. So it is meeting the total deadline of 30 minutes. Right?
Then what is the next deadline that is Tau 3? That is 45 minutes, so deadline is 45 minutes and it is taking the execution time of 15 minutes right? So 15 earlier 15 was completed so 30 minutes. So it is meeting the 45 minutes deadline.
Then Tau 2 it is taking another 20 minutes, so that is 50 minutes. So it is satisfying the total 60 minutes deadline. So we can see that based on the deadline criteria right, dynamic priority based on deadline, we would be able to schedule this complete queue of task which would not mean possible in the case of non preempt.
You fix scheduling schemes, right?
And the question is that in case of automated worlds right where we have got tools like Cheddar, why do we need to go for an analytical approach right? The analytical approach is basically provides us a deep understanding of how the scheduling works right? And even if you are using some automated tools, right and we are getting.
Some unexpected results. Do we able to debug? Do we able to find out the problems associated with those automated tools in case of some kind of?
Some kind of some, some some kind of Miss Miss deadline into some kind of shielding problems, right? And also we have to develop an intuition for an actual real time system behaviour, right and.
Since the analytical approach provides us the core principles, the first principles of scheduling right, so it helps us to design more robust and efficient systems right based on the fundamental and the core principles of the different scheduling algorithms, right?
So now so we have got the rate monotonic scheduling.
So rate mononic scheduling is obviously we have discussed a bit the core principle is it's a static priority algorithm which is inversely conversional to period that means higher priority equal to higher rate, right. So period is less means it has got the higher priority.
So that means task with shorter period, they receive the higher priority. So now once the priorities are assigned right, you do not. You cannot change during the execution. So it is a non preemptive algorithm, right? So it is simple, quite predictable and easy to implement.
For example, we can have.
Right in here, like a doctor in a government hospital. Right. So it tries to attend the most critical cases first, like like chest pain, which needs a critical frequent monitoring. So it's critical. So that is why it is given the highest priority, followed by high fever and then finally, the minor cut. Right.
Another example is the IRCTC ticketing booking analogy. So in this case we have got thousands of concurrent users, right? So we have to check the seat availability checking which runs every second and that has got the highest priority. So payment processing right? So that is the medium priority. And finally the ticket generation which runs every 30 seconds that has got.
The lowest priority because it has got the highest period right. So lower period has got the highest priority. That is the case with the rate monotonic scheduling.
So these are the examples of a rate monotonic scheduling example of a traffic lights, right? So that we have already discussed, right. So you can go through this example and find out and this is a gun chart. Now The thing is that here also.
The priority are quite important and, as we already discussed, the higher priority tasks should always interrupt the lower priority ones and that is why the idea of context switching that we have discussed in the last class is quite important, right.
So deadline verification. So from the so. So once we solve the problem so we can see that all the tasks have been able to complete before that deadlines, right? So you so basically you can go through this example and see how it is being done.
OK.
So multiprocess schedule planning, we have already discussed right now earliest deadline first as that it depends upon the deadline, the task which has got the highest deadline will be accommodated first will be given the highest priority, right?
So now if I just compare that the rate monotonic scheduling and earliest deadline first the two major important fundamental difference between the two is basically on the priority in case of rate monotonic scheduling, the priority is static. They are fixed priorities based on periods.
So shorter period having a higher priority in case of earliest deadline. First it is a dynamic priority, right? It changes at runtime right? As you have already seen. So earliest deadline is having the highest priority right? So rate monotonic is not quite optimal for a uniprocessor system, but it is simpler to implement right? But earliest deadline.
Then first is highly optimal, particularly for uniprocessor systems. Right now, what is the utilisation bound that is given by? We already have discussed that is North 2 to the power one by N -1 right for North mean. The total number of tasks right? So as N tends to Infinity, it becomes almost like 63.3%, right?
Whereas for always deadline first, the utilisation bound is 100%, which means it will be able to complete all the tasks within the deadlines, right? Implementation rate monotonic is quite simple, it has got low overhead and it has got high predictability because it's finally another does not change.
But in case of earliest deadline, the implementation is more complex, having higher overhead and the predictability is lower because the priorities are changing dynamically based on the deadline, right? So.
So that we are so basically we can have an example right of a smart smart watch example. So there can be three tasks like heart rate monitor, step counter and notification alert. So in that case based on the execution time right based on the period, right?
In RM we can find that T1 should have the highest priority because it has got the.
The time period right because shorter period is having the highest priority, right? So it is that that is how so it is 50 milliseconds. So T1 has got the highest shorter period among all them all the tasks. So it has been allocated the highest priority. But if I go we find that T4 is missing the red line.
Right. So we can see the sorry T3 is missing the deadline, right? So therefore we can see that in rate monotonic scheduling, it's not always optimal. We can sometimes some task can miss the deadline but in case of EDF, since the priorities are based on deadlines and they can dynamically change.
Based on the deadlines as well, so therefore all the tasks are able to meet their deadlines, right? So that is why EDF is considered to be far better as compared to rate monotonic schedule.
Right, so processor utilisation bound so that we've already discussed that total utilisation should be less than equal to North 2 to the power one by N -1 right and for rate monotonic scheduling As for north equal to two is 83% which means about 83% of the total task can be able to finish their.
But there is a slight possibility that all that the 100% is not able to meet the deadlines in case of RM. But in case of EDF all the tasks are able to meet the deadline because utilisation is perfectly 100% right, so it can fully utilises the processor capacity.
Right. So that is how.
We can go about right. We can see that what is utilisation bound right for for rate modelling scheduling and we can see that only for single task we have got the utilisation one as one but with the increase in the number of task it's gradually being.
Decreasing. Right. So we as we increase the number of task, right, the utilisation of the processor, that means the possibility of task missing their deadlines also increases, which is not the case in case of EDA for all these deadlines first.
Play song.
Right, so this is a utilisation bound curve, right? Based on the different number of tasks, how it is varying so for north equal to one it has got the highest value one with as we increase the number of task right? The utilisation bound is gradually reducing or decreasing right indicating that the possibility of a task missing the deadlines also.
Increases.
So non preemptive or preemptive scheduling, right? So non preemptive scheduling means that one once the tasks are allocated to the scheduler, they should always run right. You cannot preempt a task in between. You cannot stop a task in between. So it has got a simpler implementation. But obviously it has got a longer response time right and the.
Of missing the deadlines is also quite higher, right? But in case of preemptive scheduling so tasks can be or as I said can be interrupted by high priority task right? So if a particular high priority task comes in the context or the state of the low priority task that it was earlier executing would be saved and then.
Once the newly allocated high priority task has finished with execution, the scheduler will be allocated to the earlier low priority task, right? So that is why it will be more predictable, right? So better it would. It would be better responsiveness to urgent task and it is most.
Important for mission critical applications, right. So that is why we have already discussed that context switching overhead that in case of PMT scheduling we introduce the idea of context switching. So obviously for context switching some amount of cost has been incurred. So that is one area that needs to be taken care of. But.
It does have a real time guarantee, right, because it always meets the deadline.
So.
A perfect processor utilisation means what optimal scheduling, timing, precision and maximum efficiency. What is optimal scheduling, optimal scheduling means the tasks are arranged to eliminate all the ideal time. That means the that means the the the the interval of time between two tasks, right? So one task has completed their execution.
Right. The second task is coming in so that interval of time between the completion of 1 task and the arrival of the second task that should be as least as possible that should be as minimum as possible. Timing precision means each task should execute at the right moment. That means it should satisfy their deadlines and maximum efficiency. That means the processor is never ideal 100%.
Utilisation, there is no time in time loss between the completion of 1 task and the coming in of the next task, right? So that is how we say that it is very much important to have a full processor utilisation for perfect real modelling and design of a real time systems.
Yeah.
So Andor precedence constraints we have already discussed what is an Andor precedence graph. How to formulate how to draw the graph. Now two things that I have to take one care of that just in the example. The earlier example on precedence that we discussed.
Right. There are two things. One is and task and another is an odd task. So what is end task and task means that it cannot begin until all the predecessor task has completed. So it is like.
So.
So if I just go to this diagram so here what was and then what was or so and means for example. What did it say? It said that that cannot begin until all the predecessor task has completed. So that means for example T5.
So T5 is an end precedence task means that T5 cannot start until all of T2T3 and T4 have completed their execution. So for beginning of T5 all of T2T3T4 every.
Each one of T2T3T4 has to be finished before T5 starts. That is known as an and precedence task. Right now, what is an or precedence task or precedence means that it can start if either of any of the predecessor task has been completed.
Right. So that means.
That if I just modify criteria so that becomes that T5 could have started if either of T2 or T3 or T4 was executed, so that could have been a or case that is any of T2 or T3 or T4 have completed, then T5 could have started so that.
It was known as an or precedence, but what is and precedence and precedence means that everyone of the predecessor tasks have to be completed before the starting of the next task. Just in this case that T5 can start only after T2 and T3 and T4 have been completed.
Right.
OK.
OK.
So now.
Now as I said so, this is one particular example, right? This is a there is an algorithm which is called preemptive scheduling algorithm right? That see that that basically deals with this prec 1 algorithm. Just know this algorithm so that starts that that basically help the scheduling with.
Precedence constraints, right? So as I already said, that end task means until all the predecessors are completed, right, it cannot begin. So it's a standard precedence and or means that it can start with anyone of the predecessors is able to complete right? So in the figure you can see that there are six tasks T1TO.
With the dependencies right? So that means after Step 2, right? If I if I go to the the Schedule B that after Step 2 right initial allocation is based on the earliest predecessors, right? It is going back to the.
Previous process as part of the Schedule C After Step 3, so we are going for a parallel execution. So that particularly just know about this algorithm right? You do not have in details this algorithm you have got only rate monotonic scheduling and EDF.
So particularly in Indian context like railway signalling systems where certain operations can proceed after any one of the several cheques. So these are very much crucial in Indian like railway signalling systems, right. So this is just an example of that. There is one particular algorithm like prick one which is used for handling the and Oregon precedence constraints.
OK.
OK, now clock driven scheduling and tables right now, clock driven are mostly the tasks are periodic with known parameters. The schedule is constructed offline before the system execution and entire schedule is repeating. Every hyper period means after a particular given period, right? So the.
Schedule is stored as a table in memory, So what is the hyper period hyper period means? It's basically the least common multiple of all the task periods right? So for example if you have got three tasks right with periods 5 metre 545 and 10 milliseconds then the hyper period of the total schedule is 20 milliseconds. That's the LCM of these three.
So therefore, once we have to, we instead of considering every time period for individual task, we consider the hyper period and then we divide the hyper period according to the number of task, right? So if there are three task so 20 will be divided by three. So 6.33 milliseconds is the time frame for each task.
Right, and that should be less than the deadline in order to meet all the deadlines, right. And obviously that should be greater than the maximum of the execution time, right? To avoid splitting of task. So first, what do we have to do? We basically have to find the LCM of all the periods to determine the total schedule length. We have to select the frame.
Size, that is, dividing it by the total number of tasks right, and ensure that it is less than the minimum value of the deadline in order to ensure the deadlines are missed. Sorry, deadlines are not missed and it should be more than the execution time of individual task, right? So that there is no splitting of task and then we can construct the.
Schedule table right? So this is an example right of clock driven table example so we can see that a water purifier example. So first task is water quality monitoring philtre status check, UV lamp control, IoT data transmission, right.
So basically we have calculate the hyper period taking the LCM of all the time time periods 51 hundred 120. So LCM is 200 milliseconds so 200 / 4 is what 25 S 25 milliseconds. We are dividing the hyper period so 200 by 20. So what is the?
Total frames that we are getting we are getting.
8 frames, right? And now we have to map the tasks to frames using the particular periods, right? So we are right. OK, So what is the total number of frames that we are getting? So time interval that is zero to 25 millimetre is one time interval, 25 to 50 millimetre, 50 to 7575 to 100. So in this way we are.
Getting eight different time frames right within which we have to complete all the tasks, right? So that is how we are going for clock driven scheduling. So first we have to calculate the hyper period then we have to choose the frame size, divide the hyper period in an even manner right? Mostly we do it by the total number of.
Tasks right? That four tasks we can we could have done by like 200 by 4 so we could have done by 50. So we could have all we have we have taken 8 here, right. So the only idea is that we have to divide it evenly so that it satisfies the two two things. One is that it should be less than the minimum amount of deadline less than the minimum.
Value of the DI that for example what is the deadline for all the task we have to consider the minimum among them and this time this time frame should be less than the minimum of all the deadlines, right? That is if you have got 4th task we have to consider the deadline for 4th task.
Check the minimum of the deadline among the four and the frame size should be less than that minimum value, right? And it should be greater than CI. For example, in this case, what is the CI with the maximum of CI? So how many CI's are there? There are four CI's like 10515 and 20. So what is the maximum among the four CI's? Means the?
Execution name 20, right? So therefore the frame size should be greater than 20, so it can be 253035, right? So we have taken 25 S in this way we are going for a clock driven scheduling.
OK.
So these are quite important as far as mission to critical Indian systems are considered right, particularly in Indian Railways Hospital management's right UPI transactions, right. So it reduces the hardware redundancy, the information redundancy and.
Time redundancy as well. It also improves the reliability by modelling the state transition probabilities using the concept of Markov chain and it also accelerates the life testing of different equipments so as to improve the life span right.
So that is the major task of real time scheduling as far as the fault tolerance is considered.
Let.
OK, so these are some of the examples of how basically reliability evaluation and accelerator testing are being done using a fault tolerant control, right? So there are some practise problems also so you can utilisation and find whether the task is scheduling under RM scheduling and EDF scheduling and compare.
The results so OK.
So there are different practise problems. Some questions are also there. The along with the solutions. You can practise these questions. OK, so this was basically the major discussions of today's class. OK, so now yeah, so I'm. I'm unmuting you. So let me just answer your questions.
Archana Devi.
Yeah. Now you're gonna ask you a question. Archana Devi.
I want meeting.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:00:27
Yes, Sir, I have. I have doubt related to how we are listing the task in the table. So I saw some Even so can you go to the slide Sir?

ARCHANA DEVI S   2:00:33
Hello.

Nilanjan Mukherjee .   2:00:34
We reached him.
Vistas.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:00:42
And a previous one had this one, this one. This one. Yeah. So how that? Oh, first zero to 25 millisecond. I'm seeing only T1 to T3, then next is T4. So how we're doing this?

Nilanjan Mukherjee .   2:00:44
Right.
Why? Why, why? Yeah. Why? Why? T1T 2T3. See what is the execution time for T1? The execution time for T1T1 is what, 10 milliseconds, right? T2 is what, 5 milliseconds and C3 is 15 milliseconds, right? So we have to see the total. What is the total load? The total load is 30 milliseconds.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:01:02
Yes, yes.
Here.

Nilanjan Mukherjee .   2:01:10
Right. So that means and then why? Why T4 is within 25 to 50 milliseconds? Because if I add T4 there, it will cross the threshold limit which is give which which we have scheduled as 30 milliseconds right? So that is.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:01:11
Thanks.
Yeah.
But Sir still still crossing. Right, Sir. It's up to 25 millisecond only. But it's going to 30.

Nilanjan Mukherjee .   2:01:29
Yeah.
Yes, yes, time frame is 25 milliseconds right time frame is 25 milliseconds right. But The thing is that there is a threshold. You can see that the total load is never crossing 30 milliseconds. You can see in the in the in the extreme right hand column.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:01:36
Yes.
But where? Where we are calculating this 30 million, how come it is KM Sir?

Nilanjan Mukherjee .   2:01:49
Oh, I am telling that is basically depending on the particular problem. So in this particular problem it is 30 milliseconds that we have assumed, so that will be given in the problem that total load threshold, right, right. Yes, yes.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:01:51
Yeah.
OK, that will be given, OK.
OK, I understand. So this T4 is going into the next turn.

Nilanjan Mukherjee .   2:02:04
OK so.
Right. That is where T4 is going into the next time frame. The total load for a particular time. Sorry, just a second.
The total load right for the the total load the the, the, the timing that I mean the total execution time for the task that should never exceed the total load right given for a particular time interval. So that is given that will be given for the particular problem.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:02:21
Thank you.
OK, so so by adding T1 to T3 already exceeded around 25 plus so so that's why.

Nilanjan Mukherjee .   2:02:37
That is why T4T4 is T4 is going into the next time frame.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:02:41
Next time Prem. OK, got it. Got it. So.

Nilanjan Mukherjee .   2:02:43
OK, OK. Is it clear?

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:02:45
Yes, yes.

Nilanjan Mukherjee .   2:02:46
Yes, yes.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:02:46
Sir, can you repeat this again the table?

Nilanjan Mukherjee .   2:02:51
Who is asking?

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:02:53
Yeah, it's me.

Nilanjan Mukherjee .   2:02:54
Say that's OK.
Which devils?

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:02:58
Yeah.
The same table clock clock table? Yeah.

Nilanjan Mukherjee .   2:03:00
And.
Yeah. Yeah, yeah, yeah, yeah. Tell me, what is the question? Yeah.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:03:06
I was a little bit confused by T1T2T3 was in one frame and T4 was in the 2nd frame.

Nilanjan Mukherjee .   2:03:12
Right. So that I said. So basically this total load you can see now the total load that is given in the extreme right hand column, the total load, you see that the total load is never crossing 30 milliseconds. Can you check that? Can you see that?
Hello.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:03:29
Yeah. Will it be given given data?

Nilanjan Mukherjee .   2:03:30
So that. Yeah, yeah. Yeah, exactly. That will be given data that for, for, for a particular problem, the total load for each interval or for each time frame should be given. So that is 30 milliseconds for this problem that we have assumed in your exam. That will may be given as 25 milliseconds or 20 milliseconds. So depending on the total load.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:03:35
OK.

Nilanjan Mukherjee .   2:03:50
You have to assign the task within a particular time frame. Total load means it cannot exceed that that amount of timing load.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:04:00
OK, C1 plus C2 plus C3.

Nilanjan Mukherjee .   2:04:02
Right.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:04:03
Yeah. OK.
But that why we were considering only 25 within 25 milliseconds, it will be completing.

Nilanjan Mukherjee .   2:04:16
No, 25 milliseconds is what the time frame time frame is not actually that. That does not mean that we have to basically compete within the time frame is basically we are taking the total period right, because this is what this is a clock driven scheduling, right? We have got different types of execution times, right? The different times of.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:04:27
Just.

Nilanjan Mukherjee .   2:04:34
Total time period. So in order to make that uniform, what we are doing, we are basically calculating the LCM and then dividing it uniformly right. So that is the time frame that is not the deadline or that is not the execution and that is the time frame we are considering. The deadline is basically given by the total load.
That that is basically the constraint that we have to consider. Time frame is just to that is just to distribute the total time periods right uniformly among all the task. That is basically because everything is because in clock driven the what is that it is coming at every clock interval of time right. So that means.
At 25 milliseconds, at 25 to 50 milliseconds 1 frame, we are considering clock driven because at clock signal will be occurring at that particular time interval even after the clock signal. We have to continue that task because it has already been started. We cannot I mean stop that task after maybe 25 milliseconds. So we once we have started it, we have to.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:05:12
Yes.

Nilanjan Mukherjee .   2:05:29
Basically completed and that is why the concept of total load is coming into the picture.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:05:35
Sir, at the 3rd frame we were including only T1 and at that 4th frame we were not including any any one of that. Why this happening like that?

Nilanjan Mukherjee .   2:05:37
Hmm.
Yeah. No, because basically you see that because, see.
What is the execution time, the task T1 and T3 that is basically being completed right after T4 in 50 to 75 milliseconds in 25 to 50 milliseconds? What is happening T4 right? So basically T4 is in the in the is in the 2nd frame now then what will happen again a new? Then again what will happen then basically?
We from 50 to 75 milliseconds that will again and next clock cycle will come that the every time period right that clock cycle is basically given as what 50 to 75 milliseconds that we have to complete it right. But you can see that since P1 is basically given what?

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:06:26
50 million.

Nilanjan Mukherjee .   2:06:26
10 milliseconds no. T1 is basically 50 milliseconds, right? So therefore so that that means what so? But if C1 is what C1 is 10 milliseconds. But if I just go go on adding right so maximum what we can do maximum we can basically add what P1 we can add.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:06:30
Yeah.

Nilanjan Mukherjee .   2:06:45
C2 so that means become what from 5050 to 75 because T1 has got 50 milliseconds, right? It can maximum take 10 milliseconds, right? But T2 cannot start at that point of time because T2 can only start again from 100 milliseconds because what is the time period of T2? It is given as 100.
20 seconds.
Can you understand?

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:07:12
Yeah, OK. OK, OK.
Yeah.

Nilanjan Mukherjee .   2:07:15
Are you getting? Are you? Are you getting me on any doubt?

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:07:17
Yeah, I got it. Got it. Yeah, I got it.
So based on the time period it is reoccurring at that particular time scale again.

Nilanjan Mukherjee .   2:07:23
Right, right, right, right, right.

ADLURI VENKATA SHIVA RAMAKRISHNA .   2:07:26
Yeah, got it.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:07:27
Sir side bit confused, can you come again for the time period again?

Nilanjan Mukherjee .   2:07:34
So you see what is.
Zero to 25 milliseconds when we are considering the first time frame. What is the task that we can we we can we have we have T1 that will start right? So T1 means what once it has started right? So that means maximum time it will take is basically so maximum time period is 50 millisecond.
But what is the execution time? Let me complete what is C1? Yeah, 10 milliseconds. So 10 milliseconds plus T25 millisecond. That is 15 + T three that is 30. So if the threshold we are given as 30 milliseconds, why? Because you see.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:07:58
50 millisecond.
10 millisecond, right?

Nilanjan Mukherjee .   2:08:16
Wands.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:08:16
So the load is load is corresponding to the execution time and then time is corresponding to the total time whatever is assigned to the task.

Nilanjan Mukherjee .   2:08:26
Right, right, right.

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:08:27
OK, got it.

Nilanjan Mukherjee .   2:08:28
Right. So isn't there now?

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:08:31
Yes, Sir. Thank you, Sir.

Nilanjan Mukherjee .   2:08:33
OK.
Yeah, Senthil Kumar, can you ask me?
Yes, about yes, I've unmuted you.

SENTHIL KUMAR K .   2:08:58
Yes, Sir. Can you hear me?

Nilanjan Mukherjee .   2:09:00
Yeah. Yeah. You tell me.

SENTHIL KUMAR K .   2:09:01
So that the T3 actually execution time is 1515 millisecond, right? And if you see that time free time free for.

Nilanjan Mukherjee .   2:09:03
Yeah.
Right.
C3 is not CC three is 15 milliseconds execution time, yes.

SENTHIL KUMAR K .   2:09:14
Yes, OK. But if you see that we have a frame of 25 millisecond, OK? So in that frame T1 can be completed, T2 can be completed, T3 cannot be completed because we need 15 milliseconds.

Nilanjan Mukherjee .   2:09:15
Hmm.
Right.
No.
Right.

SENTHIL KUMAR K .   2:09:29
But remaining we have in the time frame is only 10 millisecond. OK so. So I feel like T3 should spread to T time frame T2 time frame two also. So T3 should be executed in frame one as well as frame 2.

Nilanjan Mukherjee .   2:09:29
Right.
Late.
Yes, you can do it also, but once the question is that once you've started you've started executing a particular task, you know in one time frame, right? So suppose you've started in one time frame, so you can basically complete it in the time frame. Or as you said, you can also go to the next time frame as well.
Yeah, that would basically suffice. The total load, yeah.

SENTHIL KUMAR K .   2:10:05
And for.
Yes, unfortunately it cannot be completed because time frame is only 25 millisecond. We need of 30 millisecond, so you should go to T3 as well as time frame time frame one as well as time frame two you should go.

Nilanjan Mukherjee .   2:10:10
No. Yeah, but The thing is that.
Yeah, yeah, you can also do that. But The thing is that since T3 has already started in time frame one right because it has already started in time frame one no.

SENTHIL KUMAR K .   2:10:29
And the total load will be if you put 23 in frame one as well as frame 2, the total load for the frame 2 will be 25 millisecond.

Nilanjan Mukherjee .   2:10:37
Let's right, so that that will also suffice, right? So see, this is one particular type of scheduling. Scheduling is basically you can go, you can basically go for I mean for one particular problem SO11 the thing that you are saying is one particular scheduling I have done in another type of scheduling. Both of them suffice all the.

SENTHIL KUMAR K .   2:10:42
Yes.

Nilanjan Mukherjee .   2:10:56
Constraints, right? But the question is that for one particular problem, there can be many different scheduling manners or the scheduler can schedule the task in different ways, right? The question is that whether we are able to satisfy all the constraints or not.

SENTHIL KUMAR K .   2:11:09
OK. So we we should just worry about initiation of the task. Is it Sir?

Nilanjan Mukherjee .   2:11:13
Absolutely.

SENTHIL KUMAR K .   2:11:15
OK, got it.

Nilanjan Mukherjee .   2:11:17
What? Any questions? Any more questions? Anyone else?

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:11:22
Sir, I asked one question. That's what that shortest remaining time in settling said where.

Nilanjan Mukherjee .   2:11:22
OK.
Oh, shortest remaining time is mostly for general operating systems. It's not that much prevalent in case of real time operating system, so that means it basically when two tasks are being so one task is being executed and another task has coming come in and preempted it right? So then once it has finished executing.
In the current task, it will check that what is the remaining amount of time left for the execution of the remaining task. Then based on that computation that what is the remaining. So for example if I say that one task is occurring at time interval 0 and it needs 10 milliseconds to compress the execution.
Right. It has got the priority three right now. When it is execution, suppose at 5 milliseconds it has come, it has worked for five milliseconds now again another task of higher priority comes in. So what will happen? It will preempt this first task, right? It has the first task has worked for five millisecond it will stop there. Now the scheduler will allocate the.
Operate on the and execute the second task. Let us say the second task is operating for 15 milliseconds. Now after the completion of the second task, right? What will happen? It will basically now check that out of the all the remaining task which one has got the shortest remaining time. It will consider the first task. The first task has.
For five milliseconds. So its remaining time is five milliseconds. Let us say task three. It has not started. So what is its total remaining time say 20 milliseconds task four say 30 milliseconds. So out of 20 milliseconds 30 milliseconds and five milliseconds. What is the shortest remaining time 5 milliseconds so it will go back to task one but.
If there was a case, the task two has got the remaining time remaining time means it has not started yet. So then so that is total execution time, let us say 3 milliseconds. If that three milliseconds is less than.
The five millisecond that is remaining of the first task, So what it will do, it will not execute the first task. It will go complete the second third task which has got a execution time of three milliseconds. After that it will again come back to first task which has got the remaining time of five milliseconds. So that is basically made by shortest remaining time that is.
We should we have a real time operating system. Mostly we have a conventional OS. Is it clear?

JHA MURALI KUMAR VIRENDRA ARCHANA DEVI .   2:13:45
Yes, I got it now, yeah.

Nilanjan Mukherjee .   2:13:47
Got it, OK, OK. OK. Thank you.

SENTHIL KUMAR K .   2:14:03
Thank you, Sir.

Nilanjan Mukherjee .   2:14:39
Yes, we are going to you tell me.
Yeah, tell me. I've unmuted you.
I unmuted you, you can ask me.

GANESH D .   2:15:00
Yes, yes, just now I got the option to unmute. So my question is like.

Nilanjan Mukherjee .   2:15:03
Yeah.

GANESH D .   2:15:09
So.
According to that task T1T 2T3, so we have all the task released at a time and then we know the capacity of the task when it'll be executed and then so we saw the load. So when the T3 is not capable to complete.

Nilanjan Mukherjee .   2:15:17
Hmm.
Great.

GANESH D .   2:15:32
Within the timeline, so it is overloaded. The time period is overloaded, right? So if that is the case, then the scheduling is possible with the criteria or else how? How would we? How would we conclude Sir?

Nilanjan Mukherjee .   2:15:37
Right, right, right.
That that is why that is why this threshold is limit is given. Now you could you could have seen that there was this threshold limit of 30 milliseconds was given that is set for a particular problem you should not cross that 30 milliseconds or what is the threshold limit for that particular problem. If it crosses that.

GANESH D .   2:16:01
OK, even.

Nilanjan Mukherjee .   2:16:03
If if you process that it has to be given allocated to the next time frame or the next time size that that is given.

GANESH D .   2:16:08
OK.
OK, so even if one frame is not fitting properly then the task 3 frame frame should not sit in the frame one and it should send to the next frame OK if next frame also effects then that frame two also should be sent to frame frame three. OK OK.

Nilanjan Mukherjee .   2:16:14
Hmm.
Go to the next frame.
Right, right.
Then then then to the next frame, right. Right. Right. Right. OK. Thank you. OK. Yeah.

GANESH D .   2:16:29
Got it, got. It's a books, yeah.

Nilanjan Mukherjee . stopped transcription