Real Time Operating Systems (Merged - ESZG524MELZG524)(S1-25)-20250907_082517-Meeting Recording

September 7, 2025, 2:55AM

1h 34m 49s

Nilanjan Mukherjee . started transcription

Nilanjan Mukherjee .   1:50
OK, so basically.
So today we'll be discussing a little bit about scheduling and some properties and features of the scheduling algorithms, right? So as we've already discussed that a real time tasks are mostly generated due to the occurrence of certain events, right? So the events can be either external or internal events, right?
So for example, a tasks may get generated due to a temperature sensor sensing a high level, sensing a high level of temperature, right? So that can be considered as a task, right? Now when a generally a task gets generated, we say that it has been released or it has actually arrived, right?
Now when the task has arrived, right? Particularly a lot of tasks are arriving in a queue, right? Then obviously it becomes quite imperative to schedule the tasks right in order to maintain the order in which the various tasks are to be executed, right? So that the efficiency is maintained, right?
Now, why is this required? Because the primary means that is adapted by an operating system is to meet the deadlines of the different tasks, right? So as a result, scheduling becomes an important problem as far as RTOS is considered, right?
Now, a particular task typically occurs a large number of times. Each time it is triggered by an event, right? And whenever a particular task is recurring, it is said to have been released or generated, right? So therefore.
At any instant of time, right? If a task T recurs, we say that that particular task instant is has been arrived, right? Now there are two deadlines that we have already discussed, right? One is called an absolute deadline, which is counted from the time 0.
And there is a relative deadline, right, which is counted from the time of occurrence of the task, right? So you can clearly see that from the measured from that that that from the from time 0 till the.
Deadline right till the till the till the maximum amount of time right within which the task needs to finish its execution. That is basically known as given as the absolute deadline, right? And when it is measured from the time of arrival of the task, right? It is said to be the relative deadline, right?
Now the time from the occurrence of the event which generates the tasks right is basically known as the to the time the results are produced is known as the response time, right? So it can happen. You can see that that green portion is basically the.
Task that is being executed, right? But why is the response time is basically being measured from the time E to T Why? Because E is basically the time at which the task is occurring, right? It may happen that.
After occurrence, it has to sit idle for a few seconds or a few milliseconds or some amount of time because it before it actually starts executing because the other tasks are getting executed by the scheduler, right? So therefore.
The time which is being measured from the occurrence of the event which generates the task. That is why it is given as E to the time the results are produced is known as the given as the response time. Now for soft real time task this response time needs to be minimized and for hard real time cases.
As the task is completing within the deadline, therefore there is no advantage, there is no, there is no significance, right? Why we should complete it early, right? Because it is already completing within its deadline, right? So they say there is no other.
Significant that is being attached right as far as completing the task before the deadline is often because it is already being covered before the deadline.
Now what is the phase for a periodic task? Now the phase for a periodic task is denoted as the time we measured from the instance 0 till the occurrence of the first instance of the task, right? Because a task can occur.
Recur several times, right? So it can occur once, it can occur twice, it can occur thrice, and maybe many times it can occur, right? So now the time instant which is measured from zero till its first.
Instance till it's first occurrence right is denoted as the phase for a periodic task because only a periodic task can require multiple times, right? Not a periodic or sporadic task, right? And that particular time instant is basically denoted by the phase, right?
Now whenever we are going for to discuss the real time scheduler, there are a few components of real time schedule that we should discuss, right? First one is the task dispatcher, right? Now what is the task dispatcher and what is the function of this component? The function of this component is that that whenever there is a lot of tasks in.
The queue, right? Particularly in the ready queue, this task dispatcher selects the next task, right? That is needs to be executed right from the ready queue and gives it control of the CPU. So now that particular task will get hold of the CPU and other resources and will get executed right?
It can also handle something which is which we have already discussed, known as the context switching. So what is context switching? Saving the state of the preempted task and restoring the state of the new task once the current task has finished its execution.
Right. And it also ensure that the different tasks are activated according to their scheduling policy, right? For example, a rate monotonic scheduling or earliest deadline first, right? So so that can be the different.
Hello.
Scheduling policy which tasks are which tasks generally follow. So task dispatcher generally ensure that this scheduling policy are maintained in order to ensure the smooth execution of the tasks. So this is 1 component of the real time scheduler.
The second component is what it is known as a clock interrupt handler, right? Or it is also known as a timer manager, right?
So what is the function? The function is that it provides timing services which is very much essential for real time scheduling, right? And also generates the periodic interrupts in order to track time and trigger events such as when some tasks are being sleeping. Sleeping means it is being held idle for some amount of time. And why it is so? Because other tasks.
Right, which is which is having higher priority is getting executed, right? So that is why it needs and when those higher priority task has completed the execution right, the tasks that is currently being idle needs to be executed, right? So that is why.
Uh.
The clock interrupt handler perform this task, performs this task of waking up right the already ideal jobs, right?
Another important function is that it is used to start the periodic task at predefined intervals, right? And it also needs to check the different deadlines and trigger the deadline miss handler that when some deadline is missed in order to it needs to indicate it needs to have some kind of a trigger that will indicate that.
That this particular task has missed its deadline, right? So these are some of the important functions that is performed by the clock interrupt handler. It also keeps the global system time updated for scheduling decisions. What is meant by global system time? The time at which.
Different tasks needs to be executed right according to the time of the operating systems, right? Because different operating systems are operating at different times, right? So basically it needs to keep a track of the time at which the different tasks are being executed right when particular task is being.
Updated right. So all this, all this, all this instances, all this function, all this.
Things needs to be updated right by the clock interrupt handler for good scheduling or maintaining good scheduling decisions, right? And final one is the scheduling policy module, right? Which is obviously that it has to implement the logic for deciding the task priorities.
Is the model of execution that what is the logic it should follow in order to decide that in case of several tasks requiring a particular resource for its execution, which resource should it give the control to the task right? That which task is having the higher priority right? Whether task A or task B, which one needs to get which one needs to?
Be provided with the resources of the CPU in order to complete its execution, right? It also needs to determine that which of the task needs to run right based on criteria such as the static priorities, for example, with monotonic scheduling, dynamic priorities.
Because dynamic priorities are always being decided by the earliest EDF or earliest deadline first algorithms, right? And also it needs to follow hybrid or a custom policy, right? So what does it maintains? It maintains.
Or ensures predictability and guarantees the deadlines under a particular policy. So it always ensures that no tasks miss the deadlines right under a particular policy.
So therefore, as I said, the three major components of a real time scheduler are the task dispatcher, the clock interrupt handler, and the scheduling policy module, right? And what are the functions? The main function of the task dispatcher is that it selects the next task and performs context switching. The clock interrupt handler, it manages the timing and.
Figures the periodic events, whereby the scheduling policy module determines the order of the different task and sets the priority in order to determine the execution of the tasks.
OK.
Is this clear till now?
OK.
Now here are some important terminologies that we need to understand before.
Moving forward, one is we have already discussed a little bit of this earlier, that is valid schedule. What is a valid schedule? Valid schedule means at one time, particularly in case of an uniprocessor, maximum one task is assigned to a particular processor if it's a uniprocessor one at a time.
And before it is ready, right? No task is scheduled before 1 particular task is ready, right? And the precedence and resource constraints of all the task needs to be satisfied. That is why it is said to be a valid schedule. Schedule is what the order in which different tasks are needs to be executed.
So sorry, why maintaining it?
Priority, right? And obviously the deadline constraint needs to be satisfied, right? And what is a feasible schedule? A valid schedule in which a valid valid schedule in which all the tasks meets their respective timing and deadline constraints are said to be a feasible schedule.
OK.
So now So what is the proficient schedule? So now suppose.
If I have got two scheduler right? If you have got two job scheduler or task scheduler, say S1 and S2 now.
If it is so that that a particular scheduler S1 right is said to be as much proficient as another scheduler S2 if.
That whatever the tasks that S1 can do right, S1 can feasibly schedule whatever task that can be performed by sorry if S2 can feasibly schedule a set of tasks right, which can be done by S1 also.
Right, but not vice versa. Vice versa means that S1 can perform what S2 can perform what S1 can do, right? But S1 cannot perform what S2 can do. That is known as a. So in that way we can say that S1 is said to be as much proficient.
Has another scheduler S2, right? So that means if if there is a particular set of task right, which S2 can feasible schedule right? But that can also be done by S1, right?
But there can be another set of task, right, which can be done by S 2, right, but cannot be done by S one and not vice versa. So that means.
There is one common set of task which can be scheduled feasibly, but why both S1 and S2? But there is another set of task which can be feasibly scheduled by S2 but it cannot be done by S1. So that set of task is not common between the two that cannot be scheduled by S1.
So that is why it can be said that S one is as proficient as S 2, right? But it is not more proficient. It is equally proficient, but not more proficient. Why? Because what S 2 can? Because what S one can perform that can be done by S 2 also.
Right. But there is some set of task where there is some extra qualities that S to have is that it can schedule another set of task which cannot be done by S1, right? So that is why it is said to be as proficient as.
S2 both are one is equal to other but not more equal right? So now.
What is equal? Now there comes this concept of equality proficient that both are equal. That means A implies B, also B implies A. That means if there is whatever the set of task that can be done by S2, that can also be done by S1.
Right. And whatever can be done by S1, that can be done by S2, right? So that is known as equally proficient schedulers.
Right. So that is why this is the major difference between the concept of these two type of schedulers, right?
OK, is this clear?
Now what is an optimal scheduler? An optimal scheduler is that it can feas it can it can feasibly schedule any set of task that can be scheduled by any other scheduler, right? That is known as an optimal scheduler.
Suppose if you have got two set of tasks, S1 and S2, right? I mean two schedulers, S1 and S2, right? And S1 can schedule a particular set of task. Now those set of tasks can also be feasibly scheduled by S2, right? Then S3 is said to be an optimal scheduler.
Right. That is the definition of what we may optimal scheduling.
So what are scheduling points?
OK, now there is something we call as the scheduling points, right? So what are scheduling points? Scheduling point means on the time instant, on the time axis, what are the points at which the scheduler makes decision that which of the task needs to be done?
Next right so so how does a scheduler decide that which of the task needs to be done next? So it has to be done by using some kind of scheduling algorithms, either rate monotonic scheduler or earliest deadline first depending on different priorities right timing constraints.
Deadline constraints and other parameters, right? So there can be two different types of.
Ways in which a scheduler makes can make this take this decision. One is the clock driven, another is event driven. So what is clock driven scheduling this scheduling points which are defined by interrupts from a periodic timer that is known as clock driven right where?
Basically I have got a timer and it periodically sends a signal in order to justify in order to give us the idea at which the scheduling needs to be taken place. That is known as a clock driven, right? Now what is event driven?
The schedule event driven are that at any instant of time a particular event is being generated and that generation of an event, right?
Gives us the scheduling points right over the time frame that at what instant that those particular events are being generated and those events basically gives us those scheduling points, right? Is this clear?
Now as far as real time scheduling is considered, so there is a lot of research that is being carried out in order to develop real time schedulers. So there have been scheduler for uniprocessors, there have been scheduler for multiprocessors and distributed systems as well.
Right. So much of the focus right on this research was carried out during the 1970s and 80s and therefore the real time schedulers broadly be classified into two parts. One as we have discussed is clock driven, another is event driven that means.
How? So basically this classification of schedulers are mostly based on how the scheduler is taking the decisions right to schedule the task, whether it is using a timer, that means it's a it's a it's a clock driven one, or whether it's based on generation of a particular event, right? It's an event driven 1.
Now when we have got the clock driven scheduling, so most of the decisions regarding that which job is to be run next, obviously given the priority of the job that is made at clock interrupts right and.
For those decisions, we generally use timers to determine the scheduling points, right? And therefore the job list as which as well as the task that needs to be run and for how long, right? What is the time taken by a task to complete right for this execution that needs to be stored in a table?
Right. So two most popular examples of clock driven scheduling are the table driven scheduler and cyclic scheduler, right? There is also another scheduler which is known as a round Robin scheduling, which can also be taken as an example of a clock driven one, right?
So how does a table driven scheduling looks like? So here we have got a simple table which which shows that we have got three tasks T1T2 and T3. So the start time for the task T1 is 0. So it initially so it starts at time 0 and it stops at time 100. The next task T2 starts at 101.
Instant of time, right? Seconds or milliseconds, whatever may be the unit and it stops at 1:50 and then the third task right arrives at 1:51. Rather it starts at 1:51 for its execution and completes at 2:25. So this clearly shows the start time, the stop time and obviously just by.
Having or getting the difference between the two timing instance, we can get an idea about the execution times as well.
Now in case of table driven scheduling, how does we schedule a set of tasks? Let us see if we have got N periodic tasks, how does we schedule it? Now this is an important question.
Yeah.
Is this clear till now?
OK, now the method or the algorithm right that it follows in order to schedule a set of N periodic tasks right is given as that the scheduler develops a permanent schedule for a period which is given by the LCM of the.
Different periods of all the tasks, right? And it scores scores the schedule in a table and that is repeated forever. So suppose we have got three tasks, right? Right. For example T1T2T3T4, right? We have got set of four tasks like T1T2T3T4 and each of the task has got a certain.
Period, right? Every time has got like for example, if I have got TT1 as a period of 100 milliseconds or seconds, 100 as 100 milliseconds, T2 has got a period of 50 milliseconds, T4 has a period of 150 milliseconds, T3 has a period of 150 milliseconds, T4 has a period of 200 milliseconds. Then how do we decide?
The common period for all these tasks. In case of table driven scheduling, what do we do? We basically determine the LCM of all the periods, right? So that is why so we will take a LCM of 100.
150.
200, right? Whatever may be the total number of tasks we have got, we have to take the LCM of the periods of all the tasks, right? And once we have got that, we can just score the store that schedule in a table, which means that every task should now be repeated at that particular period which is obtained.
By getting the LCM of the individual periods, right? And that schedule is repeated forever, right? Now what are the disadvantages? The disadvantages are is that when the number of tasks are very large, we need to have a large number of timers.
Right. And there is obviously a.
Costs, right. Both in terms of money as well in terms of computational efficiency, right, which restricts the number of timers that a particular operating system can support, right. So that is a major disadvantage of table driven scheduling.
Now how does now what is the cycle of a task?
Now let us say that we have got a few tasks that is been carried out, right? And basically we have been repeated. So basically we can see from this diagram that we have got four different tasks in one cycle, right? So and each task has got different periods as well, the green one, the red one.
The yellow one and then blue one, right? And then after completion of all this set of tasks, let us say we have got 4 tasks, P1P2P3 and P4, right? So P1P2P3 and P4 is occurring within a particular instant of time and after that it is again repeated. This set of four tasks is again getting repeated after a particular.
Or instant of time, right? Then that amount of time within which these four tasks are getting repeated, right? So that is known as a major cycle. So what is the major cycle? So major cycle is basically the time instant in which this set of four tasks or set of any number of tasks are getting repeated.
So here we have taken an example of four tasks. It can be 5, it can be 6, it can be 7. That is known as a major cycle. So therefore a major what is a major cycle of a set of task? It's a time period right in which the different tasks are recurring identically.
Is this clear?
Ok. So, now
Let me just have a water for two minutes. I'll come. I'm coming back.
OK, so.
So therefore, how do we decide it? So now suppose we have got a set of tasks, let us say St. right? So therefore the major cycle of a set of tasks, right that is T1T2TN is given by obviously the LCM of P1P2 and PN, right? Even when the tasks have got arbitrary phasing. Phasing means the phase, right?
So basically what is how? How do we decide this common period? How do we decide this major cycle that what should be the time within which it is occurring? So we have to basically take the LCM. Suppose we have got this green green one. Let us say the green one task T1. Let us say it has got maybe a time period of 100 milliseconds, right?
The second one, that is the red one, may have got a time period of maybe 20 milliseconds, so 100, then 20, then yellow one may have say a period of let us say 150 milliseconds.
Right. And green one and blue one, let us say I have a period of 125. So what should be the major cycle? It is nothing but just the LCM of these four quantities, right? So that that I've just said here right in this case that the scheduler will develop a permanent schedule.
For a period which is given by the LCM of P1P2 PN and then after that. So suppose if I say the LCM is 1, suppose the LCM is 200 milliseconds. So that means that 200 milliseconds will basically be the major cycle for this set of four tasks and after 200 milliseconds these set of four tasks will again.
P Right. So that means that major cycle will be therefore given by this 200 milliseconds, which is the LCM of these four tasks. OK.
OK, now each major cycle has got an integral number of minor cycles or frames, right? Now how the frame boundaries are marked? They are marked by the interrupts from a periodic timer, right? Now what are the frames?
So these are the frames, that is the interval between the two task right within a major cycle that is given as a minor cycle or it is also known as the frame right that what is basically the time difference between the completion of one task and the execution of the beginning of another task.
Right. So that particular time frame that that period between you see the the the time instant, the time interval between the yellow task and the blue task, between the completion of the yellow task and the starting of the blue task, that is we call it as a time frame.
OK. And that basically is also known as the minor cycle because it is repeating within each major cycle itself.
Now how do we select A-frame size right? Because you can understand whenever a particular task is getting is getting is recurring. So what do we have to ensure that we have to go for context switching? We have to basically.
Save the state of the task that is currently being preempted, complete the execution of the task with higher priority and go back, come back to the task right that that was being preempted earlier, right? So we have to minimize this context switch, right? So minimize this time.
For which we need to save the state of this preempted task. Because why? Because otherwise it will take a lot of time to store this frame frame size, right? And the context switching should be over it should be quite large, so therefore.
We have to minimize this context region. How can we ensure that? We can ensure by considering the fact that a particular frame size should be always larger than the task size. What is the task size? The amount of time that is being taken for a task to complete the execution. That is known as a task size and obviously each frame size should be larger.
than the task is, so that in ensure that there is the context switching is basically the minimum. OK, now
Next is minimization of table size, that the table size should be as minimum as possible, right? That the minor cycle should squarely divide the major cycle. That means we should get a perfect answer, right? There should be no decimal, right? Or there are no only integers, right?
And what is another final criteria that we have to satisfy the task deadline, right? That between the arrival of a task and its deadline, at least there should be one full frame, right?
OK now.
Whenever we are going for table driven scheduling, as I've said also for clock driven scheduling, when we are scheduling in periodic tasks, the schedule is stored in a table, right? Because clock driven scheduling has got two parts, one is an event.
Table driven scheduling and a cyclic scheduling. So in a table driven scheduling. So we have to design the schedule and what is the time period for which we need to develop the schedule which is given by the LCM of the periods for different task that we already have.
What?
Now, what is the advantage of this type of table driven scheduling, right? The advantage is that it's very compact, right? It requires very little storage space and it incurs very little runtime over it. Now what are the difficulties? It's very inflexible. Why? Because.
We are considering a very static dead timelines, right? So because we are we are assuming that the period for the different task is constant and based on those fact based on those consideration that the.
Time period. The time. Uh.
For each task is constant, we are developing a schedule.
So if some new task comes in with a different with a different period, it will be very difficult to accommodate because because we have already developed the schedule for a set of task, right? So that is why it is very difficult to incorporate the dynamic task.
However, the table driven scheduler is one of the most simplest type of scheduling that we can develop.
And obviously the problem is that when the number of tasks are quite large, we have to set a timer for a large for a large number of times and therefore the cost constraint, the computational constraint that overhead becomes quite significant, right? Because every task runs only for a few million or a.
Microseconds, right? But we have to set the timer for each of the time that task is being generated. So that means that the cost is quite large.
Right.
So in order to overcome these problems, what do we do? We go for another type of schedulers which is known as the cyclic schedulers, which are very popular and these are extensively being used in the industry. Right now there is a large majority of small embedded applications which uses.
The cyclic schedulers right now, how does a cyclic scheduler work?
OK.
Therefore.
Therefore, now what is the case for a cyclic schedulers? In case for cyclic schedulers, the schedule is stored for one major cycle, right? And.
The precomputed schedule that is been repeated. So therefore a major cycle consists of one or more minor cycles or frames, and the scheduling occurs only at the beginning of frames, right?
Now you can see that what is the what happens in case of each major cycle. The different tasks they recur at identical time points, right? So if I divide the entire time frame in the entire timing instant, the entire timing frame into different.
Into into into many major cycles. What is a major cycle that is computed just by getting the NCM of the different periods of all the tasks, right? So now within each major cycle, what do we observe? We observe that all the.
Tasks are occurring at the same instant with respect to a particular major cycle. So within the first major cycle, if the green task is occurring right, let us say at time T equal to 10, then in the second mega cycle also the green task is occurring at time T equal to 10.
N with respect to its starting point. So if I just use the laser pointer.
So, suppose if I say or let me just use a point n. So, suppose if I say that this period
Right. Suppose this period, this period, let us say this is given by.
1010 milliseconds. So now with with with respect to here also.
This is it.
Right. So that means.
M.
Sorry sorry this this period right. So that is basically also given by 10. So therefore the different tasks are recurring at identical time points within the given cycle within a particular major cycle right. And major cycle is determined by given by as I already discussed LCM of the.
tasks, right, even when the different tasks have got arbitrary phase.
No.
This can be greater than the LCM when F what is FF is the frame size that does not divide the major size.
Now each major cycle consists of identical integral number of minor cycles. We already call it as frames. So that means these are frames.
These are frames, right? These are small frames. So that is why we can say that each major cycle consists in integral number of minor cycles. We also call it as frames, right? Now the frame boundaries are marked by interrupts from a.
Periodic timer, right? So let us say that we have got a task because why it is task T2T3 and given because it is recurring. So suppose.
We have got a task T2, then T3 and then T2 is again recurring. So obviously between T2 and T3 there is a finite amount of time interval, right? So that is given by F1. Between T3 and T2 we have got in F2 and from T2 to the next task we will be having that F3.
That is given by the frame size that that what is basically the frame size for the different tasks right between rather because frame is what which is the inter interval time right?
So that is why each task is assigned to run in one or more frames, right? And frame size therefore is a important design parameter while we are going for a cyclic scheduler. So the selected frame size has to satisfy a few constraints. What are the constraints? It has to minimize minimize the context switch. Why?
Now within A-frame, a task instance must complete its running, right? Otherwise it has to be suspended and restarted at a later frame because it cannot complete within the particular frames, right? And therefore the scheduler would get invoked many a times.
Now another is to have unnecessary context switches, right? That is, frame size should always be larger than the execution time of each task. Execution time means the time between the arrival and the completion of a task, right? So that frame size should always be larger, right? Why?
Why? Because otherwise just otherwise what will happen that the task if it is shorter that.
Must be completed running within this assigned frame. That is why the frame size has to be larger than the time period of a task. Otherwise it has to be suspended if it is not being able to complete with the frame and restarted in a later frame, right?
And we set a lower bound for the frame size as well.
So how do we go about it? Now? Unless a minor cycle squarely divide the major cycle. Minors squarely divide means that division should always be a integer. There should be no fraction, right? So until we get a integer that storing schedule for one major cycle, that would not be sufficient, right?
So therefore the schedules with a major cycle would not repeat and this would make the table size of the table quite large, which is not the thing that we want. So that makes the task for of designing the schedulers quite difficult. OK.
Now we have already seen that another criteria needs to satisfy is that that it needs to satisfy the deadline, right? That between you can see that between the arrival of a task and its deadline, at least there has to be.
One full frame that should exist right now. If there is not even a single frame, then obviously the task would meet its deadline, right? So because it would go beyond the deadline. Why? Because.
Whenever, because if there is not a single frame, so when whenever that particular task is taken for scheduling, the deadline would be very near and it is high possibility that it would miss the deadline, right? So therefore at least one full frame must exist between the arrival of a task and its.
Deadline. Why? Because the scheduling must be completed within each frame and if the frame does not exist, right? If one full frame does not exist, then that means whenever we are trying to schedule that particular task, right, the deadline would be very closer to it, right? And therefore the high possibility that the task should.
meet its deadline okay is this clear?
So that is how you can see the frame and therefore once the frame size is basically larger than the period for each of the task, you can see that they have been able to meet the deadlines. There is no missing of deadlines right for each of the individual tasks as well.
Now therefore the worst case is that that the task arrives after the frame has started. We can clearly see in the next in in the second diagram that after the frame has start that that is sorry in the first diagram that after the frame has started a particular task has arrived.
That is the the Violet one or the pink one, right? So that means what is happening after the frame. After the frame has started, you can see that there is a possibility that the deadline can be missed, right? That TI would miss the deadline.
That is the possibility.
OK, but in the second diagram for the green one, you can see that for the there are two green ones, so there are two green tasks. But you can see that for the first task since it has started before the deadline, right?
It has been able to, sorry, since it has started before the frame has started arrived or rather the frame has started, the task has arrived. The first green one, it is being able to compete within the deadline. But for the second green one, since it is arrived after the frame has started, the frame is starting from what? The frame is started from Pi, right? So therefore obviously.
The second green one has started much after TI, that means within after the frame has started within the frame size. So therefore the possibility of it missing the deadline is quite high, right? We can clearly understand. So that is the criteria.
That the frame size should always be larger than the task size in order to ensure that the task deadline is met. Now there can be several frame sizes which can satisfy the constraints. So we have to find out what is the most possible possible frames.
And a plausible frame size has to be found out right by the fact that out of all the different frame sizes that is given, we have to choose the smallest one, right? Why? Because then only the chances of successful scheduling would be higher right if I choose.
The higher frame sizes then also scheduling will be possible, but then that the task, the cost of selecting higher frame sizes would be quite high, right? That would make the cost from the point of view of both the computational efficiency as well as money that would make the cost.
Quite significant, right? So that is why we should always choose the smallest frame size from among a set of different frames.
So, is this clear?
Is this clear till now?
OK.
So let's take a break for 5 minutes and we'll be back with the last part.
But.
Let's start.
OK now.
Suppose we have got this example and we have to compute a suitable frame size for the following task set, which is given by that U1 equal to.
Four that is period is 4, D1 deadline is 4 the second task right?
The period is 5, deadline is 5 also. So the third task the period is 20 and the deadline is 20 as well and E 3 is 1.5. How can you compute A frame size for this following task? And this is another example. How would you do it?
So this.
I therefore leave it as an exercise for you to calculate and come up with the answer in the next class that how would you compute the suitable frame size for the following set. I have already given you the formulas, I have already given you the the methods, how to calculate the periods, how to calculate the.
Total frame, my major frame size, the major cycle. So you know how to calculate it. So now The thing is that you should be able to calculate and find the frame size, right? So that is the formula, OK.
Now the problem with the cycle scheduler is that that as the number of tasks increases, it becomes very difficult to select a suitable frame size, right? So therefore it results in suboptimal schedules, right? That means the CPU times in which many frames are wasted, right? And therefore it results in poor schedulability.
OK, so therefore we go for a generalized task scheduler. What is a generalized task scheduler? A generalized task scheduler is 1 which will basically because why? Why do we not go for a generalized task scheduler? The reason is that there are many practical systems.
Which consists of a mixture right of periodic task, a periodic and sporadic tasks, right? So you cannot say with certainty that only one type of tasks are present, right? We can have a different types of task and different mixture of task, right? A task can be considered to be.
So that is the problem. Now The thing is that we can schedule a periodic task with the help of a cyclic scheduler, right? But how do we go about designing A sporadic and an aperiodic task using a, I mean, sorry, schedule of sporadic and aperiodic task?
Using a cyclic scheduler that we can use that we can do by using the concept of slack time and unused frames, right? Because what is the slack time? The time that is left right from the completion of a task.
Right in the deadline, that is the slack type unused frames. The frames are not been used so far, but the frames have been generated right? So those can be used to.
Schedule the sporadic and the aperiodic tasks right now. Next after clock driven scheduling we go for the event driven schedulers which can handle both sporadic and aperiodic task right? Why? Because clock driven scheduler.
We have already seen that there are two types of clock driven scheduler, the table driven scheduler and the cyclic schedulers which are mostly used for handling periodic tasks. But event driven can handle both sporadic and aperiary tasks as well, right? And these are used in more complex applications, right?
Now in case of.
Event-driven scheduler. The scheduling points are mostly generated by some occurrence of an event, right? And what are those events? Task completion, right? And arrival of a task. Those are the events which basically helps to decide the scheduling points in case of an event-driven scheduler.
OK, now what is preemptive schedulers? Obviously we know what is preemptive schedulers that whenever we have got a higher priority task which is coming in right, the current task may be suspended or preempted as we call it and then.
We allocate the CPU or the other resources to the high priority task until it completed its execution and once it the high priority task has finished its execution, we simply.
Go back to the initially running task and finish its competition right.
Now there are different types of event driven scheduler, right? Rate monotonic scheduling is 1, event driven scheduling is 1. I'm sorry, earliest deadline first is another. However, the simplest event driven scheduler is something we call as a foreground background scheduler.
Right.
Now another type of scheduler is known as a time slice round Robin scheduling, which is a hybrid scheduler, right? Now in time slice round Robin, time slice round Robin scheduling means it will basically have a particular slice of time for each of the task, right? So suppose.
We say that it will allocate 5 milliseconds of time to each task. So that means what will happen for every task will be given 5 milliseconds of time. Whether it finishes its completion within that amount of time or not, it will be there will be no extension of time. It will just be given 5 milliseconds of time and once that 5 milliseconds of time is over, it's exhausted.
That task will be preempted and the next task will be started and then again after the next task, after each 5 milliseconds will be preempted and then the third task will be given, right? So in this way there will be a long robin scheduling. That means every task will get opportunity of.
Getting scheduled right in a sequential manner, right? And that is known as a time slice round Robin schedule. Why time sliced? Because we are having a fixed time for each individual task, right? Now that is commonly now most commonly used in case of traditional operating system, but not in case of RTOS, right?
So what I've done.
Scheduling points in case of round Robin scheduling, the tasks completing or suspending and the clock interrupts, mostly the clock interrupt because it has got a time time instant which is fixed right. So it is however very much less proficient right than the cyclic or the table to schedule and since it provides same instant of time or same amount of.
Time on each of the individual task, so therefore it does not take into consideration the task priorities as well. Now these are generally the preemptive schedulers, right? So we have the preemptive schedulers are also known as the greedy schedulers. Why? Because.
They they never keep the processor idle if a task is ready. So whenever the task is ready, right, it will always allocate a particular task to the processor, right? Because of the priority deadlines, because of its priority consideration. So it will never keep a particular processor idle.
Now independent tasks can execute on a uniprocessor and there are two algorithms which pretty much summarize the important results as far as event driven scheduling is considered. One is known as an earliest deadline first and another is known as a rate monotonic scheduling, right. So we have already discussed in details these two algorithms, so we are not going into details.
We will just discuss the foreground background scheduler, which is a very simple type of scheduling algorithm.
OK, now.
In case of foreground background scheduler, what happens is that mostly the real time tasks are run as a foreground tasks, right? So we have got the sporadic, the aperiodic, the periodic tasks mostly we can take consider as.
The foreground task, the aperiodic task, the sporadic task and other non real time task. They are considered as the background tasks. Now for the foreground task and every scheduling point what happens? We take the highest priority foreground task and that is being scheduled right?
So out of all the foreground task that is available to us, we should consider the one which is having the highest priority, right?
No.
Let's see how how does we do it. Now suppose we take that we have got a background task as well and TB is the background task, right? It's processing time is EB amount of time that is required for execution of this background task.
So therefore, what is the time taken for it to complete that CT that is given by EB that is the processing time divided by 1 minus EI by Pi. What is Pi? We already know what is EI and what is Pi, right? Pi is a period for all the task, whether it's background or foreground and we have to.
find out the the utilization for each of the tasks one minus the utilization and then divide the entire thing by the processing time for the background task right. So, now
Suppose we have a task which is given whose period is 50 milliseconds, which is a periodic foreground task. Its E one is given by 100 milliseconds, D1 is 100 milliseconds right? And the EB is also given as 1020 milliseconds right? Now can we complete the completion time for the background task?
Right. So these you can just try it out because we have got only one periodic foreground task, right? So what will EB we know? So we have got only one EI that will be what EB divided by 1 minus EI by Pi, right? So we have to find out the completion time for the background task and and see whether it is able to satisfy the deadline or not.
That is given by 100 milliseconds, whether it is able to complete within 100 milliseconds or not.
ok So, now we see that
That whenever we have got different type of tasks, right, we generally model the task with the help of task graphs, right? Now this task, what are the task graphs? The task graphs are also known as the directed acyclic graphs, right?
And therefore a task graph can be used to model these tasks as well, right? Where we have got individual nodes which represents the task. For example, in case if I take a real-time task, right? So it can be data acquisition, filtering, control computation, this can be the nodes.
And what are the edges that connects the two nodes? The edges mostly represent the precedence of the dependency. That is, task A must compete before the beginning of task B, right? So what are the advantage of task graphs? The major advantage of the task graph is that it helps us to visualize the.
Concurrency, right? That is the independent tasks, right? That what? What do we mean by independent task? The task which have got no connecting edges that can run in parallel, right? And dependent task, which means which have got connected nodes that must respect the order, right? So let us see an example. So here.
If I take an example of a robotic arm control cycle, what is the task one? Reading the sensors. What is the task two? Compute the control law and what is the task three that is sent the torque command. Now this graph is a linear graph. Why? Because T1 because T3 depends on T2 and T2 depends on T1, right? So therefore this.
Precedence is maintained and each graph task is basically dependent on the previous task. That is, T3 is dependent on T2 and T2 is dependent on T1, right? However, if we have got another add another node, like suppose if I have.
A node here, right?
Then this path, then this task, let us say this is.
T4 then this T2. This is T2.
Then this T2 and T4 can be considered as independent because they are at the same priority and then from T4 also we can reach this.
OK, fine. Is it clear understood?
So that is how we call we model a particular set of task using the task graphs.
Now in many cases we mostly use a DAG. What is a DAG? That are directed acyclic graphs. What does it mean? That this graph does not contain any cycles, which means it ensures.
When a particular task terminates and no circular dependencies occur, means there is no cycle.
So these although seems to be a acyclic graph, it can seem that there is cycle, but you can see that there is no cycle because there is only one directional that is edges that is present we do not have.
The edges from T3 to T2 or T2 to T1. So in this way this is not a directed acyclic graph.
So, therefore, a DAG ensures that
There are no circular dependencies and it is good for maintaining real time workflows, data flow and parallel compute. So what are the key properties? Now each path represents A execution sequence, flexible execution sequence path from one node to another node, right?
And what is the critical path length that the longest path that that is available through the directed acyclic graph which gives you the minimum execution time. So as I said there can be independent task. What are the independent T2 and T4 are independent task?
And T3 is basically.
Dependent task T2 is a dependent task, right? Because T3 depends on T2T2 depends on T1, right? Also T3 depends on T4 and T4 depends on T1. But T2 and T4 is independent. Why? Because.
They are not depending on each other. There is no connection between the two. There is no precedence between these two tasks. So therefore T2 and T4 are independent of each other.
So now if I take an example of an image processing system where the task one is basically we have to acquire an image. The second task is that we have to detect an edge. That's the the second task. Why it is called task 2B?
Because it is an independent task of task two, right? So it is at the same priority, it is the same level that is what given by the color segmentation. And final task is that we have to fuse, we have to get the fusion of all the results that we have to merge all the results together. So how can we model it?
This can be modeled as you see that we have got T1 which is acquiring an image. From T1 we are going to T2A, that is edge detection. Also from T1 we are going to T2B color segmentation. That means T2A and T2B.
Basically are at the same level, just like the previous graph, right? So T2A and T2B are dependent task with respect to T1. Why? Because they are depending on T1, but they are independent with respect to each other because there is no order of connective precedence between them.
and T3, T3 basically depends on both T2a and T2b. So, therefore T3 is obviously a dependent task.
Now, what are the criteria for mapping the task graphs onto scheduling? Now, once the tasks are modeled in a graph, right now they must be mapped onto the processors, right? And they need to be scheduled as well, right? So for the dependent task, what are the criteria that needs to be satisfied? They must preserve the precedence constraints.
Right. And its performance, the scheduling performance depends upon the critical path length, right? We are also called as a Cpl. right? Whereas for independent task they can be freely assigned to available processors as well, right? And.
In in for those tasks, the scheduler should also maximize the parallelism and can balance the load. Now what are the mapping criteria? How we that it means? How can we schedule the? How can we map the tasks on to the graphs, right?
So the first one is the task priority, right? Then based on the period, deadline, criticality, whether it is a hard real time task or a soft real time task, that is 1 mapping criteria. How can we map the different tasks on to the task graphs, right? One is the processor or the code availability.
That multiprocessor can be used for partitioning, we also call a static assignment or it can be used for global scheduling. That is a dynamic assignment as well. Now communication overhead, right? Now the data transfer between the different tasks running on different cores that must be considered that that how?
The data is being transferred between these different tasks, right? Using the communication channels, right? That is also that needs to be taken care of, right? Because every communication channel does not have got the same bandwidth right for processing or sending or receiving the information that we need to that intends to send or receive.
So that is also another criteria. How can we schedule the tasks right into the processor based on the communication bandwidth?
And fault tolerance that the safety critical systems must duplicate the tasks right and needs to be re execute once it is failed. Now what are the different functional parameters of tasks?
Now every task.
Has got.
Some functional parameters which affects the schedulability, which is.
Given by #1 is the execution time. Rather we we call it as the worst case execution time, which is very much important in order to ensure or guarantee the hard real-time deadlines, right? And average execution time, which is mostly used for the simulation and the statistical modeling purpose, right?
Now what are the different type of tasks, periodic tasks, sporadic tasks and aperadic tasks that we have got? And another parameter is the deadline, whether the task has got a hard deadline in case for example a braking system of an automotive of an automobile or whether we have got a soft deadline right where some.
Occasional misses are quite tolerable, for example the in case of a video frame analysis, right?
Now when we are going for the parametric modeling of task, one way of modeling the task is by using a distribution called Weibull distributions, right where we mostly consider the task arrivals of the failures as a stochastic one that.
That that they are probabilistic, they are not deterministic. That we cannot determine or or or have an idea a priori that how they are going to arrive or how they are going to fail, right. What is the Weibull distribution? The probability density function for a Weibull distribution is given by.
That beta by NT by T by beta by ETA T by ETA.
Whole to the power beta -1 E to the power minus T by ETA whole to the power beta right? But beta is what? Beta is basically the shape parameter which decrease which increase which shows the.
Failure rate that how the particular task is being failure right? For example the infant mortality right that how everything that how the childs are getting died right? What is the rate at which infants are getting are dying right? If beta equal to 1, it indicates a constant failure rate.
Right that the failure can occur randomly at any particular time that is beta greater than equal to one. It means an increasing failure rate for machines, for industrial machines which with gradual, with which gradual usage it will start to fail, wear out.
Right. So therefore beta is the shape parameter which basically used to indicate the failure rate. The values of beta are used to indicate the value failure rate. Alright, so that is how we look now here for different values of beta and ETA.
We can find out how the graphs are looking, how the distribution that how the different tasks are getting modeled. So now how can we understand the schedule of behavior with this distributions?
Now, as I said we can assume that the EDF or the R N they can be stochastic right. So, therefore, the varying beta changes
And indicates rather I should say that how spread out or how bursty the tasks are, right? That means how its failure, how its shape is changing, right? So in case of failure. So it can also be used to indicate the failure rate of different tasks that we have just discussed, right?
So if beta less than one, you can see if beta is less than one, right 0.5, you can see it's more of a clustered arrivals that the tasks are arriving very closely to each other, right? And the load is quite bursty, means the load is quite heavy.
But when beta is increasing right and it is increasing and becomes equal to 1 right? What do we see? Beta equal to 1 is getting a exponential right? So see the the the green one is basically exponential beta equal to 1 but is obviously.
We can also use to indicate the failure rate as well, right? So therefore it indicates that the tasks are arriving exponentially in a random way, right? But it does not have any memory means they do not remember how the previous tasks are arriving, right?
Take into consideration the arrival of the previous tasks, right? But if beta greater than equal to 1, the arrivals are very much regular and is quite easier to schedule, right? So therefore the we will PDF for different shape parameters are often used for modeling the.
Tasks in case of RTOS and how as I said that is beta less than one. In this case if I take a value of 0.5 S it indicates that there is a.
Arrivals or the failures, right? As I said, it can also be used to indicate the failure rate. The probability is quite high and it is a bursty like behavior, right? The load is quite high. If it is equal to 1, we consider to be an exponential distribution, right? And.
If beta is greater than one, like if I take it as true, it indicates A increasing failure rate like aging or wear out of machines in industrial situations, right? So is this clear to everyone?
OK, so for the next for the second question.
What is uniprocessor and multiprocessor option? So what is uniprocessor? From the name itself it is quite obvious that we have got just one processor.
Right. And a single processor is used to schedule all the task, or rather I should say that a single processor is being allocated to the different tasks either sequentially or dynamically depending on the priorities, right? So there's so, so.
So one process. So at any time only a single task can be allocated to a unit processor, right? It cannot incorporate the features of parallelism, whereas in a multiprocessor that means what we have got many different processors that are available to us. So obviously with many different processors, different tasks.
Can be taken care of simultaneously. We do not need to always go into go for sequential modeling or sequential scheduling. We can go for parallel modeling as well or parallel scheduling as well, right? Because there are more than one processors, so task one can be given to processor one task.
Two can be given to processor 2. Task 3 can be given to processor 3, so obviously the computational time, right?
For the all the tasks in in a multiprocessor system will be much lesser right? Because it employs A parallelism option. It does not go for a sequential option, right? So is this clear?
OK, now for the first question, what is the difference between frame size and major cycle? OK, what is major cycle? Major cycle is the total period. What is the total period? The LCM of the?
Periods of the different tasks. So suppose if I got 4 tasks right and the period of the first task is, let us say 100, period of the second task is 150, period of the third task is say 175 and 4th task is 200. So 100, one 50175 and 200 we have to take the LCM of.
all these four tasks right and the and the answer that we obtain after getting the LCM that is basically the major cycle period. That means suppose if I get the answer as 225, so 225 is the period is the is the cycle time within which all the four tasks are getting completed and after this two
25 seconds or milliseconds, whatever the units, again the 4th task will get repeated right? Again task one, then task two, then task three and then task four and this that will basically be operating in a.
Sequential manner within the next 225 period. So that is basically known as the major cycle. Now the next thing is what is the frame size? What is the frame size? The frame size is basically the within a cycle within one major cycle.
So within one major cycle, right, there are two different tasks, say a task T1 and task T2, and there is certain amount of time interval right between the completion of task T2 and arrival of task T4, right? So that difference of time between the.
There are two tasks right? That is known as the frames, and that frame size has to be larger than the what individual task period. Why? In order to satisfy the deadline constraint, right? Why? Because if the frame size is shorter, then what will happen? A particular task can occur after the frame has started.
And therefore the deadline will be very near to it, right? So it can miss the possibility of missing the deadline will be quite higher. See the order to overcome that problem. Why? What do we do? We basically take the frame size which is larger than the.
Task sets right? And what is? So if you if you just go this diagram.
This is the frame. So cycle is what? That is the total period that is obtained after taking the LCM of all the periods of the four tasks. And frame is what? That is the time interval or the interval of time between the completion of one task and the starting of the another task. So let us say between the green yellow task.
And the blue task, that time interval between these two tasks is called the frame size. OK, so that is the that is so frame size. Frames are also known as the minor cycles. Why? Because if within a particular major cycle, what is the time interval after which the particular task is getting repeated. So that is why it is also called the minor cycle and it is a part of the major cycle as well.
OK, is this clear?
OK.
OK. So with that we basically cover the cover today's session. So in the next session, the last session before your EC2, so we'll take the 6th contact session and the 7th contact session together and with that we'll basically be completing our EC2.
OK.
OK. Thank you. Thank you for your time.
OK. OK. Thank you. Thanks a lot.

Nilanjan Mukherjee . stopped transcription